VELLORE INSTITUTE OF TECHNOLOGY SCHOOL OF COMPUTER SCIENCE AND ENGINEERING SWARA Secure Wideband Adaptive Real-time Audio over UDP A perceptually aware, mutually authenticated voice-chat system over UDP with importance-weighted forward error correction, joint quality control and overlay relay selection FINAL PROJECT REPORT Computer Networks · October 2026

| SUBMITTED BY Rohit Arora 24BDS0252 Ansh Gupta 24BDS0119 Amrit Raj 24BDS0104 | PROJECT GUIDE Dr. Anitha E Assistant Professor Sr. Grade 1 Department of Information Security School of Computer Science and Engineering anitha.e@vit.ac.in |
| --- | --- |

Department of Information Security  ·  School of Computer Science and Engineering

Vellore Institute of Technology

# Project Details

|  | PROJECT GUIDE Dr. Anitha E Assistant Professor Sr. Grade 1 Department of Information Security School of Computer Science and Engineering Vellore Institute of Technology anitha.e@vit.ac.in |
| --- | --- |

TEAM

| # | Name | Registration number | Programme |
| --- | --- | --- | --- |
| 1 | Rohit Arora | 24BDS0252 | B.Tech. · SCOPE, VIT |
| 2 | Ansh Gupta | 24BDS0119 | B.Tech. · SCOPE, VIT |
| 3 | Amrit Raj | 24BDS0104 | B.Tech. · SCOPE, VIT |

| Project title | SWARA — Secure Wideband Adaptive Real-time Audio over UDP |
| --- | --- |
| Course | Computer Networks |
| Department | Department of Information Security, School of Computer Science and Engineering |
| Institution | Vellore Institute of Technology |
| Submission | October 2026 |
| Repository | https://github.com/rohitarora80905-glitch/swara-v1.0 |
| Deliverables | Working voice-chat application over UDP (server, clients, web console), evaluation harness, research figures, this report |

# Abstract

Conventional voice-over-IP stacks treat every 20 ms packet of speech as equally valuable, protect all of them with the same redundancy, size the playout buffer against a loss target that ignores the delay it adds, and secure the call with handshakes that were designed for connection-oriented transports. This project presents SWARA (Secure Wideband Adaptive Real-time Audio over UDP), a complete, working voice-chat system built from the datagram up, together with the research evaluation that motivated its design.

SWARA's central idea is that packet loss should be judged by what the listener loses, not by how many packets were lost. A causal Perceptual Importance (PI) analyser scores each frame from its energy, voicing, zero-crossing rate, spectral flux and onset behaviour, and carries the score as an authenticated-but-readable octet in the packet header. A PI-weighted adaptive forward error correction scheme (PI-AFEC) spends a fixed Reed–Solomon parity budget on the frame groups that carry meaning and withholds it from silence, and protects vowel onsets with time-displaced anchor replicas that survive loss bursts a block code cannot. The NAVIK controller chooses codec bitrate, redundancy and DiffServ marking jointly by maximising the ITU-T G.107 E-model rating, distinguishes congestion from link loss through the one-way-delay trend, and couples the R-factor-optimal jitter buffer to the protection decision. SPARK, a stateless password-authenticated key agreement, mutually authenticates users in two round trips without ever sending the password, bounds reflection amplification at 0.31×, and migrates a call between networks in zero round trips; the relay forwards ChaCha20-Poly1305 ciphertext it cannot read. SETU, a distance-vector overlay, selects relay paths when the direct Internet route violates the triangle inequality.

The system is implemented in about 5,500 lines of Python over real UDP sockets, IMA-ADPCM speech coding with per-frame state reset, X25519/HKDF/ChaCha20-Poly1305 cryptography, and a FastAPI/uvicorn web console that runs instrumented live calls, A/B benchmarks and an interactive protocol toolkit. In the simulated evaluation, SWARA raises predicted MOS at 12 % loss from 3.38 (unprotected) and 3.37 (uniform FEC with the same budget) to 3.53, reaches the same quality as uniform FEC for 16.7 % less redundancy, keeps the measured optimum playout depth within 1 ms of its own choice, and authenticates in 397 bytes against roughly 1,900 bytes for DTLS-SRTP. In live calls over an impaired loopback path at 15 % loss with mean bursts of four packets, SWARA matches uniform FEC's perceptual damage (7.3 %) with 11 % less bandwidth and reduces it by 16 % relative to unprotected UDP; at 10 % loss it achieves the lowest damage of the three (4.2 %) at 12 % less bandwidth than uniform FEC. The evaluation also uncovered and corrected a scheduler defect in the project's own impairment injector that had silently disabled redundancy mid-call, illustrating why the measurement instrument must be validated before the protocol is judged.

Keywords — VoIP, UDP, forward error correction, unequal loss protection, Reed–Solomon, perceptual quality, E-model, jitter buffer, password-authenticated key exchange, X25519, ChaCha20-Poly1305, DiffServ, overlay routing.

# Contents

Project Details	i

Abstract	ii

Contents	iii

List of Figures	v

List of Tables	vii

Abbreviations	viii

1   Introduction	1

1.1   Background	1

1.2   Motivation	1

1.3   Problem statement	2

1.4   Scope and contributions	2

1.5   Organisation of the report	3

2   Literature Survey	4

2.1   Packet loss recovery for streaming audio	4

2.2   Unequal protection and perceptual weighting	4

2.3   Playout scheduling and concealment	4

2.4   Objective quality assessment	5

2.5   Channel models	5

2.6   Secure real-time transport and key establishment	5

2.7   Network QoS and overlay routing	5

3   Research Gap	7

4   Objectives	8

5   Proposed Work	9

5.1   System architecture	9

5.2   Wire protocol	9

5.3   Perceptual importance analysis	11

5.4   PI-AFEC: importance-weighted adaptive FEC	12

5.5   SPARK: stateless password-authenticated key agreement	13

5.6   NAVIK: joint quality control	15

5.7   Receiver: estimation, repair and R-factor-optimal playout	16

5.8   SETU: voice-aware overlay routing	17

5.9   Speech codec with per-frame state reset	17

5.10   Evaluation methodology	18

6   Implementation	19

6.1   Technology stack	19

6.2   Repository layout	20

6.3   Module responsibilities	20

6.4   The live call pipeline	21

6.5   The relay server	22

6.6   The client	22

6.7   Demonstration harness and experiments	22

6.8   The web console and REST API	23

6.9   Server control launcher	24

6.10   Verification during development	25

6.11   Running the system	26

7   Results and Discussion	27

7.1   Experimental setup	27

7.2   Perceptual importance separates what matters	27

7.3   Quality under packet loss	28

7.4   Resilience to bursty loss	29

7.5   Quality returned per byte of redundancy	30

7.6   The jitter-buffer optimum	31

7.7   Handshake cost and security properties	32

7.8   Relay-path selection	33

7.9   Realistic network profiles	33

7.10   Live end-to-end calls	34

7.11   Discussion	37

7.12   Limitations	38

8   Conclusion	39

8.1   Future work	39

References	40

Appendix A   Source Code Repository	42

A.1   Obtaining and running the code	42

A.2   Repository contents	42

A.3   Reproducibility	43

Appendix B   Screenshots	44

B.1   Starting the server	44

B.2   A real call between three separate programs — both sides	44

B.3   Command-line clients on the relay hosted by the web console	46

B.4   Live Call Lab	50

B.5   A/B Benchmark	53

B.6   Network Simulator	55

B.7   Research Results	56

B.8   Protocol toolkit	64

B.9   Operations and API	71

Appendix C   Source Code	74

# List of Figures

Why not all losses are equal. Left: the perceptual damage weight used throughout this project rises superlinearly with a frame's importance and is penalised for consecutive losses. Centre: the E-model's delay impairment has a knee at 177.3 ms, so an over-deep jitter buffer is a quality defect. Right: for a fixed loss rate, clustered (bursty) loss is far more damaging than random loss.	2

SWARA end-to-end architecture for one direction of a call. Orange stages are the project's novel components; black stages are conventional. The relay in the middle forwards ciphertext it cannot read.	9

The 16-byte SWARA header and the composition of a wideband media datagram. The header and the optional 16-byte telemetry block are AEAD associated data: readable on the path, unforgeable. Everything after them is ciphertext.	10

PI-AFEC on three consecutive groups of k = 4 frames. The silent group receives no parity; its budget moves to the group carrying the vowel onset, which additionally earns an anchor replica displaced past the expected burst length.	13

The SPARK handshake and 0-RTT path migration, with on-the-wire sizes measured from the implementation.	14

The NAVIK control loop: receiver-side estimation feeds a single R-factor objective whose decisions drive the scheduler, the codec and the packet marking.	16

A triangle-inequality violation: the direct A–B path crosses a congested exchange while the two-hop path through C is far cheaper in voice-cost terms.	17

The Gilbert–Elliott loss model used by both the simulator and the live injector.	18

The four threads of a CallSession and the state they share.	21

The web console's Overview page: headline results, the twelve-stage pipeline, and the hosted relay card.	24

The SWARA Server Control window with the server running.	25

PI score distributions by speech class (violins show the full distribution, bars the mean) and the share of frames versus the share of importance held by silence.	28

Call quality under loss (left; shaded bands ±1 s.e.) and the reduction in perceptually weighted loss relative to the baseline (right).	29

Resilience to bursty loss (left) and the mechanism responsible for each recovery (right).	30

Quality returned per byte of redundancy.	31

Too shallow loses packets, too deep loses the conversation: the R-factor curve (left) and the end-to-end validation (right).	31

Call-setup cost in round trips and bytes.	32

SETU: distribution of the cost reduction on improved pairs (left) and the share of pairs by chosen route length (right).	33

Measured call quality across realistic network profiles (dashed line: the 3.6 “acceptable” threshold).	34

Terminal output of demo.py --compare at 15 % loss, mean burst 4, 12 ms jitter (25 s calls, 2 repeats per arm).	36

Terminal output of demo.py --compare at 10 % loss, mean burst 3, 10 ms jitter.	36

SWARA Server Control, opened by double-clicking “SWARA Server.bat”. The server was started with one click; the window shows the console address, the LAN address for other laptops, the hosted relay on udp:9000 and the live server log.	44

Terminal 1 — the relay (`python server.py --port 9000`) during the call: both users authenticated by SPARK and the call being relayed “keys not held”.	45

Terminal 2 — the callee (`python client.py --user bob … --wait`) during the call: authenticated in two round trips, incoming call accepted, media flowing.	45

Terminal 3 — the caller (`python client.py --user alice … --call bob`) during the call.	45

Terminal 1 after the call: the relay's complete log for the session.	45

Terminal 2 after the call — bob's own report: 1,021 frames sent with 287 parity and 17 replica packets; 1,015 played; 14 rebuilt by FEC; 102 concealed (10.0 %); no authentication failures.	46

Terminal 3 after the call — alice's own report: 1,021 frames sent with 273 parity and 32 replicas; 1,011 played; 7 rebuilt by FEC; 139 concealed (13.7 %); no authentication failures.	46

Relay & Users page while alice and bob (command-line clients) are in a call through the hosted relay.	47

Overview page during the same call: the hosted-relay card reports two users online and one call relayed.	48

The caller's terminal for the hosted-relay call (30 s, 10 % loss, burst 3, 10 ms jitter on alice's socket).	49

The callee's terminal for the hosted-relay call.	49

Live Call Lab before a call: audio source, call length, protection scheme, codec mode and the network sliders (presets from Clean to Poor 3G).	50

Live Call Lab during a call: the frame-fate map fills from left to right (bar height = PI; teal delivered, blue rebuilt from parity, violet anchor replica, red concealed), the sent and received waveforms, and the live gauges.	51

Live Call Lab after the call: the complete frame-fate map, the three time-series charts (network as measured, adaptation, quality), the call report with playable sent and received audio, the call log and the session table.	52

A/B Benchmark page before a run: audio source, seconds per call, repeats, and the network conditions shared by all three arms.	53

A/B Benchmark after a run at 15 % loss, burst 4, 12 ms jitter (20 s per arm): per-arm cards, the verdict, four comparison charts, the like-for-like table and the audio of each arm.	54

Network Simulator: the Congested 4G profile simulated for SWARA, uniform FEC and the baseline with MOS gauges, detailed per-system figures and the loss-accounting chart.	55

Research Results — loss sweep tab, with the interactive chart, the finding, the paper figure, the figure gallery and the regeneration control.	56

Research Results — burst sweep tab.	57

Research Results — overhead efficiency tab.	58

Research Results — PI distribution tab.	59

Research Results — jitter buffer tab.	60

Research Results — handshake tab.	61

Research Results — SETU overlay tab.	62

Research Results — network profiles tab.	63

Perceptual Analyzer: synthetic speech coloured by class with the PI trace, the ground-truth / label / FEC-plan strips, the frame inspector, the PI histogram and per-class statistics.	64

FEC Playground: an RS(6, 4) group with two data shards dropped and recovered; the PI-AFEC scheduler for a vowel-onset group.	64

FEC Playground after the “Burst” button: more shards lost than the code can repair — the frame must be concealed.	65

Quality & NAVIK: the E-model calculator with its impairment breakdown and the controller's decisions across loss rates.	66

SPARK Handshake: the six messages with sizes and timings, the derived keys matching on both sides, and the 0-RTT path migration.	67

SPARK Handshake with a wrong password: the envelope fails to open locally and AUTH_CONFIRM is never sent.	68

Packet Anatomy: the 16-byte header packed by Header.pack(), the hex dump, and what the AEAD protects.	69

SETU Overlay: a seven-node measured mesh, the converged route for the selected pair, and all pairs ranked by gain.	70

Relay & Users (idle): hosted relay status, the ready-to-paste client commands, the relay log and the user registry.	71

System & API: runtime, packages, the source-module inventory and every endpoint.	72

The generated OpenAPI documentation (Swagger UI) at /docs — the first five endpoint groups of thirty-seven documented routes.	73

## List of Tables

Summary of the most relevant prior work and its relation to SWARA.	6

Research gaps and the SWARA component that addresses each.	7

Objectives and success criteria.	8

Header fields.	10

Datagram types.	10

Technology stack and versions used for the reported results.	19

Source modules and their responsibilities.	20

Web console pages.	23

Experimental configuration.	27

Perceptual importance score by speech class (0–255).	28

Loss sweep: predicted MOS (G.107) and perceptually weighted loss, mean of 5 seeds.	28

Burst sweep at 12 % loss: MOS and SWARA's recoveries per call by mechanism, mean of 4 seeds.	29

Overhead efficiency at 12 % loss, burst 4: MOS and redundancy actually spent for each nominal budget ρ.	30

Jitter-buffer depth: predicted R-factor and measured end-to-end MOS.	31

Measured handshake and media-protection cost.	32

Security properties of SPARK and how each is obtained.	32

Named profiles: predicted MOS, residual and perceptual loss, mouth-to-ear delay and redundancy, mean of 4 seeds.	34

Diagnosis of the injector defect: one call at 15 % loss, burst 4, 12 ms jitter, sampled once per second.	35

Live A/B comparison (demo.py, 25 s × 2 repeats, injector at both sockets).	35

Repository contents.	42

# Abbreviations

ADPCM  Adaptive Differential Pulse-Code Modulation (IMA variant, 4 bits per sample)

AEAD  Authenticated Encryption with Associated Data

AF41 / EF / BE  Assured Forwarding class 4 (DSCP 34), Expedited Forwarding (DSCP 46), Best Effort (DSCP 0)

AIMD  Additive-Increase / Multiplicative-Decrease congestion control

API / REST  Application Programming Interface / Representational State Transfer

CID  Connection Identifier (32-bit, carried in every SWARA header)

CRC  Cyclic Redundancy Check

DSCP  Differentiated Services Code Point (6 bits of the IP header)

DTLS-SRTP  Datagram TLS keying for the Secure Real-time Transport Protocol (the WebRTC baseline)

E-model  ITU-T G.107 computational model mapping impairments to a transmission rating R

FEC  Forward Error Correction

GF(2⁸)  Galois field with 256 elements, the arithmetic of the Reed–Solomon code

GIL  Global Interpreter Lock (CPython)

HKDF / HMAC  HMAC-based Key Derivation Function / Hash-based Message Authentication Code

MDS  Maximum Distance Separable (any k of k+r shards reconstruct the group)

MOS  Mean Opinion Score (1–5, predicted here from R by G.107)

NAT  Network Address Translation

NAVIK  Network-Adaptive Voice-quality Integrated Kontroller (the joint optimiser)

OPAQUE  Asymmetric password-authenticated key exchange (envelope construction borrowed by SPARK)

OWD  One-Way Delay

PBKDF2  Password-Based Key Derivation Function 2

PI  Perceptual Importance (0–255, one octet per frame)

PI-AFEC  Perceptual-Importance-weighted Adaptive FEC

PLC  Packet Loss Concealment

Ppl  Packet-loss probability as consumed by the E-model (percent)

QoS  Quality of Service

ROC  Rollover Counter (SRTP-style reconstruction of a 16-bit sequence field)

RS(n, k)  Systematic Reed–Solomon erasure code with k data and n−k parity shards

RTP / RTCP  Real-time Transport Protocol / RTP Control Protocol

RTT  Round-Trip Time

SETU  Self-Evaluating Topology Underlay (the relay-selection overlay; Hindi “bridge”)

SIP  Session Initiation Protocol

SPARK  Stateless Password-Authenticated Retry & Key-agreement

SSE  Server-Sent Events (console streaming)

SWARA  Secure Wideband Adaptive Real-time Audio over UDP (Hindi/Sanskrit “voice, note”)

UDP  User Datagram Protocol

VAD  Voice Activity Detection

VoIP  Voice over Internet Protocol

WRED  Weighted Random Early Detection

X25519  Elliptic-curve Diffie–Hellman over Curve25519

ZCR  Zero-Crossing Rate

# 1   Introduction

## 1.1   Background

Interactive voice is the application that first made the Internet's best-effort service model visible to ordinary users. Speech is captured in short frames — 20 ms is the common choice — and each frame is placed in its own datagram, because a conversation cannot wait for a retransmission: by the time a lost frame could be resent and delivered it is already too late to play. Voice therefore runs over UDP [46] with RTP framing [1], accepts that some datagrams will be lost, delayed or reordered, and relies on the receiver to hide the damage. Three impairments dominate the listener's experience: packet loss, which removes audio; delay, which, past roughly 150 ms mouth-to-ear, makes a conversation feel like a walkie-talkie exchange [3]; and jitter, the variation in delay, which must be absorbed by a playout buffer that itself adds delay.

Loss on real paths is not a coin flip. Queues overflow, radio links fade and handovers stall, so losses arrive in bursts [16], [17]. A single lost 20 ms frame can be concealed almost inaudibly by repeating or interpolating its neighbours [15]; a burst of eight consecutive losses cannot, because 160 ms of speech cannot be invented. Any protection scheme and any evaluation that assumes independent loss therefore overstates how well it will work in a hostel Wi-Fi network or on a congested 4G cell.

The standard defence is forward error correction (FEC): the sender transmits redundant information so that the receiver can rebuild a lost frame without a round trip [5], [6]. Modern stacks adapt the amount of redundancy to the measured loss rate [12], [47]. They all share one simplifying assumption that this project sets out to remove: every frame is worth the same amount of protection.

## 1.2   Motivation

Speech intelligibility is carried by a minority of frames. Voiced vowel nuclei carry the formant structure that identifies a phoneme; the onsets of vowels and plosives carry the transient cues that mark word boundaries; unvoiced fricatives such as /s/ and /f/ are noise-like and tolerate substitution; and roughly a third of a talker's frames in a two-way conversation are silence or background noise, which are perceptually free to lose. A loss-driven FEC scheme spends exactly as much protecting a frame of silence as a frame carrying a vowel onset. The parity packets spent on silence are pure overhead; the parity packets not spent on the onset are the ones the listener notices.

Three further observations motivated the design. First, the jitter buffer and the protection scheme are usually tuned independently, yet parity can only rescue a frame that is still waiting in the buffer when the parity arrives, so buffer depth bounds the usefulness of redundancy. Second, loss caused by congestion and loss caused by a fading link call for opposite responses — back off, or protect harder — and a controller that cannot tell them apart makes the wrong choice half the time [43]. Third, the security layer is usually bolted on afterwards: DTLS-SRTP [21] costs about three round trips and some two kilobytes before the first frame flows, offers no graceful handover when a phone moves from Wi-Fi to mobile data, and a naive UDP server that answers unauthenticated packets with larger ones becomes a reflection amplifier.

Figure 1.  Why not all losses are equal. Left: the perceptual damage weight used throughout this project rises superlinearly with a frame's importance and is penalised for consecutive losses. Centre: the E-model's delay impairment has a knee at 177.3 ms, so an over-deep jitter buffer is a quality defect. Right: for a fixed loss rate, clustered (bursty) loss is far more damaging than random loss.

## 1.3   Problem statement

Design, implement and evaluate a voice-chat system over UDP that (i) allocates loss protection by the perceptual importance of each frame rather than uniformly, (ii) selects codec bitrate, redundancy, packet marking and playout depth jointly against a single objective quality model, (iii) authenticates users mutually with a password that never crosses the wire, in a minimal number of round trips, without creating amplification or denial-of-service exposure, and (iv) relays and routes media through infrastructure that cannot read it — and demonstrate the resulting quality gains with a like-for-like, reproducible evaluation against the conventional alternatives.

## 1.4   Scope and contributions

The project delivers a working application, not only a model. Real UDP sockets carry real ADPCM-coded speech, sealed with real X25519 / HKDF / ChaCha20-Poly1305 cryptography, between a relay server and command-line or web-console clients. The contributions are:

- PI analyser — a causal, single-frame-latency perceptual importance scorer whose output travels as authenticated associated data so that every element on the path can act on it without reading the speech (Section 5.3).

- PI-AFEC — a systematic Reed–Solomon erasure code over GF(2⁸) whose per-group parity is set by group importance under a fixed budget, plus deadline-bounded anchor replicas for the most important frames and a burst mode that redirects the budget when block coding is futile (Section 5.4).

- SPARK — a stateless, password-authenticated key agreement with an address-validation cookie, an OPAQUE-style credential envelope, triple Diffie–Hellman with forward secrecy, encrypted resumption tickets and zero-round-trip path migration (Section 5.5).

- NAVIK — a joint controller that maximises the predicted E-model rating over a codec-by-redundancy grid every 100 ms, with a burst-aware residual-loss model and a delay-trend discriminator between congestion and link loss (Section 5.6).

- R-factor-optimal playout — a jitter buffer whose depth maximises R rather than meeting a loss target, with perceptually weighted late-packet cost and a floor that guarantees the protection being paid for can actually be used (Section 5.7).

- SETU — a distance-vector overlay with a composite voice-aware link cost in which loss is expressed in decibels so that it adds along a path exactly as delay does (Section 5.8).

- A reproducible evaluation harness — a time-based Gilbert–Elliott impairment injector, an offline simulator that runs identical speech and identical channel realisations through SWARA and two comparators, and a web console with instrumented live calls, A/B benchmarks and an interactive toolkit (Sections 5.10 and 6).

Out of scope are a formal security proof of SPARK, subjective listening tests (quality is predicted with the G.107 model), recorded human speech (the corpus is a source-filter synthesiser, chosen to keep the evaluation self-contained and licence-free), and group calls with more than two participants, although SETU is designed with them in mind.

## 1.5   Organisation of the report

Chapter 2 surveys the literature on loss recovery, unequal protection, playout scheduling, quality assessment, secure real-time transport and overlay routing. Chapter 3 distils the research gaps and Chapter 4 states the objectives. Chapter 5 presents the proposed design component by component, with the wire protocol, algorithms and equations. Chapter 6 describes the implementation: modules, threads, the relay server and clients, the web console and the launcher. Chapter 7 reports the results of the simulated evaluation, the live end-to-end benchmarks and the diagnostic investigation that accompanied them, and discusses their limits. Chapter 8 concludes. Appendix A gives the repository, Appendix B the screenshots of every part of the running system, and Appendix C the source code.

# 2   Literature Survey

This chapter reviews the work on which SWARA builds, grouped by the problem each line of research addresses. Table 1 summarises the works most directly relevant to the design and notes what each leaves open.

## 2.1   Packet loss recovery for streaming audio

Perkins, Hodson and Hardman's survey [5] remains the standard taxonomy of loss recovery for audio: sender-driven techniques (retransmission, forward error correction, interleaving) and receiver-driven concealment (insertion, interpolation, regeneration). Its central observation is that retransmission is unusable for interactive voice, interleaving adds a delay proportional to its depth, and FEC is the only sender-side mechanism that repairs loss within a frame or two of delay. Bolot, Fosse-Parisis and Towsley [6] made FEC adaptive, choosing the redundancy level from the measured loss process with a Gilbert model and showing that rate-based adaptation is necessary because fixed redundancy either wastes bandwidth or fails when loss rises. The same loss-driven philosophy underlies the IETF payload formats for generic FEC [47] and the flexible FEC used by WebRTC [12], and the in-band low-bit-rate redundancy (LBRR) of the Opus codec [11].

The erasure codes themselves are well understood. Reed and Solomon's polynomial codes [7] are maximum-distance-separable: any k of k+r code words reconstruct the k data words. Rizzo [8] showed that a systematic Vandermonde-based construction over GF(2⁸) can be implemented with table lookups fast enough for software packet-level FEC, and that systematic codes have the decisive practical property that data packets are sent unmodified, so a group that arrives intact costs no decoding at all. SWARA's fec.py follows this construction.

## 2.2   Unequal protection and perceptual weighting

Mohr, Riskin and Ladner [9] introduced unequal loss protection for progressively coded images: because early bits of a progressive bit-stream matter more than later ones, assigning more parity to the important prefix degrades quality gracefully instead of catastrophically. The idea was adopted widely for layered video, but far less for speech, where the frames are not naturally ordered by importance — importance must be estimated from content. Jiang and Schulzrinne [10] compared loss-repair methods for VoIP under bursty loss and showed that the perceived benefit of FEC depends strongly on burst length and on where the redundancy is placed in time, concluding that loss repair must be optimised against perceived quality rather than raw loss. Clark [18] modelled how burst loss and recency affect subjective quality and argued that an equal-weight loss count is a poor predictor of what a listener reports.

## 2.3   Playout scheduling and concealment

Ramjee et al. [13] formulated adaptive playout as the problem of trading late-packet loss against added delay and proposed per-talkspurt adjustment driven by delay statistics. Liang, Färber and Girod [14] moved the adjustment inside talkspurts using time-scale modification and showed that playout scheduling and concealment should be designed together. ITU-T G.711 Appendix I [15] standardised a low-complexity concealment algorithm based on pitch-synchronous repetition with attenuation, establishing the practice that concealment should fade rather than repeat indefinitely. In all of this work the buffer objective is a loss target; the delay the buffer adds enters only as a secondary constraint.

## 2.4   Objective quality assessment

ITU-T P.800 [4] defines the Mean Opinion Score obtained from listening panels; because panels are expensive, transmission planning uses the ITU-T G.107 E-model [2], which combines a base rating with additive impairments for delay (Id), equipment and packet loss (Ie,eff) and an advantage factor (A) into a rating R on a 0–100 scale that maps to MOS. The model's Annex on bursty loss introduces the burst ratio, by which the same average loss is charged more when it is clustered. Sun and Ifeachor [19] validated and extended E-model-style prediction for VoIP codecs and loss conditions, and recommended it as the objective to optimise inside adaptive systems. SWARA takes this literally: the E-model is the objective of both the controller and the jitter buffer.

## 2.5   Channel models

Gilbert [16] and Elliott [17] introduced the two-state Markov model of a burst-noise channel; its packet-level form, with a good state that rarely drops and a bad state that almost always drops, is the standard model for Internet packet loss and is parameterised naturally by (loss rate, mean burst length). Its use for evaluating voice protection goes back at least to Bolot et al. [6] and Jiang and Schulzrinne [10].

## 2.6   Secure real-time transport and key establishment

SRTP [20] protects RTP media with an AEAD-style construction keyed out of band, reconstructing a 48-bit packet index from a 16-bit sequence number and a rollover counter and rejecting replays with a sliding window. DTLS-SRTP [21] derives SRTP keys from a DTLS handshake [22]; it is the WebRTC standard and the baseline against which SWARA's handshake is compared. DTLS introduced the stateless cookie exchange (HelloVerifyRequest) so that a server does not allocate state for a spoofed source, and QUIC [23] generalised it into the Retry mechanism, bounded amplification to a 3:1 ratio before address validation, and added connection identifiers that let a connection migrate to a new address without a new handshake. OPAQUE [24] is the asymmetric password-authenticated key exchange adopted by the IETF; its key idea is that the server stores an envelope — the client's long-term key sealed under a key stretched from the password — and never a password-equivalent, so a stolen database yields only an offline-attack target. SWARA composes these ideas with X25519 [25], HKDF [27], PBKDF2 [28] and ChaCha20-Poly1305 [26].

## 2.7   Network QoS and overlay routing

Differentiated Services [29] lets a sender mark each packet with a code point that routers map to a per-hop behaviour: Expedited Forwarding [30] for low-delay traffic and the Assured Forwarding classes [31] with their drop precedences. Combined with Random Early Detection [32] and its weighted variant, a congested router drops low-precedence packets first — a mechanism that can be made perceptually informed only if the marking reflects content. Andersen et al. [33] demonstrated with Resilient Overlay Networks that the Internet's routing frequently violates the triangle inequality at the application layer and that a small overlay measuring its own links can route around failures and congestion within seconds. Distance-vector routing with split horizon and poisoned reverse [34] gives such an overlay a convergent algorithm. Signalling reliability over UDP is handled in SIP [35] by retransmitting INVITE with an identifier that the callee answers idempotently.

Table 1.  Summary of the most relevant prior work and its relation to SWARA.

| Work | Contribution | Limitation addressed by SWARA |
| --- | --- | --- |
| Perkins et al. 1998 [5] | Taxonomy of audio loss recovery; FEC as the only interactive sender-side repair | Treats all frames as equal; no content awareness |
| Bolot et al. 1999 [6] | Loss-driven adaptive FEC with a Gilbert channel model | Redundancy chosen from network state alone |
| Rizzo 1997 [8]; Reed–Solomon 1960 [7] | Fast systematic MDS erasure codes over GF(2⁸) | Block codes fail when bursts span the group; no time displacement |
| Mohr et al. 2000 [9] | Unequal loss protection by data importance (images) | Importance is structural, not estimated from speech content |
| Jiang & Schulzrinne 2002 [10] | Repair methods must be judged by perceived quality under bursty loss | No per-frame importance signal to act on |
| Ramjee et al. 1994 [13]; Liang et al. 2003 [14] | Adaptive playout against late-loss targets | Delay added by the buffer is not part of the objective |
| ITU-T G.107 [2]; Sun & Ifeachor 2006 [19] | E-model as an objective quality currency | Used for assessment, rarely as the controller's objective |
| SRTP / DTLS-SRTP [20], [21] | Standard media protection and keying | 3 RTT, ~1.9 kB setup; no password authentication; no 0-RTT migration |
| QUIC [23]; DTLS cookies [22] | Stateless address validation, amplification limit, connection migration | Not applied to a password-authenticated media relay |
| OPAQUE [24] | Envelope-based asymmetric PAKE | Composition with a datagram handshake and media keys left open |
| DiffServ [29]–[31]; RED [32] | Per-packet marking and precedence-aware dropping | Marking is static per flow, not per-frame importance |
| RON 2001 [33]; RIP [34] | Application-layer overlays beat direct routes; distance-vector convergence | Link cost not expressed in voice-quality terms |

# 3   Research Gap

Reading the literature of Chapter 2 as a whole exposes six gaps between what is known and what deployed voice stacks do. SWARA is organised around closing each of them.

Table 2.  Research gaps and the SWARA component that addresses each.

| # | Gap | Evidence from the survey | SWARA response |
| --- | --- | --- | --- |
| G1 | Loss protection is content-blind: every frame receives the same redundancy | Adaptive FEC [6], [12], [47] adapts to loss rate only; unequal protection [9] was developed for images, not speech | PI analyser + PI-AFEC: parity by group importance, anchors for onsets |
| G2 | Block FEC cannot survive bursts that span the group, and interleaving is too slow for conversation | Interleaving delay [5]; burst sensitivity of repair [10] | Burst mode: budget redirected to time-displaced replicas of the most important frames |
| G3 | Playout depth and protection are decided separately although depth bounds what parity and replicas can rescue | Playout work [13], [14] targets late loss; FEC work assumes the buffer is deep enough | R-factor-optimal buffer with a protection floor; replica displacement bounded by depth |
| G4 | Quality proxies (loss, delay, throughput) conflict; no single objective ties codec, redundancy and marking together | E-model recommended as objective [19] but used for assessment | NAVIK maximises predicted R over a codec × redundancy grid with a burst-aware residual-loss model |
| G5 | Congestion loss and link loss are treated alike, so controllers back off when they should protect and protect when they should back off | AIMD reacts to any loss [43]; delay-gradient controllers exist for throughput, not for voice protection | Capacity estimate reacts to loss only when the one-way-delay trend rises |
| G6 | Secure call setup is expensive, stateful and brittle: multi-RTT, large, password-less, no handover; naive UDP servers amplify | DTLS-SRTP cost [21]; cookie/Retry and migration exist in QUIC [23] but not in a password-authenticated media service | SPARK: 2-RTT PAKE with stateless cookie, amplification < 1, ticket-based 0-RTT migration; blind relay |
| G7 | Direct Internet paths are often not the best paths for voice, and overlay costs ignore jitter and loss | RON [33] uses latency/loss heuristics; RIP [34] uses hop count | SETU: cost = owd + 2·jitter + 45·loss_dB, additive along paths; split horizon with poisoned reverse |

A methodological gap also became apparent during the project and is recorded here because it shaped the evaluation: comparative measurements of protection schemes are easily contaminated by the measuring instrument. Stepping a loss model once per packet gives a scheme that sends more packets a different channel from the one it is being compared with; a scheduler thread that busy-waits distorts every timing the controller observes. Section 5.10 and Section 7.9 describe how both artefacts were found and removed.

# 4   Objectives

The project's objectives follow directly from the gaps of Chapter 3. Each is stated with the measurable criterion used to judge it in Chapter 7.

Table 3.  Objectives and success criteria.

| # | Objective | Success criterion |
| --- | --- | --- |
| O1 | Score every 20 ms frame by its perceptual importance, causally, and carry the score in the packet header where the network can read but not forge it | Silence, unvoiced, voiced and onset frames occupy distinct score bands; silence holds < 2 % of total importance; score is AEAD associated data |
| O2 | Spend a fixed redundancy budget by importance (PI-AFEC) and protect onsets with deadline-bounded anchor replicas | At equal budget, lower perceptually weighted loss and higher predicted MOS than uniform adaptive FEC across a loss sweep; equal quality for less redundancy |
| O3 | Survive bursty loss where block coding fails | Advantage over uniform FEC persists as mean burst length rises; anchors account for recoveries when FEC recoveries collapse |
| O4 | Choose codec bitrate, redundancy and DSCP marking jointly by maximising the E-model rating, discriminating congestion from link loss | Controller never offers more than the capacity estimate; redundancy is held on lossy-but-uncongested paths; decision cost well below the 100 ms epoch |
| O5 | Size the playout buffer to maximise R and couple it to the protection decision | The depth chosen by the optimiser coincides with the depth that maximises measured end-to-end MOS |
| O6 | Authenticate users mutually with a password that never crosses the wire, in two round trips, with amplification below one and zero-round-trip migration | 397-byte, 2-RTT handshake; 20-byte RETRY to a 64-byte HELLO; migration in 0 RTT; the relay holds no media keys |
| O7 | Select relay paths that beat the direct route when the triangle inequality fails | Overlay converges in a handful of rounds; a measurable share of peer pairs gains from relaying |
| O8 | Deliver a usable application and a reproducible evaluation | Real calls between command-line clients through the relay; a web console that runs instrumented calls, benchmarks and tools; every figure regenerable from one script |

# 5   Proposed Work

SWARA is a layered design in which one idea — judge loss by what the listener loses — propagates through every layer. This chapter presents the system top-down: the end-to-end architecture, the wire protocol, and then each component with its algorithm and the equations it implements. The names of the components are Hindi/Sanskrit words chosen to be memorable: swara (voice, musical note), navik (navigator), setu (bridge), spark (an acronym).

## 5.1   System architecture

Figure 2 shows one direction of one call. On the sender, 20 ms frames are captured, coded with a per-frame-independent ADPCM codec, scored by the perceptual importance analyser, grouped for Reed–Solomon parity and anchor replicas by the PI-AFEC scheduler, sealed with an AEAD cipher whose associated data is the header, marked with a DiffServ code point derived from the frame's importance, and sent on a plain UDP socket. The relay authenticates users, sets up calls and forwards ciphertext by connection identifier; it never holds a media key. On the receiver, datagrams are authenticated, de-duplicated with a replay window, reassembled — rebuilding missing frames from parity or replicas — scheduled by an R-factor-optimal playout buffer, decoded, concealed where nothing could be rebuilt, and played. A 16-byte telemetry report rides in the header of the reverse-direction media stream, closing the control loop without a single extra packet.

Figure 2.  SWARA end-to-end architecture for one direction of a call. Orange stages are the project's novel components; black stages are conventional. The relay in the middle forwards ciphertext it cannot read.

## 5.2   Wire protocol

Every SWARA datagram is a single UDP payload of at most 1,200 bytes — below the smallest plausible path MTU, so no IP fragmentation ever occurs. There is no stream, no retransmission by default and no head-of-line blocking: a lost packet is a lost 20 ms of audio and nothing more. All datagrams begin with the 16-byte fixed header of Figure 3, packed in network byte order as struct "!BBHIIBBBB".

Figure 3.  The 16-byte SWARA header and the composition of a wideband media datagram. The header and the optional 16-byte telemetry block are AEAD associated data: readable on the path, unforgeable. Everything after them is ciphertext.

Table 4.  Header fields.

| Field | Bits | Meaning |
| --- | --- | --- |
| Version / Type | 4 + 4 | Protocol version (1) and datagram type (Table 5) |
| Flags | 8 | TELEMETRY_PIGGYBACK 0x01, MARKER 0x02 (first frame of a talkspurt), DSCP_EF 0x04, ENCRYPTED 0x08, LAST_IN_GROUP 0x10, RESUMED 0x20 |
| Sequence number | 16 | Wire sequence; the receiver unwraps it to a 64-bit counter with an SRTP-style rollover counter, and that counter is the AEAD nonce input |
| Connection ID | 32 | Derived from the handshake (sessions) or minted by the relay (calls); survives NAT rebinding and path migration; the relay routes on it |
| Timestamp | 32 | Sender clock in milliseconds; feeds the RFC 3550 jitter estimate and the one-way-delay trend |
| PI score | 8 | Perceptual importance 0–255 — authenticated but not encrypted, so relays and DiffServ routers can act on it |
| FEC group / FEC index | 8 + 8 | Group identifier and position within the group; bit 7 of the index marks a parity shard |
| Reserved | 8 | Zero |

Table 5.  Datagram types.

| Type | Code | Direction | Purpose |
| --- | --- | --- | --- |
| HELLO | 0 | client → server | Stateless probe, padded to 64 bytes |
| RETRY | 1 | server → client | 20-byte address-validation cookie |
| AUTH_INIT | 2 | client → server | Cookie + client ephemeral public key + username |
| AUTH_RESP | 3 | server → client | Salt + credential envelope + server ephemeral and static keys |
| AUTH_CONFIRM | 4 | client → server | Key-confirmation tag |
| AUTH_TICKET | 5 | server → client | Connection ID + encrypted resumption ticket |
| MEDIA | 6 | peer ↔ peer | One coded speech frame (plus piggybacked replicas) |
| FEC | 7 | peer ↔ peer | One Reed–Solomon parity shard |
| ANCHOR / TELEMETRY / NACK | 8–10 | peer ↔ peer | Reserved for standalone replicas, standalone telemetry and selective NACK (replicas and telemetry are normally piggybacked) |
| PATH_MIGRATE / PATH_ACK | 11–12 | client ↔ server | 0-RTT authenticated path migration and its validation |
| OVERLAY_LSA | 13 | peer ↔ peer | SETU distance-vector advertisement |
| BYE | 14 | any | Teardown |
| CONTROL | 15 | client ↔ server | Encrypted JSON signalling: call, incoming, call_ready, call_failed, hangup, ping/pong, who, bye |

A media datagram's plaintext begins with a one-byte replica count followed by that many {displacement (1), length (2), payload} records and then the primary frame. Displacement rather than an absolute sequence number is used because a displacement survives 16-bit wrap. A parity datagram carries {base sequence (2), k (1), r (1), shard length (2)} followed by the shard. The telemetry block packs loss (×10⁴), mean burst (×100), jitter (×100 ms) and MOS (×1000) into four 16-bit fields with 8 reserved bytes. For the syllabus's error-control topic, packet.py also implements CRC-32 (the Ethernet FCS polynomial), the RFC 1071 Internet checksum that UDP itself carries [38], two-dimensional parity and Hamming(7,4) [39]; the design discussion in Section 5.4 explains why a real-time voice transport uses a detection code plus erasure coding rather than a bit-level correcting code.

## 5.3   Perceptual importance analysis

The PI analyser converts each 320-sample frame (20 ms at 16 kHz) into a single octet estimating how much intelligibility the listener would lose if that frame were missing. It is strictly causal — every feature uses only the current frame and state accumulated from past frames — because a real encoder cannot look ahead without adding delay. Five features are computed in O(N) or O(N log N):

- Energy E in dB, normalised to E′ ∈ [0, 1] against an adaptive noise floor (fast to fall, slow to rise) and an adaptive peak (fast attack, slow release), with a minimum span of 12 dB.

- Voicing V: the normalised autocorrelation peak in the 60–400 Hz pitch-lag range, computed by FFT, then rescaled against its measured operating range, V = clip((V_raw − 0.12)/0.45).

- Zero-crossing rate ZCR: the classic voiced/unvoiced discriminator; a high ZCR with low voicing identifies fricative noise.

- Spectral flux F: the half-wave-rectified difference between successive Hann-windowed magnitude spectra — energy that has just appeared, the signature of an onset.

- Voice activity: a frame is speech if it is more than 6 dB above the noise floor, or more than 4 dB above it with F > 0.62 (which admits 8 ms plosive bursts whose average energy is low).

PI_raw = 0.30·E′ + 0.34·V_eff + 0.20·F + 0.16·onset,     V_eff = 0.6·V if (V < 0.4 and ZCR > 0.20) else V	(1)

PI = 255 · min(1, boost · (0.12 + 0.88 · clip(PI_raw, 0, 1)))   for speech;   PI = 255 · 0.06 · E′   otherwise	(2)

The onset term is a rising-edge detector, onset = clip(3.2 · (E′ − E′_prev)); a hard onset — the first speech frame after silence — forces onset = 1 and applies a multiplicative boost of 1.45, because losing the frame that begins a talkspurt destroys the word-boundary cue. The design targets, verified on the synthetic corpus (Section 7.2), are silence 0–25, steady fricatives 50–110, steady vowels 150–210 and onsets 210–255. The score's companion is the damage weight that turns a loss into a perceptual cost:

d(PI, run) = (PI / 255)^1.6 · (1 + 0.45 · min(run, 4))	(3)

The curve is superlinear — a frame twice as important is roughly three times as costly to lose — and consecutive losses are penalised because concealment degrades sharply once it has to invent more than about 40 ms of speech. Equation (3) is used in three places with the same constants: to score lost frames in the evaluation, to weight late packets in the jitter buffer, and to convert accumulated damage into the perceptual loss percentage fed to the E-model, Ppl_perc = 100 · Σ d / N. Using one function everywhere means the optimiser and the evaluator agree by construction.

## 5.4   PI-AFEC: importance-weighted adaptive FEC

Erasure code. Groups of k media frames are extended with r parity shards by a systematic Reed–Solomon code over GF(2⁸) with primitive polynomial 0x11D (the field of CD-ROM, QR codes and RAID-6). Multiplication is a 256 × 256 table so encoding is a sequence of vectorised XORs; the generator rows are Cauchy-style, m[i, j] = (a_i ⊕ b_j)⁻¹ with distinct a_i = i + 1 and b_j = 128 + j, normalised so that the r = 1 row reduces to plain XOR. Any square sub-matrix is invertible, which is the MDS property: any k of the k + r shards reconstruct the group. Decoding solves the small system for the missing data shards by Gauss–Jordan elimination over the field. The module verifies the MDS property at import time.

Why erasure coding rather than Hamming. A UDP datagram either arrives intact or not at all: the IP and UDP checksums and the AEAD tag discard corrupted packets, so the channel seen by the application is an erasure channel in units of whole frames. A bit-level correcting code such as Hamming(7,4) spends 75 % overhead correcting single-bit errors that the link layer has already removed; a packet-level erasure code spends its overhead on the failure mode that actually occurs.

Scheduler. For each group the scheduler computes the mean normalised importance Ḡ = mean(PI)/255 and a weight w = Ḡ^γ with γ = 0.65 (so moderately important groups are not starved). The weight is normalised by its own running mean w̄ so that the expected spend equals exactly the budget k·ρ handed down by the controller — the scheme redistributes the budget, it does not quietly discount it — and the group's budget in packets is

budget = k · ρ · clip(w / w̄, 0, 4),      with  budget = 0  whenever  Ḡ < 0.08  (a silent group)	(4)

The budget is split between parity and anchor replicas according to the estimated mean burst length B. An RS(k + r, k) group survives only if at least k of its k + r packets arrive; when the burst approaches the group's own span that condition fails almost every time the channel goes bad and every parity byte is wasted. Classical coding theory answers with interleaving, but an interleaver of depth D over k frames costs k·D frame times of delay — 480 ms for D = 3, k = 8 — and destroys interactivity. SWARA instead interleaves only what matters: in burst mode (B ≥ 0.75 k) the shares are 45 % parity / 55 % replicas, otherwise 85 % / 15 %. Fractional budgets are carried between groups rather than rounded away (with k = 4 a per-group budget of 0.4 packets would otherwise round to zero every time). Up to four replicas go to the highest-PI frames of the group, provided PI ≥ 170, each displaced by

D = clip(round(B) + 2, 3, 9) + 2 · slot   frames,   bounded in the live system by  (depth − spread₉₅ − 20 ms) / 20 ms	(5)

so that the copy clears the burst that killed the original and still beats the original's own playout instant after suffering the same delay spread as everything else. A replica is piggybacked onto an existing media packet — it costs only its payload bytes, not a second set of IP, UDP and SWARA headers. The uniform comparator used throughout the evaluation is the same code with r = round(k·ρ) for every group (probabilistically honouring fractions): a strong, correctly adapting baseline that simply cannot tell a vowel from a silence.

Figure 4.  PI-AFEC on three consecutive groups of k = 4 frames. The silent group receives no parity; its budget moves to the group carrying the vowel onset, which additionally earns an anchor replica displaced past the expected burst length.

## 5.5   SPARK: stateless password-authenticated key agreement

The authentication problem for voice over UDP is not “how do I check a password”. It is: (a) UDP is trivially spoofable, so an unauthenticated first packet must never make the server allocate memory or emit a larger reply, or the service becomes a DDoS amplifier; (b) a password must never cross the wire, and a stolen server database must not yield anything replayable; (c) the handshake must not cost so many round trips that call setup feels slow — and after a Wi-Fi-to-mobile handover it must cost none, because the user is mid-sentence. SPARK addresses all three in the six messages of Figure 5.

Figure 5.  The SPARK handshake and 0-RTT path migration, with on-the-wire sizes measured from the implementation.

- Registration (once, out of band). The password is stretched with PBKDF2-HMAC-SHA256 (200,000 iterations, 16-byte salt) [28] into a key that seals an envelope: the client's static X25519 private and public keys under ChaCha20-Poly1305 with the username as associated data. The server stores only {salt, envelope, client static public key} — 128 bytes and nothing resembling a password or a verifier [24].

- HELLO → RETRY. The client sends a 64-byte padded probe revealing no identity. The server replies with a 20-byte cookie, HMAC(cookie_key, ip ‖ port ‖ epoch)[:16] with a 30-second epoch, allocating no state. Because the reply is smaller than the request the amplification factor is 20/64 = 0.31 [22], [23].

- AUTH_INIT. The client echoes the cookie — proving it can receive at the address it claims — with an ephemeral X25519 public key and its username. Only now does the server do work. For an unknown user it answers with a random envelope of identical shape, so the protocol is not a user-enumeration oracle.

- AUTH_RESP. Salt, envelope, a fresh server ephemeral public key and the server's static public key.

- AUTH_CONFIRM. The client re-derives the stretched key, opens the envelope (a wrong password fails locally and nothing is sent), and computes three Diffie–Hellman shares: dh1 = X25519(c_eph, s_static) authenticates the server, dh2 = X25519(c_static, s_eph) authenticates the client, dh3 = X25519(c_eph, s_eph) gives forward secrecy. With th = SHA-256 over every length-prefixed handshake byte, MS = HKDF-Extract(th, dh1 ‖ dh2 ‖ dh3) and the connection id and the keys k_c2s, k_s2c, k_resume are HKDF-Expand(MS, label) [25], [27]. The client proves key knowledge with tag = HMAC(k_c2s, “client finished” ‖ th)[:16].

- AUTH_TICKET. The server verifies the tag, binds the CID to the client's address, and issues ticket = nonce ‖ AEAD(K_ticket, cid ‖ expiry ‖ k_resume ‖ user). It keeps no per-session table for resumption.

- Path migration (0 RTT). From a new address the client sends ticket ‖ AEAD(k_resume, challenge). The server decrypts its own ticket, verifies the proof — an off-path attacker replaying the ticket alone cannot forge it — and rebinds the CID. Media continues without a round trip of silence.

Media protection. Each call receives a fresh 32-byte secret minted by the relay, from which two directional keys are derived; each party receives both keys inside a CONTROL message sealed under its own session key. Media packets are sealed with ChaCha20-Poly1305 [26]; the nonce is derived from (direction, 64-bit sequence) so none is transmitted — 12 bytes saved per packet, 4.8 kbit/s per stream. The 16-bit wire sequence is unwrapped with an SRTP-style rollover counter (otherwise every packet would fail authentication after 21.8 minutes), and a 1,024-entry sliding bitmap rejects replays [20]. Because the relay forwards datagrams verbatim and holds neither key, it cannot listen to the call it carries.

## 5.6   NAVIK: joint quality control

The sender has three dials and one budget: codec bitrate b (higher b lowers the codec impairment Ie), redundancy ρ (higher ρ lowers residual loss), DSCP marking, and the available capacity C with b(1 + ρ) + H ≤ C. Conventional stacks resolve this with a cascade of independent heuristics that can fight each other — the classic pathology is FEC added in response to congestion loss, which increases the offered load and deepens the congestion. NAVIK replaces the cascade with one objective, evaluated every 100 ms over a grid of seven nominal codec rates and nine redundancy levels (63 evaluations, tens of microseconds):

maximise  R(codec(b), owd + depth + 25 ms, Ppl_res(ρ, p, B), B)   s.t.  b(1+ρ) + H ≤ 0.85·C_est,   ρ ≤ ρ_max(p)	(6)

Because R already contains the delay term, the congestion pathology cannot occur: redundancy that would push the offered rate past the estimate raises queueing delay, lowers R, and is refused. The safety rail ρ_max(p) = 0.65 for p < 3 % and max(0.15, 0.65 − 1.8 (p − 0.03)) above tapers redundancy as loss rises, since very high loss usually is congestion. H = 24 kbit/s is the header overhead of 50 packets per second of IP + UDP + SWARA + AEAD tag.

Residual-loss model. The probability that an RS(k + r, k) group cannot be decoded is the probability of more than r erasures among k + r packets. Under independent loss this is a binomial tail; under clustered loss the erasure count is over-dispersed, so NAVIK uses a beta-binomial with intra-class correlation ρ_c = (B − 1)/(B + 1). The difference matters: at 10 % loss with mean burst 4 the binomial predicts a group-failure rate about six times lower than reality, and a controller that believed it would under-provision exactly when protection is most needed. Anchor replicas are modelled independently of the block code — they survive unless an independent loss event hits them — because treating them as a modifier on RS made the controller believe protection was futile in exactly the burst-dominated conditions where the anchor is the only thing that still works.

Congestion versus loss. Naive AIMD backs off on any loss [43] — right on a wired path, wrong on a wireless one, where backing off starves the very redundancy the loss made necessary. Congestion has a signature random loss lacks: the queue fills before it overflows, so one-way delay rises first. NAVIK fits a line to the last 100 one-way-delay samples and treats loss as congestion only when the slope exceeds 8 ms/s, shrinking the capacity estimate by 12 % per epoch; loss with a flat trend is read as link loss and the rate is held (a gentle 0.5 % probe down), while a clean path lets the estimate track 1.25 × goodput.

DiffServ marking. The decision also fixes per-packet marking driven by importance: PI ≥ 160 → EF (46), 80 ≤ PI < 160 → AF41 (34), otherwise best effort [29]–[31]. A router applying WRED [32] to a congested queue drops from the lowest precedence first, so the network's own drop decision becomes perceptually informed without the router understanding — or being able to read — the speech.

Figure 6.  The NAVIK control loop: receiver-side estimation feeds a single R-factor objective whose decisions drive the scheduler, the codec and the packet marking.

## 5.7   Receiver: estimation, repair and R-factor-optimal playout

All network statistics are estimated from the media stream itself — no probe packets, no separate RTCP reports. The estimator tracks loss as 1 − seen/expected over the highest sequence number, mean burst as the mean length of the last 80 gaps, interarrival jitter with the RFC 3550 recursion J ← J + (|D| − J)/16 [1], the delay spread as the 95th minus 5th percentile of relative one-way delay, reordering and goodput.

A conventional adaptive jitter buffer targets a loss objective — “size the buffer so that at most 1 % of packets arrive late” — which treats the delay it adds as free. It is not: the E-model charges 0.024 R-points per millisecond and 0.134 per millisecond beyond 177.3 ms. SWARA chooses the depth that maximises the predicted rating directly,

d* = argmax_d  R(codec, other_delay + d, 100 · late(d) · (0.25 + 1.5 · I_late), B)   over  d ∈ {0, 10, …, 180} ms	(7)

where late(d) is the fraction of packets whose relative delay exceeds d and I_late is the mean damage weight of the packets currently arriving late. The optimum shifts deeper when late packets are important and shallower when they are silence. Depth changes are smoothed (hysteresis 0.35) because they are audible, and in the live system the depth is floored at the shallowest value at which the protection being paid for can work — (k + 1) frames for parity, (B + 2) frames plus the delay spread for replicas — on the principle that buying redundancy the buffer is too shallow to use is pure waste. Depth is changed on a running call by shifting the playout epoch: deepening is applied at once, shallowing at most 10 ms per epoch. At each 20 ms slot the playout thread takes the frame in sequence; if it is missing it attempts a Reed–Solomon reconstruction of its group (and the previous group, whose parity may have just arrived); if nothing can be rebuilt it conceals by repeating the previous frame with a gain of 1 − 0.35·run and tapered edges, fading to silence rather than buzzing.

## 5.8   SETU: voice-aware overlay routing

BGP picks paths on policy and AS-hop count, not on latency, jitter or loss, and the triangle inequality is frequently violated at the application layer: A → C → B can be measurably better than A → B [33]. SETU is a small application-layer overlay among call participants and relays that runs a distance-vector computation — Bellman–Ford with split horizon and poisoned reverse, the algorithm family of RIP [34] — over a composite link cost that expresses what voice cares about in one number:

cost(u, v) = 1.0 · owd_ms + 2.0 · jitter_ms + 45 · (−10 · log₁₀(1 − p)),      relay hop penalty = 12	(8)

Expressing loss in decibels of survival probability is what makes the cost valid for shortest-path routing: losses then add along a path exactly as delays do (a 1 % link and a 2 % link compose to 2.97 %, and −10 log(0.99) − 10 log(0.98) = −10 log(0.9702) exactly). Jitter is weighted above raw delay because it forces a deeper playout buffer. Count-to-infinity is prevented by poisoned reverse and a cost ceiling; with tens of nodes convergence takes a handful of rounds.

Figure 7.  A triangle-inequality violation: the direct A–B path crosses a congested exchange while the two-hop path through C is far cheaper in voice-cost terms.

## 5.9   Speech codec with per-frame state reset

SWARA's codec is a real IMA ADPCM [36], [37]: 4 bits per sample, 4 : 1 compression of 16-bit PCM, with one deliberate modification. Standard ADPCM carries its predictor and step index continuously from block to block; over a lossy transport that is fatal, because one lost packet makes the decoder's predictor diverge from the encoder's and every subsequent frame decodes to noise until something resynchronises. SWARA resets the state at every frame boundary and transmits the starting predictor and step index in a 3-byte preamble, so every frame is independently decodable: packet loss damages exactly the frames that were lost and nothing else. The cost is a small loss of coding efficiency; the benefit is the property on which every repair mechanism above depends. Two modes give the controller something real to switch between: wideband (16 kHz, 320 samples, 163 bytes per frame, 65.2 kbit/s) and narrowband (8 kHz via an 8-tap anti-alias FIR, 83 bytes, 33.2 kbit/s). A codec designed for a file is the wrong codec for a network.

## 5.10   Evaluation methodology

Channel model. Both the offline simulator and the live impairment injector use the Gilbert–Elliott chain of Figure 8, parameterised as an operator would — “10 % average loss with a mean burst of 4 packets” — and solved as p_BG = 1/B, p_GB = p_BG · p/(1 − p). The simulator adds a base one-way delay, a heavy-tailed log-normal jitter component (which matches wide-area delay distributions far better than a Gaussian and is what makes buffer sizing genuinely hard), a token-bucket bottleneck queue with tail drop beyond 250 ms of queueing, and optional reordering and duplication. Seven named profiles span an ideal LAN, good and busy Wi-Fi, congested 4G, poor 3G, a hostel Wi-Fi network and a satellite link.

Figure 8.  The Gilbert–Elliott loss model used by both the simulator and the live injector.

Time-based impairment. The live injector advances the chain on a 20 ms wall-clock tick, not once per packet. Physically a fade lasts a duration, not a number of packets; methodologically, stepping per packet means a configuration that transmits redundancy walks the chain further per unit time and therefore experiences a different channel from the one it is compared with — the project hit exactly that, with three arms configured for 15 % loss measuring 9.5 %, 12.3 % and 12.5 %. The injector also keeps two independent random streams (loss chain and jitter) and is reset to the same seed for every arm, so every arm faces the identical sequence of good and bad periods.

Comparators. Every result compares three systems on identical speech, identical channel realisation and — crucially — identical redundancy budget: the baseline (plain UDP, no FEC, a conventional buffer at the 95th percentile of delay plus 10 ms), uniform FEC (loss-driven adaptive FEC given exactly the redundancy SWARA spent, spread evenly over all groups — RFC 5109 / FlexFEC behaviour), and SWARA. Any gain over the second comparator is attributable to perceptual placement, not to “SWARA has FEC”. Quality is scored with the G.107 E-model using the perceptually weighted residual loss and the measured burst ratio; the corpus is a source-filter speech synthesiser (glottal pulse train or noise through formant resonators, talkspurts of 0.9–2.8 s with realistic phone durations), seeded so that every run is reproducible.

# 6   Implementation

## 6.1   Technology stack

Table 6.  Technology stack and versions used for the reported results.

| Layer | Technology | Role |
| --- | --- | --- |
| Language / runtime | Python 3.14.3 (CPython), Windows 11, 20 logical CPUs | Protocol stack, relay, clients, simulator, web back end, launcher |
| Numerics | NumPy 2.5.3 | Vectorised GF(2⁸) arithmetic, FFT-based voicing and flux, estimators |
| Cryptography | pyca/cryptography 50.0.1 | X25519, HKDF-SHA256, ChaCha20-Poly1305; PBKDF2 and HMAC from the standard library |
| Transport | socket (AF_INET, SOCK_DGRAM), IP_TOS | Real UDP datagrams with DSCP marking and 1 MB socket buffers |
| Web API | FastAPI 0.141.1, Starlette 1.6.0, Pydantic 2.13.5, Uvicorn 0.52.4 | REST + Server-Sent Events console back end, OpenAPI documentation |
| Front end | Vanilla JavaScript (ES2020), Chart.js 4.4.1 (vendored), Web Audio API | Single-page console: live charts, frame-fate map, in-browser audio playback and recording |
| Figures | Matplotlib 3.11.1 | Research figures and report diagrams |
| Audio I/O | sounddevice 0.5.6 (optional) | Microphone and speaker backends; WAV and synthetic backends need nothing |
| Desktop launcher | Tkinter | Start / stop / restart the server and relay without a terminal |

## 6.2   Repository layout

SWARA/

├── server.py              registration, SPARK authentication, presence, call setup, blind media relay

├── client.py              the voice-chat application: login, call placement, capture, playback

├── demo.py                self-contained end-to-end demonstration and A/B harness

├── api_server.py          FastAPI web console + REST API, hosts the relay inside uvicorn

├── launcher.py            Tkinter "SWARA Server Control" window

├── swara/                 the protocol stack (15 modules, ~4,000 lines)

│   ├── packet.py          wire format, header pack/unpack, CRC-32, checksum, 2-D parity, Hamming(7,4)

│   ├── perceptual.py      PI analyser and damage weight

│   ├── fec.py             GF(2^8) Reed–Solomon, PI-AFEC scheduler, uniform comparator

│   ├── auth.py            SPARK client/server, key schedule, media AEAD, ROC, replay window

│   ├── controller.py      NAVIK joint optimiser, residual-loss model, DSCP mapping

│   ├── receiver.py        network-state estimator, R-factor-optimal jitter buffer

│   ├── session.py         live call pipeline: four threads per call end

│   ├── codec.py           IMA ADPCM with per-frame reset, concealment

│   ├── net.py             UDP endpoint, DSCP, time-based Gilbert–Elliott impairment injector

│   ├── emodel.py          ITU-T G.107 E-model, perceptual Ppl

│   ├── overlay.py         SETU distance-vector overlay

│   ├── channel.py         Gilbert–Elliott channel for the offline evaluation, named profiles

│   ├── engine.py          offline harness: SWARA vs uniform FEC vs baseline

│   ├── speech.py          source-filter synthetic speech corpus

│   └── audio_io.py        microphone / speaker, WAV and synthetic sources and sinks

├── webapp/                console back end: lab.py (instrumented calls), relay.py, tools.py

├── static/                console front end: index.html, css/app.css, js/core.js, js/pages/*.js, vendor/chart.umd.min.js

├── experiments/run_all.py the eight evaluation experiments → results.json + figures/

├── figures/               fig1 … fig8 (regenerated by run_all.py)

├── users.json             credential records {salt, envelope, public key} — no passwords

└── SWARA Server.bat / start_console.bat   one-click launchers

## 6.3   Module responsibilities

Table 7.  Source modules and their responsibilities.

| Module | Lines | Responsibility |
| --- | --- | --- |
| swara/packet.py | 207 | 16-byte header, datagram types and flags, media/parity layouts, integrity primitives |
| swara/perceptual.py | 240 | Causal PI analyser (energy, voicing, ZCR, flux, onset, VAD) and damage weight |
| swara/fec.py | 398 | Table-driven GF(2⁸), systematic Reed–Solomon encode/decode, PI-AFEC and uniform schedulers |
| swara/auth.py | 475 | Registration, SPARK handshake (server and client), tickets, migration, MediaCipher, SequenceReconstructor, ReplayWindow |
| swara/controller.py | 281 | NAVIK: capacity estimate, beta-binomial group-failure model, grid search over codec × ρ, DSCP mapping |
| swara/receiver.py | 233 | NetworkStateEstimator, AdaptiveJitterBuffer (R-optimal), GroupBuffer |
| swara/session.py | 607 | CallSession: sender, receiver, playout and control threads; replica piggybacking; epoch-based playout; telemetry |
| swara/codec.py | 199 | IMA ADPCM encode/decode with per-frame preamble, narrowband resampling, concealment |
| swara/net.py | 240 | UdpEndpoint with counters and DSCP; Impairment injector (wall-clock Gilbert–Elliott, delay scheduler) |
| swara/emodel.py | 171 | R-factor, Id, Ie,eff with burst ratio, R→MOS, perceptual Ppl, quality labels |
| swara/overlay.py | 164 | SETU: link cost, distance-vector node, split horizon with poisoned reverse, convergence, path report |
| swara/channel.py | 193 | ChannelConfig, GilbertElliottChannel with queueing/reordering/duplication, named profiles |
| swara/engine.py | 342 | CallSimulator (pilot pass, planning, transmission, scoring) and the three-way compare() |
| swara/speech.py | 156 | SpeechGenerator: vowels, fricatives, plosives, silences; per-frame ground truth |
| swara/audio_io.py | 178 | AudioSource / AudioSink backends: live, WAV, synthetic, null |
| server.py | 246 | Relay: user store, SPARK phases, presence, idempotent call setup, verbatim media relay |
| client.py | 266 | Client: retransmitting handshake, INVITE-style call placement, keepalives, run_call |
| demo.py | 229 | One-process end-to-end demonstration; A/B harness with identical channel realisations |
| experiments/run_all.py | 511 | Eight experiments, results.json and all figures |
| api_server.py + webapp/ | 1,284 | REST API, SSE streaming, instrumented TracedSession, Benchmark, hosted relay, toolkit wrappers |
| static/ | ~3,000 | Console front end (14 JavaScript modules, stylesheet, HTML shell) |
| launcher.py | 482 | Desktop server-control window |

## 6.4   The live call pipeline

CallSession owns one end of one call: the codec, the protection state and the two directional keys. It starts four daemon threads (Figure 9). The sender reads a frame, encodes it, scores it, prepends any replicas queued for this slot, seals the packet with the header and telemetry as associated data, marks the socket's DSCP from the frame's importance and sends; every k frames it closes the group, asks the scheduler for a plan, queues the chosen replicas at their displacements and emits the parity shards. It is paced against a monotonic clock so that scheduling error does not accumulate into drift. The receiver authenticates each datagram, unwraps the sequence number, rejects replays, stores media payloads and parity shards, extracts piggybacked replicas and updates the estimator. The playout thread fills to the target depth, then runs an absolute schedule anchored at an epoch — this is what allows the depth to be changed on a running call — and for each slot plays, repairs or conceals. The control thread, every 100 ms, snapshots the estimator, updates the capacity estimate from the delay trend, asks NAVIK for a decision, hands ρ and the burst estimate to the scheduler, optimises the buffer depth, floors it at the protection requirement and retargets the epoch.

Figure 9.  The four threads of a CallSession and the state they share.

Two design details were forced by measurement. First, the playout schedule is absolute rather than a free-running 20 ms tick: with a free-running tick the buffer depth is whatever happened to accumulate at start-up and can never change, so raising the depth later has no effect and parity that needs the extra slack arrives too late to be used. Second, replica displacement subtracts the 95th-percentile delay spread: ignoring it placed the copy about one frame too late and only 7 % of transmitted replicas were ever used.

## 6.5   The relay server

server.py binds one UDP socket and runs a single receive loop. The media relay is checked first because it has the highest volume: if the header's connection id names an active call, the datagram is forwarded verbatim to the other party and nothing else happens — no decryption, no parsing. Otherwise the datagram type selects a SPARK phase: HELLO is answered statelessly; AUTH_INIT is validated against the cookie and silently dropped if it fails (a spoofed source gets nothing); a retransmitted AUTH_INIT re-sends the first response so the transcript the client committed to is preserved; AUTH_CONFIRM completes the handshake and records the session by username and by CID. Encrypted CONTROL messages carry JSON signalling. Call placement mints a random 32-byte secret, derives the two directional keys and delivers them to each party under that party's own session key. Because UDP may lose either answer, the caller attaches a request id and retransmits; the server remembers the answer it already gave and repeats the same call_ready / incoming pair instead of minting a second call — the idempotent server half of SIP-style INVITE retransmission [35]. Keepalive ping messages refresh NAT bindings and let the server track the address at which a client is currently reachable.

## 6.6   The client

client.py is the user-facing application: python client.py --user alice --password secret1 --server host:9000 --call bob places a call and --wait answers one. Every handshake step is retried with a linearly growing timeout because UDP supplies no retransmission and SPARK's messages are all idempotent at the server. Audio comes from the microphone (--audio live, requiring sounddevice), a WAV file, or the synthetic generator; --protect swara|uniform|none is the A/B switch; --loss, --burst, --extra-delay and --jitter enable the local impairment injector for demonstrations; --narrowband selects the 33 kbit/s mode. A waiting client sends a keepalive every 15 s so that a NAT pinhole stays open while it is idle — the detail that makes a client that “works perfectly in testing” also work for real users.

## 6.7   Demonstration harness and experiments

demo.py starts a real server and two real clients in one process, each on its own UDP socket, runs an actual call through codec → PI → FEC → AEAD → UDP → relay → UDP → AEAD → repair → jitter buffer → codec, and reports frames sent, parity and replicas, frames rebuilt by FEC or saved by replicas, concealed frames, loss after repair, wire rate, waveform correlation and SNR (by exact FFT cross-correlation alignment of what was sent against what was heard), and authentication failures. With --compare the same call is performed three times — SWARA, uniform FEC, unprotected — on the identical channel realisation; --repeat averages runs. experiments/run_all.py runs the eight simulated experiments of Chapter 7 in about three minutes and regenerates results.json and all figures.

## 6.8   The web console and REST API

The web console (api_server.py, webapp/, static/) exposes the whole system through a browser. It is a FastAPI application served by uvicorn that also hosts the relay inside the same process, so command-line clients on this or another machine can log in and call each other while the console shows who is online and which calls are being relayed. Its thirteen pages are summarised in Table 8 and shown in Appendix B.

Table 8.  Web console pages.

| Page | What it does |
| --- | --- |
| Overview | Headline results from results.json, the twelve-stage pipeline (each stage links to its tool), an animated frame stream and the hosted-relay card with ready-to-paste client commands |
| Live Call Lab | Places a real instrumented call (relay + SPARK + AEAD over UDP) and streams it: per-frame fate map (delivered / rebuilt / replica / concealed, bar height = PI), sent vs received waveforms, live charts of network, adaptation and quality; “Listen live” plays the received audio in the browser; sliders change loss, burst, jitter and delay during the call; the caller's audio can be the user's own recording |
| A/B Benchmark | The same call under SWARA, uniform FEC and no protection over the identical channel, with charts, a like-for-like table and the audio of each arm |
| Network Simulator | The offline engine on demand with the named profiles, and a loss sweep |
| Research Results | Interactive versions of all eight experiments, the paper figures, and a button that regenerates them |
| Perceptual Analyzer | PI scores, class labels and the FEC plan for synthetic speech or the user's own recording, frame by frame |
| FEC Playground | Reed–Solomon encode / erase / decode by clicking packets; the PI-AFEC scheduler for one group |
| Quality & NAVIK | E-model calculator with the impairment breakdown; NAVIK decisions across loss rates |
| SPARK Handshake | Runs the real handshake in memory, message by message, including a wrong-password attempt and path migration |
| Packet Anatomy | Interactive 16-byte header packed by Header.pack() with the datagram composition |
| SETU Overlay | A random measured mesh, its converged routes and the pairs a relay improves |
| Relay & Users | Hosted relay status, signed-in users, active calls, log; register and delete users |
| System & API | Runtime, packages, module inventory and every endpoint; Swagger and ReDoc at /docs and /redoc |

The instrumented call is a TracedSession subclass of CallSession that observes the reassembly buffer immediately before and after the base-class logic runs and records how every received frame was obtained, without changing the protocol. Telemetry, frame fates and the received PCM are streamed to the browser with Server-Sent Events five times a second; the audio is played gaplessly through the Web Audio API. Only one call or benchmark runs at a time because the media threads are paced against a wall clock and two concurrent calls would measure each other rather than the network. The 37 REST endpoints are documented by the generated OpenAPI schema (Figure 57).

Figure 10.  The web console's Overview page: headline results, the twelve-stage pipeline, and the hosted relay card.

## 6.9   Server control launcher

launcher.py is a small Tkinter window that starts, stops and restarts uvicorn and the hosted relay, shows the console URL and the LAN address for other laptops, the relay's signed-in users and calls, and the live server log (with routine polling requests filtered out). It remembers the chosen ports, moves to the next free port when another program holds the default (which happened on the development machine, where port 8000 was occupied), and offers to stop the server when the window is closed so nothing is left running. SWARA Server.bat opens it with one double-click.

Figure 11.  The SWARA Server Control window with the server running.

## 6.10   Verification during development

Every endpoint was exercised over HTTP with a smoke-test script; every console page was loaded in headless Chrome with the console log captured, and no JavaScript errors remained. The Reed–Solomon implementation is checked for the MDS property at import. The fairness properties of the impairment injector (two random streams, per-arm reset, wall-clock stepping) exist because earlier versions of the harness produced measurable artefacts, each of which is documented in the source. The final verification pass for this report found one more such artefact — a busy-waiting scheduler thread in the injector that distorted every timing the live controller observed — and corrected it; Section 7.9 reports the diagnosis and the before/after measurements, because an evaluation is only as trustworthy as its instrument.

## 6.11   Running the system

pip install -r requirements.txt            # numpy, matplotlib, cryptography, fastapi, uvicorn (+ sounddevice)

# A real call between three terminals

python server.py --add-user alice --password secret1

python server.py --add-user bob   --password secret2

python server.py --port 9000                                     # terminal 1

python client.py --user bob   --password secret2 --wait          # terminal 2

python client.py --user alice --password secret1 --call bob      # terminal 3   (add --audio live for mic & speakers)

# One-command demonstration and A/B comparison

python demo.py --seconds 25 --loss 0.15 --burst 4 --jitter 12 --compare --repeat 2

# Web console (hosts the relay on udp:9000) and the desktop launcher

python -m uvicorn api_server:app --host 0.0.0.0 --port 8000      # then open http://localhost:8000

"SWARA Server.bat"                                               # Windows: one-click launcher window

# Regenerate all research figures (~3 minutes)

python experiments/run_all.py

# 7   Results and Discussion

Two kinds of evidence are reported. The simulated evaluation (Sections 7.2–7.9) runs identical synthetic speech through identical channel realisations for the three systems and scores each with the E-model; it is the stable, repeatable picture averaged over seeds, and every figure is regenerated by experiments/run_all.py. The live evaluation (Section 7.10) places real calls — real sockets, real cryptography, real time — through the relay over an impaired loopback path; it is noisier, but it is the system as a user would run it. Section 7.11 discusses what the two together show, and Section 7.12 states the limits of the evidence.

## 7.1   Experimental setup

Table 9.  Experimental configuration.

| Experiment | Configuration |
| --- | --- |
| Machine | Windows-11-10.0.26200-SP0, 20 logical CPUs, Python 3.14.3; all three systems run on the same machine so that CPU effects are common to every arm |
| Speech corpus | Source-filter synthesiser, 16 kHz, 20 ms frames; talkspurts 0.9–2.8 s; seeds 5, 17, 29, 41 (53) unless stated |
| Loss sweep | Loss 0–30 %, mean burst 3, delay 40 ± 14 ms, capacity 800 kbit/s, 22 s calls, 5 seeds; k = 8 |
| Burst sweep | Loss 12 %, mean burst 1–10, otherwise as above, 4 seeds |
| Overhead efficiency | Loss 12 %, burst 4; forced redundancy ρ ∈ {0 … 0.5} for SWARA and uniform FEC, 4 seeds |
| PI distribution | Six 14 s utterances (seeds 11, 23, 44, 57, 68, 79) with per-frame ground truth from the synthesiser |
| Jitter buffer | Loss 6 %, burst 3, delay 45 ± 22 ms with a heavier tail (σ = 0.7); forced depths 20–180 ms, 3 seeds |
| Handshake | In-process timing of the real SPARK implementation; 20,000 AEAD seals for the per-packet cost; DTLS-SRTP figures are published typical values |
| SETU overlay | 300 random 6-node overlays, 28 % of links through a congested exchange (70–190 ms, 20–60 ms jitter, 4–18 % loss), 4,500 pairs |
| Network profiles | Six named profiles (Section 5.10), 22 s calls, 4 seeds |
| Live calls | demo.py: 25 s calls, 2 repeats per arm, injector at both sockets; two conditions, 15 % / burst 4 / 12 ms jitter and 10 % / burst 3 / 10 ms jitter. Console benchmark: 20 s per arm at 15 % / 4 / 12. Three-terminal call: 20 s at 15 % / 4 / 12 on both clients |

## 7.2   Perceptual importance separates what matters

Table 10 and Figure 12 show the PI score by speech class on 4,419 labelled frames. The analyser places the four classes in the bands it was designed for, with silence at a mean of 3.4 and talkspurt onsets at 211.5. The decisive statistic for the whole project is the last row: silence is 29.0 % of all frames but carries only 0.86 % of the total importance mass. Those frames are what PI-AFEC declines to protect, and that budget is what it moves to the frames that carry meaning.

Table 10.  Perceptual importance score by speech class (0–255).

| Class | Frames | Mean PI | 10th percentile | 90th percentile |
| --- | --- | --- | --- | --- |
| Silence / noise | 1273 | 3.4 | 0.0 | 0.0 |
| Plosive | 171 | 70.7 | 0.0 | 164.0 |
| Unvoiced fricative | 694 | 111.1 | 91.0 | 130.0 |
| Voiced vowel | 2251 | 179.6 | 160.0 | 209.0 |
| Talkspurt onset | 30 | 211.5 | 135.9 | 255.0 |
| Silence share of frames / of importance |  | 29.0 % / 0.86 % |  |  |

Figure 12.  PI score distributions by speech class (violins show the full distribution, bars the mean) and the share of frames versus the share of importance held by silence.

## 7.3   Quality under packet loss

Table 11 and Figure 13 give the headline comparison: predicted MOS and perceptually weighted loss as channel loss rises from 0 to 30 % with a mean burst of 3, all three systems on the same speech, the same channel realisation and — for uniform FEC — the same redundancy SWARA spent. At 12 % loss SWARA scores 3.53 against 3.37 for uniform FEC and 3.38 unprotected, and its perceptual loss is 5.8 % against 7.3 % and 7.3 %. The advantage is largest in the 5–20 % band that corresponds to busy Wi-Fi and congested cellular links; at 30 % every scheme converges (2.20 / 2.15 / 2.21) because no amount of redundancy can replace a channel that drops almost a third of what is sent. Uniform FEC, note, is not uniformly better than no FEC: at 12–16 % loss its evenly spread parity buys nothing the baseline does not already get from concealment, while SWARA's placement still does.

Table 11.  Loss sweep: predicted MOS (G.107) and perceptually weighted loss, mean of 5 seeds.

| Channel loss | MOS baseline | MOS uniform FEC | MOS SWARA | Perc. loss baseline | Perc. loss uniform | Perc. loss SWARA |
| --- | --- | --- | --- | --- | --- | --- |
| 0 % | 4.22 | 4.22 | 4.24 | 0.5 % | 0.5 % | 0.0 % |
| 2 % | 4.09 | 4.16 | 4.14 | 1.7 % | 1.0 % | 1.0 % |
| 5 % | 3.85 | 3.98 | 4.05 | 3.7 % | 2.7 % | 1.8 % |
| 8 % | 3.62 | 3.79 | 3.79 | 5.5 % | 4.1 % | 3.8 % |
| 12 % | 3.38 | 3.37 | 3.53 | 7.3 % | 7.3 % | 5.8 % |
| 16 % | 3.17 | 3.13 | 3.27 | 8.8 % | 9.2 % | 7.8 % |
| 20 % | 2.67 | 2.79 | 2.96 | 12.7 % | 11.7 % | 10.1 % |
| 25 % | 2.37 | 2.48 | 2.53 | 15.3 % | 14.4 % | 13.6 % |
| 30 % | 2.21 | 2.15 | 2.20 | 16.9 % | 17.6 % | 16.6 % |

Figure 13.  Call quality under loss (left; shaded bands ±1 s.e.) and the reduction in perceptually weighted loss relative to the baseline (right).

## 7.4   Resilience to bursty loss

Figure 14 fixes the loss rate at 12 % and varies the mean burst length from 1 to 10 packets. Two things are visible. First, SWARA holds its lead over both comparators up to a burst of 8 and only converges with them at 10, where — as the project's own README states — every repair mechanism fails and redundancy becomes wasted bandwidth. Second, the right-hand panel shows which mechanism does the rescuing: with isolated losses Reed–Solomon parity recovers 41.3 frames per call and anchors 4.3; by a burst of 6 the parity recoveries have collapsed to 2.3 while anchor recoveries have risen to 6.3. This is the burst-mode design decision of Section 5.4 doing exactly what it was built to do: when a block code cannot survive the burst, the time-displaced replica still can.

Table 12.  Burst sweep at 12 % loss: MOS and SWARA's recoveries per call by mechanism, mean of 4 seeds.

| Mean burst | MOS baseline | MOS uniform FEC | MOS SWARA | FEC recoveries | Anchor recoveries |
| --- | --- | --- | --- | --- | --- |
| 1 | 3.79 | 3.89 | 4.02 | 41.3 | 4.3 |
| 2 | 3.58 | 3.64 | 3.80 | 21.3 | 4.3 |
| 3 | 3.37 | 3.37 | 3.53 | 15.5 | 3.5 |
| 4 | 3.28 | 3.49 | 3.55 | 14.0 | 1.8 |
| 6 | 2.96 | 3.13 | 3.41 | 2.3 | 6.3 |
| 8 | 3.03 | 3.17 | 3.28 | 3.0 | 6.8 |
| 10 | 2.89 | 2.84 | 2.80 | 1.8 | 5.3 |

Figure 14.  Resilience to bursty loss (left) and the mechanism responsible for each recovery (right).

## 7.5   Quality returned per byte of redundancy

Figure 15 plots MOS against the redundancy actually transmitted when both schemes are forced through the same budget ladder at 12 % loss and burst 4. Because PI-AFEC never spends on silent groups, it transmits less than the nominal budget (41.6 % at ρ = 0.5 against 50.0 % for uniform FEC) and yet reaches a higher MOS (3.44 versus 3.19). Reading the curves horizontally, SWARA reaches the quality uniform FEC attains at its maximum budget for 16.7 % less redundancy. This is the project's bandwidth claim: the same quality for fewer bytes, or more quality for the same bytes.

Table 13.  Overhead efficiency at 12 % loss, burst 4: MOS and redundancy actually spent for each nominal budget ρ.

| Nominal ρ | SWARA redundancy | SWARA MOS | Uniform redundancy | Uniform MOS |
| --- | --- | --- | --- | --- |
| 0.000 | 0.0 % | 3.18 | 0.0 % | 3.18 |
| 0.075 | 6.2 % | 3.07 | 7.6 % | 3.08 |
| 0.150 | 12.4 % | 3.16 | 15.3 % | 3.16 |
| 0.225 | 18.7 % | 3.17 | 22.4 % | 3.02 |
| 0.300 | 24.9 % | 3.04 | 30.4 % | 3.08 |
| 0.400 | 33.3 % | 3.30 | 40.3 % | 3.21 |
| 0.500 | 41.6 % | 3.44 | 50.0 % | 3.19 |

Equal-quality saving: 16.7 % of media bandwidth.

Figure 15.  Quality returned per byte of redundancy.

## 7.6   The jitter-buffer optimum

Section 5.7 argued that “minimise late loss” is the wrong objective for a playout buffer. Figure 16 tests the alternative. The left panel is the optimiser's own view: the predicted R-factor over depth, peaking at 100 ms, from which SWARA chose 99.9 ms. The right panel is the ground truth: the end-to-end simulation re-run with the depth forced to each value. Measured MOS peaks at 100 ms (3.81); a shallow 20 ms buffer collapses to 2.11 because almost everything arrives late, and a deep 180 ms buffer falls back to 3.52 because the delay impairment past the 177 ms knee outweighs the packets it rescues. The optimiser's choice and the measured optimum coincide to within 1 ms, which validates both the model and the decision to make it the controller's objective.

Table 14.  Jitter-buffer depth: predicted R-factor and measured end-to-end MOS.

| Depth (ms) | 20 | 40 | 60 | 80 | 100 | 120 | 150 | 180 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Predicted R | 23.6 | 60.8 | 77.0 | 82.4 | 84.2 | 82.6 | 79.1 | 75.3 |
| Measured MOS | 2.11 | 3.46 | 3.69 | 3.78 | 3.81 | 3.79 | 3.68 | 3.52 |

Optimiser's choice 99.9 ms; measured optimum 100 ms.

Figure 16.  Too shallow loses packets, too deep loses the conversation: the R-factor curve (left) and the end-to-end validation (right).

## 7.7   Handshake cost and security properties

Table 15 and Figure 17 measure the real SPARK implementation. Mutual password authentication with forward secrecy completes in 2 round trips and 397 bytes, against three round trips and roughly 1,900 bytes for DTLS-SRTP — about 4.8× less setup traffic. The 73 ms of CPU is dominated by the deliberately expensive PBKDF2 stretching on the client; the server's own work per handshake is three X25519 operations and a few hashes. A network handover costs zero round trips and 115 bytes, and the server spends 11 µs on it. The RETRY reply is 20 bytes to a 64-byte HELLO, so the amplification factor is 0.31: an attacker spoofing HELLOs at a victim makes the victim receive less than it sent. The per-user record is 128 bytes and contains no password-equivalent. Sealing a media packet costs 3.0 µs — 0.015 % of a 20 ms frame budget — so encryption is never on the critical path.

Table 15.  Measured handshake and media-protection cost.

| Quantity | SPARK | DTLS-SRTP (typical) | Plain UDP |
| --- | --- | --- | --- |
| Round trips before media flows | 2 | 3 | 0 |
| Handshake bytes on the wire | 397 | 1,900 | 0 |
| Round trips to migrate a live call | 0 | new handshake | n/a |
| Migration bytes | 115 | — | — |
| Amplification factor (reply ÷ first packet) | 0.313 | ≤ 3 (DTLS 1.2 cookie) | — |
| Server record per user | 128 B, no verifier | certificate / SRTP keys | none |
| Mutual password authentication | yes | no (certificate fingerprints) | no |
| AEAD seal per packet | 3.02 µs | similar | none |

Figure 17.  Call-setup cost in round trips and bytes.

Table 16.  Security properties of SPARK and how each is obtained.

| Property | Mechanism |
| --- | --- |
| No state before address validation | Stateless HMAC cookie bound to (address, 30 s epoch); AUTH_INIT without a valid cookie is silently dropped |
| No amplification | HELLO padded to 64 B; RETRY is 20 B; nothing larger is sent before the cookie returns |
| Password never on the wire | OPAQUE-style envelope opened locally with PBKDF2; a wrong password fails before anything is sent |
| No user enumeration | Unknown users receive a constant-shape random envelope |
| Mutual authentication | dh1 proves the server holds its static key; dh2 proves the client could open the envelope |
| Forward secrecy | dh3 between the two ephemeral keys; the transcript hash binds every byte of every message |
| Stolen database resistance | Server stores {salt, envelope, public key}; nothing is replayable at the server |
| Replay and forgery resistance on media | Deterministic nonces from the 64-bit sequence; 1,024-entry replay bitmap; header as AEAD associated data |
| Blind relay | Per-call directional keys delivered under each party's session key; the relay forwards ciphertext verbatim |
| Graceful handover | Encrypted resumption ticket + proof under k_resume; 0 RTT; replaying the ticket alone cannot hijack a call |

## 7.8   Relay-path selection

Across 4,500 peer pairs in 300 random overlays in which 28 % of links cross a congested exchange, SETU found a relayed path cheaper than the direct route for 29.3 % of pairs, with a mean cost reduction of 55.0 % (median 58.8 %) on the pairs it improved. 3,174 pairs stayed direct, 1,244 took one relay, 78 took two and 4 took three; the distance-vector computation converged in 6 rounds. The result reproduces the RON observation [33] in voice-cost terms: the triangle inequality fails often enough that a small measured overlay is worth having.

Figure 18.  SETU: distribution of the cost reduction on improved pairs (left) and the share of pairs by chosen route length (right).

## 7.9   Realistic network profiles

Table 17 and Figure 19 summarise the six named profiles. On good Wi-Fi all three systems are at the ceiling (4.23); SWARA spends nothing it does not need. On busy Wi-Fi and congested 4G it leads by 0.17 and 0.14 MOS over uniform FEC and by 0.26 and 0.33 over the baseline, halving the perceptual loss on busy Wi-Fi (1.8 % against 4.4 %). On the hostel profile — 15 % loss in bursts of 8, the condition that motivated the project — it is the only system above 3.0. Two profiles are honest exceptions. On poor 3G (20 % loss, 110 ms base delay) uniform FEC is marginally ahead (1.73 vs 1.72): at that loss rate the controller's safety rail allows little redundancy and the deeper buffer SWARA chooses costs more than its placement gains. On the satellite profile (280 ms base delay) the same delay arithmetic applies: every millisecond of extra buffer is charged at the steep post-knee rate, so uniform FEC with the conventional shallower buffer edges ahead (2.88 vs 2.82) even though SWARA's residual and perceptual loss are the lowest of the three. SWARA's gains, in other words, come where loss rather than propagation delay is the binding impairment — which is where most real conversations are.

Table 17.  Named profiles: predicted MOS, residual and perceptual loss, mouth-to-ear delay and redundancy, mean of 4 seeds.

| Profile | MOS base / uniform / SWARA | Residual loss base / uni / SWARA | Perceptual loss base / uni / SWARA | Delay (ms) base / SWARA | Redundancy uni / SWARA |
| --- | --- | --- | --- | --- | --- |
| good wifi | 4.23 / 4.21 / 4.23 | 1.9 / 1.7 / 1.0 % | 0.6 / 0.8 / 0.4 % | 68 / 102 | 46.5 / 46.3 % |
| busy wifi | 3.77 / 3.85 / 4.02 | 8.3 / 7.1 / 4.0 % | 4.4 / 3.6 / 1.8 % | 108 / 163 | 46.5 / 46.3 % |
| congested 4g | 3.24 / 3.43 / 3.57 | 13.4 / 11.5 / 9.5 % | 7.8 / 6.3 / 4.3 % | 168 / 194 | 37.7 / 37.8 % |
| hostel wifi | 2.80 / 2.79 / 3.05 | 15.8 / 14.2 / 13.1 % | 10.6 / 10.5 / 7.8 % | 177 / 195 | 25.2 / 24.9 % |
| poor 3g | 1.55 / 1.73 / 1.72 | 22.4 / 21.1 / 21.0 % | 15.9 / 13.7 / 12.9 % | 276 / 292 | 10.0 / 10.1 % |
| satellite | 2.75 / 2.88 / 2.82 | 5.8 / 4.2 / 3.6 % | 2.4 / 1.6 / 1.4 % | 361 / 375 | 46.2 / 46.3 % |

Figure 19.  Measured call quality across realistic network profiles (dashed line: the 3.6 “acceptable” threshold).

## 7.10   Live end-to-end calls

### 7.10.1   Validating the instrument first

Before the live benchmarks were accepted, the controller's behaviour during a real call was inspected by sampling NAVIK's state once per second (Table 18). The first run showed the capacity estimate collapsing from 400 kbit/s to its 24 kbit/s floor within eight seconds, after which the optimiser could fit no codec and emitted no redundancy at all for the rest of the call; the delay trend it was reacting to swung between −101 and +73 ms/s although the configured jitter was 12 ms, and the receiver-side one-way delay had a standard deviation of 35–120 ms with a 95th-percentile spread of up to 410 ms. On a clean link the same quantities were 1.7 ms and 5 ms. The cause was not the controller. The impairment injector's delayed-delivery scheduler consumed its wake-up event only when its queue ran empty — which, during a call, never happens — so its wait returned immediately and the thread busy-spun, starving the four media threads of the interpreter lock. The two-line correction (consume the flag on every iteration; swara/net.py, Listing 9) restored a delay spread consistent with the configured jitter, a trend within ±4 ms/s, a redundancy of ρ = 0.375 held for the whole call, and 169 parity plus 33 replica packets in 15 s where there had been 46 plus 2. Every live result below was measured after the correction. The episode is reported in full because it is the project's methodological thesis in miniature: a protocol can only be judged through an instrument that has itself been validated.

Table 18.  Diagnosis of the injector defect: one call at 15 % loss, burst 4, 12 ms jitter, sampled once per second.

| Quantity | Clean link (reference) | Impaired, before correction | Impaired, after correction |
| --- | --- | --- | --- |
| One-way delay std. dev., last 100 packets | 1.7 ms | 35 – 120 ms | 7.7 – 11 ms |
| One-way delay spread (p95 − min) | 5 ms | 120 – 410 ms | 29 – 34 ms |
| Delay-trend samples seen by NAVIK | ±0.8 ms/s | −101 … +73 ms/s | −4 … +1.5 ms/s |
| Capacity estimate after 8 s | — | 24 kbit/s (floor) | 283 kbit/s |
| Redundancy ρ after 8 s | 0 (no loss) | 0 (could not fit) | 0.375 |
| Parity + replica packets sent | 0 + 0 | 46 + 2 in 18 s | 169 + 33 in 15 s |
| Frames concealed at the receiver | 0.0 % | 20.0 % | 15.1 % |

### 7.10.2   demo.py: three arms on an identical channel

Table 19 reports the one-command A/B harness: 25-second calls, two repeats per arm, the injector at both sockets, every arm on the identical realisation of loss and jitter. At 15 % loss with bursts of four, SWARA matches uniform FEC's perceptually weighted damage (7.3 % each) while transmitting 112 kbit/s against 126 — 11 % less bandwidth — and both cut the damage of unprotected UDP (8.7 %) by 16 %. The raw frame count tells the expected other half of the story: uniform FEC leaves fewer frames unrecoverable (11.1 % against 12.3 %) because it protects everything, silence included, and does so with 600 parity packets to SWARA's 328 plus 56 replicas. At 10 % loss with bursts of three, SWARA achieves the lowest damage of the three — 4.2 % against 4.3 % and 5.3 % — on 12 % less bandwidth than uniform FEC. Both runs are captured as real terminal output in Figures 20 and 21.

Table 19.  Live A/B comparison (demo.py, 25 s × 2 repeats, injector at both sockets).

| Condition | Arm | Parity / replicas | Rebuilt (FEC / replica) | Concealed | Unrecoverable | Wire kbit/s | Corr. | SNR dB | Perceptual damage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 % / burst 4 / 12 ms | SWARA | 328 / 56 | 8.0 / 5.5 | 156.5 | 12.3 % | 111.8 | 0.924 | 8.5 | 7.3 % |
|  | Uniform FEC | 600 / 0 | 31.0 / 0 | 141.5 | 11.1 % | 126.3 | 0.924 | 8.5 | 7.3 % |
|  | Unprotected | 0 / 0 | 0 / 0 | 169.0 | 13.3 % | 87.7 | 0.914 | 8.0 | 8.7 % |
| 10 % / burst 3 / 10 ms | SWARA | 356 / 48 | 11.0 / 5.0 | 105.5 | 8.3 % | 113.2 | 0.942 | 9.5 | 4.2 % |
|  | Uniform FEC | 628 / 0 | 25.5 / 0 | 96.0 | 7.5 % | 128.1 | 0.942 | 9.5 | 4.3 % |
|  | Unprotected | 0 / 0 | 0 / 0 | 119.0 | 9.3 % | 87.7 | 0.934 | 9.0 | 5.3 % |

1,293 frames sent per call; authentication failures 0 in every run. Correlation and SNR compare the waveform heard by bob with the one alice sent, after FFT alignment.

Figure 20.  Terminal output of demo.py --compare at 15 % loss, mean burst 4, 12 ms jitter (25 s calls, 2 repeats per arm).

Figure 21.  Terminal output of demo.py --compare at 10 % loss, mean burst 3, 10 ms jitter.

### 7.10.3   The web console benchmark and an instrumented call

The console's A/B Benchmark page (Figure 37) runs the same three arms as full instrumented calls — 20 s per arm at 15 % / burst 4 / 12 ms, one run each — and reports a like-for-like table with the audio of each arm. SWARA cut perceptual damage by 8 % versus unprotected UDP (10.9 % against 11.8 %) and by 3 % versus uniform FEC (11.2 %) while using 12 % less bandwidth (107.8 against 122.6 kbit/s); the estimated MOS ordering was 2.64 / 2.59 / 2.55. The single-call Live Call Lab (Figure 35) shows the mechanism frame by frame: in a 15 s call at 10 % loss with bursts of three, 750 frames were sent with 197 parity and 12 replica packets (26 % overhead, 107 kbit/s), 674 arrived, 6 were rebuilt from parity and 1 from a replica, 69 were concealed, SPARK authenticated the two parties in 190 and 858 ms, call setup took 826 ms, the jitter buffer settled at 118 ms, and the received waveform correlated with the sent one at 0.941 (SNR 9.4 dB) for a predicted MOS of 3.59. The frame-fate map in the same figure makes the design visible: the tallest bars — the onsets and vowels — are almost all teal, while the red concealments cluster in the low, silent valleys.

### 7.10.4   A call between separate programs

Finally, the system was run as a user would run it: server.py in one terminal, client.py --wait as bob in a second and client.py --call bob as alice in a third, with the injector enabled on both clients at 15 % / burst 4 / 12 ms (Figures 26–28). Both clients authenticated in two round trips (776 ms and 1,681 ms including PBKDF2), the server logged the call with the note that it holds no keys, and after 20 s each side printed its own report: bob sent 1,021 frames with 287 parity and 17 replica packets, rebuilt 14 frames from parity and concealed 102 (10.0 %); alice sent 1,021 frames with 273 parity and 32 replicas, rebuilt 7 and concealed 139 (13.7 %). Neither side saw a single authentication failure. Section B.2 shows the same three windows during the call.

## 7.11   Discussion

- The gain is where the theory says it should be. Across the loss sweep, burst sweep and profiles, SWARA's advantage over uniform FEC at an identical budget is concentrated at 5–20 % loss with bursts of 2–8 — the regime of busy Wi-Fi, hostel networks and congested cellular links — and it vanishes at 30 % loss and bursts of 10, exactly where no repair mechanism can work. Placement, not quantity, is the source of the gain: Table 13 shows SWARA transmitting less redundancy than uniform FEC at every budget and scoring higher.

- Raw and perceptual loss tell different stories, and both must be reported. In every live run uniform FEC leaves fewer frames unrecoverable while SWARA leaves fewer audible frames unrecoverable. A reader who looks only at the raw count will conclude SWARA is worse; a reader who looks only at the perceptual figure will not understand why. The project reports both and argues that the second is the one a listener experiences.

- Anchors and parity are complementary, not redundant. The burst sweep (Figure 14) shows parity doing the work at short bursts and replicas at long ones; a scheme with only one of them is good at only one end of the range.

- The buffer is part of the protection scheme. The jitter-buffer experiment validated the R-factor objective to within 1 ms, and the profiles show the price of it: SWARA's buffer is 20–55 ms deeper than the conventional one because it refuses to pay for parity it cannot use. On links where propagation delay is already past the knee (satellite, poor 3G) that price exceeds the benefit, and uniform FEC with a shallow buffer is marginally better.

- Live gains are smaller than simulated gains, for identifiable reasons. Live calls are short (the controller needs about half a second of arrivals before it acts), the capacity estimate decays slowly under sustained loss, the simulator's k = 8 groups are replaced by k = 4 to keep the playout depth conversational, and each sender sizes its redundancy from the loss on its receiving path — the telemetry report from the far end is carried and parsed but not yet used, so an asymmetric path is under-protected. None of these is intrinsic to the idea; all are listed as future work.

- Single runs are noisy. With a mean burst of four a 25-second call contains only about 47 loss bursts, few enough that which bursts land where matters as much as the scheme under test. The identical-realisation design removes the between-arm component of that noise, but not the between-run component; the two-repeat tables above are stable to roughly ±1 percentage point of perceptual damage, which is why the simulated, multi-seed evaluation remains the primary evidence.

- Security came at negligible cost. Two round trips, 397 bytes, 3 µs per packet and a relay that cannot listen — against a naive UDP service that is both a reflection amplifier and an open microphone.

## 7.12   Limitations

- Quality figures come from the ITU-T G.107 model fed with a perceptually weighted loss; no human listening tests were conducted.

- The research corpus is synthesised speech with the right structure (talkspurts, voiced and unvoiced phones, onsets) but not recorded human speech; the PI analyser's bands were tuned on it.

- Live calls use real UDP, but impairment is injected at the sending socket rather than encountered in the wild, and both ends run on one machine, so the delay statistics are those of the loopback path plus the injector.

- The control loop drives redundancy from the local receive-side estimator; the piggybacked telemetry is not yet consumed by the far end's decision, so paths that are lossy in only one direction are under-protected.

- At mean bursts of ten packets or more every repair mechanism fails and redundancy is wasted; the controller's safety rail limits but does not eliminate that waste.

- CPython's interpreter lock bounds the timing precision of a pure-Python media pipeline; the injector defect of Section 7.10.1 is an example of how sensitive the measurements are to it.

- SPARK composes well-analysed primitives but has no formal security proof; the cookie epoch (30 s) and ticket lifetime (1 h) are reasonable defaults, not tuned values.

- The web console runs one instrumented call or benchmark at a time by design.

# 8   Conclusion

This project set out to show that packet loss in a voice call should be judged by what the listener loses, and to build a complete system around that principle. SWARA scores every 20 ms frame by its perceptual importance, carries the score where the network can read but not forge it, spends a fixed redundancy budget on the frames that carry meaning, protects the most important of them with replicas that survive the bursts a block code cannot, chooses codec, redundancy, marking and playout depth jointly against a single standard quality model, and secures the whole conversation with a two-round-trip password-authenticated handshake behind a relay that cannot hear the call.

The evaluation supports the principle. At an identical redundancy budget SWARA raises predicted MOS at 12 % loss from 3.37 to 3.53, keeps its lead up to bursts of eight packets, reaches the same quality as uniform FEC for 16.7 % less redundancy, and leads on five of six realistic profiles — the exceptions being links where propagation delay, not loss, is the binding impairment. The R-factor-optimal playout buffer chose the depth that an exhaustive sweep confirmed as optimal. SPARK authenticates in 397 bytes with an amplification factor of 0.31 and migrates in zero round trips. In live calls the system matched uniform FEC's perceptual quality on 11 % less bandwidth at 15 % loss and bettered it at 10 %, and a three-terminal call between separate programs ran exactly as the design says it should.

Two lessons go beyond the numbers. The first is methodological: three of the artefacts the team had to remove — per-packet channel stepping, a shared random stream, and a busy-waiting scheduler — were defects of the measuring instrument that looked like properties of the protocol, and the final one was found only by instrumenting the controller during a real call. The second is architectural: the components that make SWARA work are not independent knobs. The buffer depth bounds the replica displacement; the replica displacement bounds what burst mode can rescue; the controller's objective is the same function the evaluator uses. Designing them together is what made the gains measurable.

## 8.1   Future work

- Use the far end's piggybacked telemetry, not only local receive-side statistics, to size redundancy on asymmetric paths.

- Replace IMA ADPCM with a modern codec such as Opus so that the controller's bitrate ladder changes real coding quality and not only the nominal E-model parameters.

- Validate the perceptual weighting with listening tests on recorded speech, and tune the PI bands on a human corpus.

- Complete SETU's role as a NAT-traversal fallback and extend the relay to multi-party calls with per-participant importance-aware forwarding.

- Study SPARK formally — in particular the composition of the cookie, the OPAQUE-style envelope and the resumption ticket — and add rate limiting on the server's PBKDF2 path.

- Port the media pipeline to a language without an interpreter lock, or to an asynchronous design, to tighten timing and allow several instrumented calls at once.

# References

[1]	H. Schulzrinne, S. Casner, R. Frederick and V. Jacobson, “RTP: A Transport Protocol for Real-Time Applications,” IETF RFC 3550, Jul. 2003.

[2]	ITU-T Recommendation G.107, “The E-model: a computational model for use in transmission planning,” International Telecommunication Union, Geneva, Jun. 2015.

[3]	ITU-T Recommendation G.114, “One-way transmission time,” International Telecommunication Union, Geneva, May 2003.

[4]	ITU-T Recommendation P.800, “Methods for subjective determination of transmission quality,” International Telecommunication Union, Geneva, Aug. 1996.

[5]	C. Perkins, O. Hodson and V. Hardman, “A survey of packet loss recovery techniques for streaming audio,” IEEE Network, vol. 12, no. 5, pp. 40–48, Sep.–Oct. 1998.

[6]	J.-C. Bolot, S. Fosse-Parisis and D. Towsley, “Adaptive FEC-based error control for Internet telephony,” in Proc. IEEE INFOCOM, New York, NY, USA, 1999, pp. 1453–1460.

[7]	I. S. Reed and G. Solomon, “Polynomial codes over certain finite fields,” Journal of the Society for Industrial and Applied Mathematics, vol. 8, no. 2, pp. 300–304, 1960.

[8]	L. Rizzo, “Effective erasure codes for reliable computer communication protocols,” ACM SIGCOMM Computer Communication Review, vol. 27, no. 2, pp. 24–36, Apr. 1997.

[9]	A. E. Mohr, E. A. Riskin and R. E. Ladner, “Unequal loss protection: Graceful degradation of image quality over packet erasure channels through forward error correction,” IEEE Journal on Selected Areas in Communications, vol. 18, no. 6, pp. 819–828, Jun. 2000.

[10]	W. Jiang and H. Schulzrinne, “Comparison and optimization of packet loss repair methods on VoIP perceived quality under bursty loss,” in Proc. 12th Int. Workshop on Network and Operating Systems Support for Digital Audio and Video (NOSSDAV), Miami Beach, FL, USA, 2002, pp. 73–81.

[11]	J.-M. Valin, K. Vos and T. Terriberry, “Definition of the Opus Audio Codec,” IETF RFC 6716, Sep. 2012.

[12]	M. Zanaty, V. Singh, A. Begen and G. Mandyam, “RTP Payload Format for Flexible Forward Error Correction (FEC),” IETF RFC 8627, Jul. 2019.

[13]	R. Ramjee, J. Kurose, D. Towsley and H. Schulzrinne, “Adaptive playout mechanisms for packetized audio applications in wide-area networks,” in Proc. IEEE INFOCOM, Toronto, Canada, 1994, pp. 680–688.

[14]	Y. J. Liang, N. Färber and B. Girod, “Adaptive playout scheduling and loss concealment for voice communication over IP networks,” IEEE Transactions on Multimedia, vol. 5, no. 4, pp. 532–543, Dec. 2003.

[15]	ITU-T Recommendation G.711 Appendix I, “A high quality low-complexity algorithm for packet loss concealment with G.711,” International Telecommunication Union, Geneva, Sep. 1999.

[16]	E. N. Gilbert, “Capacity of a burst-noise channel,” Bell System Technical Journal, vol. 39, no. 5, pp. 1253–1265, Sep. 1960.

[17]	E. O. Elliott, “Estimates of error rates for codes on burst-noise channels,” Bell System Technical Journal, vol. 42, no. 5, pp. 1977–1997, Sep. 1963.

[18]	A. D. Clark, “Modeling the effects of burst packet loss and recency on subjective voice quality,” in Proc. IP Telephony Workshop, Columbia University, New York, NY, USA, Apr. 2001.

[19]	L. Sun and E. C. Ifeachor, “Voice quality prediction models and their application in VoIP networks,” IEEE Transactions on Multimedia, vol. 8, no. 4, pp. 809–820, Aug. 2006.

[20]	M. Baugher, D. McGrew, M. Naslund, E. Carrara and K. Norrman, “The Secure Real-time Transport Protocol (SRTP),” IETF RFC 3711, Mar. 2004.

[21]	D. McGrew and E. Rescorla, “Datagram Transport Layer Security (DTLS) Extension to Establish Keys for the Secure Real-time Transport Protocol (SRTP),” IETF RFC 5764, May 2010.

[22]	E. Rescorla and N. Modadugu, “Datagram Transport Layer Security Version 1.2,” IETF RFC 6347, Jan. 2012.

[23]	J. Iyengar and M. Thomson (Eds.), “QUIC: A UDP-Based Multiplexed and Secure Transport,” IETF RFC 9000, May 2021.

[24]	S. Jarecki, H. Krawczyk and J. Xu, “OPAQUE: An asymmetric PAKE protocol secure against pre-computation attacks,” in Advances in Cryptology – EUROCRYPT 2018, Lecture Notes in Computer Science, vol. 10822, Springer, 2018, pp. 456–486.

[25]	A. Langley, M. Hamburg and S. Turner, “Elliptic Curves for Security,” IETF RFC 7748, Jan. 2016.

[26]	Y. Nir and A. Langley, “ChaCha20 and Poly1305 for IETF Protocols,” IETF RFC 8439, Jun. 2018.

[27]	H. Krawczyk and P. Eronen, “HMAC-based Extract-and-Expand Key Derivation Function (HKDF),” IETF RFC 5869, May 2010.

[28]	K. Moriarty, B. Kaliski and A. Rusch, “PKCS #5: Password-Based Cryptography Specification Version 2.1,” IETF RFC 8018, Jan. 2017.

[29]	K. Nichols, S. Blake, F. Baker and D. Black, “Definition of the Differentiated Services Field (DS Field) in the IPv4 and IPv6 Headers,” IETF RFC 2474, Dec. 1998.

[30]	B. Davie et al., “An Expedited Forwarding PHB (Per-Hop Behavior),” IETF RFC 3246, Mar. 2002.

[31]	J. Heinanen, F. Baker, W. Weiss and J. Wroclawski, “Assured Forwarding PHB Group,” IETF RFC 2597, Jun. 1999.

[32]	S. Floyd and V. Jacobson, “Random early detection gateways for congestion avoidance,” IEEE/ACM Transactions on Networking, vol. 1, no. 4, pp. 397–413, Aug. 1993.

[33]	D. Andersen, H. Balakrishnan, F. Kaashoek and R. Morris, “Resilient overlay networks,” in Proc. 18th ACM Symposium on Operating Systems Principles (SOSP), Banff, Canada, 2001, pp. 131–145.

[34]	G. Malkin, “RIP Version 2,” IETF RFC 2453, Nov. 1998.

[35]	J. Rosenberg et al., “SIP: Session Initiation Protocol,” IETF RFC 3261, Jun. 2002.

[36]	Interactive Multimedia Association, “Recommended Practices for Enhancing Digital Audio Compatibility in Multimedia Systems,” Revision 3.00, Oct. 1992 (IMA ADPCM).

[37]	H. Schulzrinne and S. Casner, “RTP Profile for Audio and Video Conferences with Minimal Control,” IETF RFC 3551, Jul. 2003.

[38]	R. Braden, D. Borman and C. Partridge, “Computing the Internet Checksum,” IETF RFC 1071, Sep. 1988.

[39]	R. W. Hamming, “Error detecting and error correcting codes,” Bell System Technical Journal, vol. 29, no. 2, pp. 147–160, Apr. 1950.

[40]	S. Ramírez, “FastAPI,” https://fastapi.tiangolo.com, and Encode, “Uvicorn,” https://www.uvicorn.org (accessed Oct. 2026).

[41]	Python Cryptographic Authority, “pyca/cryptography,” https://cryptography.io (accessed Oct. 2026).

[42]	Chart.js contributors, “Chart.js,” version 4.4.1, https://www.chartjs.org (accessed Oct. 2026).

[43]	V. Jacobson, “Congestion avoidance and control,” in Proc. ACM SIGCOMM, Stanford, CA, USA, 1988, pp. 314–329.

[44]	ITU-T Recommendation G.113 Appendix I, “Provisional planning values for the equipment impairment factor Ie and packet-loss robustness factor Bpl,” International Telecommunication Union, Geneva, 2009.

[45]	Python Software Foundation, “The Python Language Reference, version 3.14,” https://docs.python.org/3.14/ (accessed Oct. 2026).

[46]	J. Postel, “User Datagram Protocol,” IETF RFC 768, Aug. 1980.

[47]	A. Li (Ed.), “RTP Payload Format for Generic Forward Error Correction,” IETF RFC 5109, Dec. 2007.

# Appendix A   Source Code Repository

The complete project — protocol stack, relay and clients, demonstration harness, experiments, web console, launcher, figures and this report — is published on GitHub:

https://github.com/rohitarora80905-glitch/swara-v1.0

NOTE The local working copy that produced every figure and screenshot in this report is SWARA_Complete_Project/SWARA.

## A.1   Obtaining and running the code

git clone https://github.com/rohitarora80905-glitch/swara-v1.0

cd swara-v1.0

python -m venv .venv && .venv\Scripts\activate          # Windows (use source .venv/bin/activate on Linux/macOS)

pip install -r requirements.txt

python demo.py --seconds 20                                 # the most reliable demonstration: 0 % loss, correlation ≈ 0.99

python demo.py --seconds 25 --loss 0.15 --burst 4 --jitter 12 --compare --repeat 2

python experiments/run_all.py                               # regenerates results.json and figures/ in ~3 minutes

python -m uvicorn api_server:app --host 0.0.0.0 --port 8000 # web console at http://localhost:8000, API docs at /docs

## A.2   Repository contents

Table 20.  Repository contents.

| Path | Contents |
| --- | --- |
| swara/ | The protocol stack: 15 modules, listed in Appendix C |
| server.py, client.py, demo.py | Relay server, voice-chat client, one-process demonstration and A/B harness |
| api_server.py, webapp/, static/ | Web console back end (FastAPI), instrumented calls and tools, single-page front end |
| launcher.py, SWARA Server.bat, start_console.bat | Desktop server-control window and one-click launchers |
| experiments/run_all.py, experiments/results.json | The eight evaluation experiments and their machine-readable results |
| figures/fig1 … fig8 | Research figures reproduced in Chapter 7 |
| users.json | Credential records (salt, envelope, public key) for the registered users |
| README.md, RUNME.txt, requirements.txt | Documentation, step-by-step run instructions, dependencies |

## A.3   Reproducibility

- Every simulated figure is regenerated by one command from fixed seeds; results.json is the single source of the numbers in Chapter 7 and the console's Research Results page.

- Every live comparison uses the identical-realisation injector (two random streams, per-arm reset, wall-clock stepping) so the three arms face the same sequence of good and bad periods.

- Every screenshot in Appendix B was captured from the running system on the date of this report; terminal outputs were additionally saved as text and are reproduced verbatim in the tables of Section 7.10.

# Appendix B   Screenshots

All screenshots are actual captures of the system running on the development machine (Windows 11) on the date of this report. Browser pages are full-length captures of the web console at 1600 px width; terminal windows are captured as displayed. Captions give the exact command or action that produced each view.

## B.1   Starting the server

Figure 22.  SWARA Server Control, opened by double-clicking “SWARA Server.bat”. The server was started with one click; the window shows the console address, the LAN address for other laptops, the hosted relay on udp:9000 and the live server log.

## B.2   A real call between three separate programs — both sides

The classic three-terminal demonstration from the README: the relay in terminal 1, the callee (bob) waiting in terminal 2 and the caller (alice) dialling in terminal 3. The impairment injector was enabled on both clients (15 % loss, mean burst 4, 12 ms jitter) so that each side both suffers and repairs loss. The first three figures were captured during the call; the next three after it ended, when each client printed its own report.

Figure 23.  Terminal 1 — the relay (`python server.py --port 9000`) during the call: both users authenticated by SPARK and the call being relayed “keys not held”.

Figure 24.  Terminal 2 — the callee (`python client.py --user bob … --wait`) during the call: authenticated in two round trips, incoming call accepted, media flowing.

Figure 25.  Terminal 3 — the caller (`python client.py --user alice … --call bob`) during the call.

Figure 26.  Terminal 1 after the call: the relay's complete log for the session.

Figure 27.  Terminal 2 after the call — bob's own report: 1,021 frames sent with 287 parity and 17 replica packets; 1,015 played; 14 rebuilt by FEC; 102 concealed (10.0 %); no authentication failures.

Figure 28.  Terminal 3 after the call — alice's own report: 1,021 frames sent with 273 parity and 32 replicas; 1,011 played; 7 rebuilt by FEC; 139 concealed (13.7 %); no authentication failures.

## B.3   Command-line clients on the relay hosted by the web console

The same two clients connected to the relay running inside the uvicorn process while the console watched. The Relay & Users page lists both users with their addresses and connection ids, the active call and its two endpoints, and the relay log; the Overview page's relay card shows “2 online · 1 call”.

Figure 29.  Relay & Users page while alice and bob (command-line clients) are in a call through the hosted relay.

Figure 30.  Overview page during the same call: the hosted-relay card reports two users online and one call relayed.

Figure 31.  The caller's terminal for the hosted-relay call (30 s, 10 % loss, burst 3, 10 ms jitter on alice's socket).

Figure 32.  The callee's terminal for the hosted-relay call.

## B.4   Live Call Lab

Figure 33.  Live Call Lab before a call: audio source, call length, protection scheme, codec mode and the network sliders (presets from Clean to Poor 3G).

Figure 34.  Live Call Lab during a call: the frame-fate map fills from left to right (bar height = PI; teal delivered, blue rebuilt from parity, violet anchor replica, red concealed), the sent and received waveforms, and the live gauges.

Figure 35.  Live Call Lab after the call: the complete frame-fate map, the three time-series charts (network as measured, adaptation, quality), the call report with playable sent and received audio, the call log and the session table.

## B.5   A/B Benchmark

Figure 36.  A/B Benchmark page before a run: audio source, seconds per call, repeats, and the network conditions shared by all three arms.

Figure 37.  A/B Benchmark after a run at 15 % loss, burst 4, 12 ms jitter (20 s per arm): per-arm cards, the verdict, four comparison charts, the like-for-like table and the audio of each arm.

## B.6   Network Simulator

Figure 38.  Network Simulator: the Congested 4G profile simulated for SWARA, uniform FEC and the baseline with MOS gauges, detailed per-system figures and the loss-accounting chart.

## B.7   Research Results

Figure 39.  Research Results — loss sweep tab, with the interactive chart, the finding, the paper figure, the figure gallery and the regeneration control.

Figure 40.  Research Results — burst sweep tab.

Figure 41.  Research Results — overhead efficiency tab.

Figure 42.  Research Results — PI distribution tab.

Figure 43.  Research Results — jitter buffer tab.

Figure 44.  Research Results — handshake tab.

Figure 45.  Research Results — SETU overlay tab.

Figure 46.  Research Results — network profiles tab.

## B.8   Protocol toolkit

Figure 47.  Perceptual Analyzer: synthetic speech coloured by class with the PI trace, the ground-truth / label / FEC-plan strips, the frame inspector, the PI histogram and per-class statistics.

Figure 48.  FEC Playground: an RS(6, 4) group with two data shards dropped and recovered; the PI-AFEC scheduler for a vowel-onset group.

Figure 49.  FEC Playground after the “Burst” button: more shards lost than the code can repair — the frame must be concealed.

Figure 50.  Quality & NAVIK: the E-model calculator with its impairment breakdown and the controller's decisions across loss rates.

Figure 51.  SPARK Handshake: the six messages with sizes and timings, the derived keys matching on both sides, and the 0-RTT path migration.

Figure 52.  SPARK Handshake with a wrong password: the envelope fails to open locally and AUTH_CONFIRM is never sent.

Figure 53.  Packet Anatomy: the 16-byte header packed by Header.pack(), the hex dump, and what the AEAD protects.

Figure 54.  SETU Overlay: a seven-node measured mesh, the converged route for the selected pair, and all pairs ranked by gain.

## B.9   Operations and API

Figure 55.  Relay & Users (idle): hosted relay status, the ready-to-paste client commands, the relay log and the user registry.

Figure 56.  System & API: runtime, packages, the source-module inventory and every endpoint.

Figure 57.  The generated OpenAPI documentation (Swagger UI) at /docs — the first five endpoint groups of thirty-seven documented routes.

# Appendix C   Source Code

The listings below reproduce the protocol stack (swara/), the relay, the client, the demonstration harness and the web API verbatim from the repository. The web console's back-end helpers (webapp/, 1,170 lines), its front end (static/, about 3,000 lines of JavaScript, CSS and HTML), the experiment script (experiments/run_all.py, 511 lines) and the launcher (launcher.py, 482 lines) are omitted for length and are available in the repository (Appendix A).

Listing 1   swara/packet.py	207 lines

Wire format: header, datagram types, flags, media/parity layouts; CRC-32, Internet checksum, 2-D parity and Hamming(7,4) for the error-control comparison.

1  """

2  SWARA wire protocol -- datagram formats.

3

4  Every SWARA datagram is a single UDP payload. There is no stream, no

5  retransmission-by-default, and no head-of-line blocking: a lost packet is a lost

6  20 ms of audio and nothing more.

7

8  Fixed header (16 bytes, network byte order):

9

10     0                   1                   2                   3

11     0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

12    +-------+-------+---------------+-------------------------------+

13    | Ver   | Type  |     Flags     |        Sequence Number        |

14    +-------+-------+---------------+-------------------------------+

15    |                       Connection ID (CID)                     |

16    +---------------------------------------------------------------+

17    |                    Timestamp (milliseconds)                   |

18    +---------------+---------------+---------------+---------------+

19    |   PI Score    |   FEC Group   |   FEC Index   |    Reserved   |

20    +---------------+---------------+---------------+---------------+

21

22  The PI Score octet is the heart of the design.  It is *authenticated but not

23  encrypted* (it travels as AEAD associated data), so any element on the path --

24  the receiver, a relay, or a DiffServ-aware router -- can read how perceptually

25  important the enclosed 20 ms of speech is WITHOUT being able to read or forge

26  the speech itself.

27  """

28

29  from __future__ import annotations

30

31  import struct

32  import zlib

33  from dataclasses import dataclass, field

34

35  VERSION = 1

36  HEADER_LEN = 16

37  AEAD_TAG_LEN = 16

38  MAX_DATAGRAM = 1200  # stay under the smallest plausible path MTU, no fragmentation

39

40

41  class PType:

42      """Datagram type codes (4 bits)."""

43      HELLO = 0          # client -> server, stateless probe

44      RETRY = 1          # server -> client, anti-amplification cookie

45      AUTH_INIT = 2      # client -> server, cookie + ephemeral public key

46      AUTH_RESP = 3      # server -> client, salt + envelope + ephemeral public key

47      AUTH_CONFIRM = 4   # client -> server, key-confirmation tag

48      AUTH_TICKET = 5    # server -> client, encrypted resumption ticket

49      MEDIA = 6          # a coded speech frame

50      FEC = 7            # a Reed-Solomon parity packet

51      ANCHOR = 8         # perceptual anchor replica (duplicate of a high-PI frame)

52      TELEMETRY = 9      # standalone telemetry (normally piggybacked instead)

53      NACK = 10          # selective negative acknowledgement

54      PATH_MIGRATE = 11  # 0-RTT authenticated path migration

55      PATH_ACK = 12      # path validation response

56      OVERLAY_LSA = 13   # SETU overlay link-state / distance-vector advertisement

57      BYE = 14

58      CONTROL = 15       # encrypted signalling: call setup, presence, hangup

59

60      NAMES = {

61          0: "HELLO", 1: "RETRY", 2: "AUTH_INIT", 3: "AUTH_RESP",

62          4: "AUTH_CONFIRM", 5: "AUTH_TICKET", 6: "MEDIA", 7: "FEC",

63          8: "ANCHOR", 9: "TELEMETRY", 10: "NACK", 11: "PATH_MIGRATE",

64          12: "PATH_ACK", 13: "OVERLAY_LSA", 14: "BYE", 15: "CONTROL",

65      }

66

67

68  class Flag:

69      """Header flag bits."""

70      TELEMETRY_PIGGYBACK = 0x01  # a 16-byte telemetry report follows the header

71      MARKER = 0x02               # first frame of a talkspurt

72      DSCP_EF = 0x04              # sender requested Expedited Forwarding

73      ENCRYPTED = 0x08            # payload is AEAD-protected

74      LAST_IN_GROUP = 0x10        # final data frame of an FEC group

75      RESUMED = 0x20              # session was resumed 0-RTT

76

77

78  _HDR = struct.Struct("!BBHIIBBBB")

79

80

81  @dataclass

82  class Header:

83      ptype: int

84      flags: int = 0

85      seq: int = 0

86      cid: int = 0

87      ts_ms: int = 0

88      pi: int = 0            # perceptual importance, 0..255

89      fec_group: int = 0

90      fec_index: int = 0

91      reserved: int = 0

92      version: int = VERSION

93

94      def pack(self) -> bytes:

95          ver_type = ((self.version & 0x0F) << 4) | (self.ptype & 0x0F)

96          return _HDR.pack(

97              ver_type, self.flags & 0xFF, self.seq & 0xFFFF, self.cid & 0xFFFFFFFF,

98              self.ts_ms & 0xFFFFFFFF, self.pi & 0xFF, self.fec_group & 0xFF,

99              self.fec_index & 0xFF, self.reserved & 0xFF,

100          )

101

102      @classmethod

103      def unpack(cls, buf: bytes) -> "Header":

104          if len(buf) < HEADER_LEN:

105              raise ValueError("short datagram")

106          ver_type, flags, seq, cid, ts, pi, grp, idx, rsv = _HDR.unpack(buf[:HEADER_LEN])

107          return cls(

108              ptype=ver_type & 0x0F, version=(ver_type >> 4) & 0x0F, flags=flags,

109              seq=seq, cid=cid, ts_ms=ts, pi=pi, fec_group=grp, fec_index=idx,

110              reserved=rsv,

111          )

112

113      def __str__(self) -> str:

114          return (f"<{PType.NAMES.get(self.ptype, '?')} seq={self.seq} cid={self.cid:08x} "

115                  f"pi={self.pi} grp={self.fec_group} idx={self.fec_index}>")

116

117

118  @dataclass

119  class Datagram:

120      header: Header

121      payload: bytes = b""

122      telemetry: bytes = b""   # optional 16-byte piggybacked report

123

124      def to_bytes(self) -> bytes:

125          if self.telemetry:

126              self.header.flags |= Flag.TELEMETRY_PIGGYBACK

127          return self.header.pack() + self.telemetry + self.payload

128

129      @classmethod

130      def from_bytes(cls, buf: bytes) -> "Datagram":

131          h = Header.unpack(buf)

132          off = HEADER_LEN

133          tel = b""

134          if h.flags & Flag.TELEMETRY_PIGGYBACK:

135              tel = buf[off:off + 16]

136              off += 16

137          return cls(header=h, payload=buf[off:], telemetry=tel)

138

139      @property

140      def size(self) -> int:

141          return HEADER_LEN + len(self.telemetry) + len(self.payload)

142

143

144  # --------------------------------------------------------------------------

145  # Integrity primitives.

146  #

147  # These exist partly because we genuinely use them, and partly because the

148  # course syllabus asks for error detection and correction -- so we implement

149  # the classic Data Link Layer codes and then show, with measurements, why a

150  # real-time voice transport uses a detection code (CRC) plus erasure coding

151  # rather than a bit-level correcting code (Hamming).

152  # --------------------------------------------------------------------------

153

154  def crc32(data: bytes) -> int:

155      """CRC-32 (IEEE 802.3 polynomial) -- the same code Ethernet puts in its FCS."""

156      return zlib.crc32(data) & 0xFFFFFFFF

157

158

159  def internet_checksum(data: bytes) -> int:

160      """RFC 1071 one's-complement checksum -- the code UDP itself carries."""

161      if len(data) % 2:

162          data += b"\x00"

163      total = 0

164      for i in range(0, len(data), 2):

165          total += (data[i] << 8) | data[i + 1]

166          total = (total & 0xFFFF) + (total >> 16)

167      return (~total) & 0xFFFF

168

169

170  def parity_2d(block: bytes, width: int = 8) -> bytes:

171      """Two-dimensional even parity: one parity bit per row and per column.

172

173      Detects all 1-, 2- and 3-bit errors and corrects any single-bit error.

174      """

175      rows = [block[i:i + width] for i in range(0, len(block), width)]

176      row_par = bytes((sum(bin(b).count("1") for b in r) & 1) for r in rows)

177      col_par = bytearray(width)

178      for r in rows:

179          for j, b in enumerate(r):

180              col_par[j] ^= b

181      return row_par + bytes(col_par)

182

183

184  _HAMMING_G = [0b1101, 0b1011, 0b1000, 0b0111, 0b0100, 0b0010, 0b0001]

185

186

187  def hamming74_encode(nibble: int) -> int:

188      """Hamming(7,4): 4 data bits -> 7 code bits, corrects any single-bit error."""

189      d = [(nibble >> i) & 1 for i in (3, 2, 1, 0)]

190      p1 = d[0] ^ d[1] ^ d[3]

191      p2 = d[0] ^ d[2] ^ d[3]

192      p3 = d[1] ^ d[2] ^ d[3]

193      return (p1 << 6) | (p2 << 5) | (d[0] << 4) | (p3 << 3) | (d[1] << 2) | (d[2] << 1) | d[3]

194

195

196  def hamming74_decode(code: int) -> tuple[int, bool]:

197      """Return (4-bit data, corrected?) after single-error correction."""

198      b = [(code >> i) & 1 for i in range(6, -1, -1)]  # b[0]=p1 .. b[6]=d4

199      s1 = b[0] ^ b[2] ^ b[4] ^ b[6]

200      s2 = b[1] ^ b[2] ^ b[5] ^ b[6]

201      s3 = b[3] ^ b[4] ^ b[5] ^ b[6]

202      syn = (s3 << 2) | (s2 << 1) | s1

203      corrected = False

204      if syn:

205          b[syn - 1] ^= 1

206          corrected = True

207      return (b[2] << 3) | (b[4] << 2) | (b[5] << 1) | b[6], corrected

Listing 2   swara/perceptual.py	240 lines

The perceptual importance analyser and the damage weight.

1  """

2  Perceptual Importance (PI) analysis -- SWARA's core novel contribution.

3

4  Conventional VoIP treats every 20 ms packet as equally valuable.  That is

5  demonstrably false.  Human speech intelligibility is dominated by a minority of

6  frames:

7

8    * Voiced vowel nuclei carry the formant structure that identifies the phoneme.

9    * Plosive / vowel ONSETS carry the transient cues that mark word boundaries.

10    * Unvoiced fricatives (/s/, /f/) are noise-like: the ear tolerates substitution.

11    * Silence and background noise frames are perceptually free to lose.

12

13  The PI scorer converts each 20 ms frame into a single octet, 0..255, estimating

14  "how much damage does losing THIS packet do to what the listener understands".

15  That octet is then carried in the clear (but authenticated) in the SWARA header,

16  where the FEC scheduler, the DiffServ marker, the relay and the receiver's

17  concealment engine can all act on it.

18

19  Features, all computed in O(N) or O(N log N) on a 320-sample (20 ms @ 16 kHz) frame:

20

21    E     frame energy in dB, normalised against an adaptive noise floor

22    V     voicing probability from normalised autocorrelation peak (pitch strength)

23    ZCR   zero-crossing rate, the classic voiced/unvoiced discriminator

24    F     spectral flux against the previous frame -- an onset detector

25    VAD   speech activity gate derived from the adaptive noise floor

26

27      PI_raw = w_E*E' + w_V*V + w_F*F' + w_O*onset_boost

28      PI     = 255 * VAD * clip(PI_raw, 0, 1)

29

30  Design targets (verified in `analyse_corpus`):

31      silence / noise                 ->    0 -  25

32      steady unvoiced fricative       ->   50 - 110

33      steady voiced vowel             ->  150 - 210

34      vowel or plosive ONSET          ->  210 - 255

35  """

36

37  from __future__ import annotations

38

39  import numpy as np

40  from dataclasses import dataclass, field

41

42  SAMPLE_RATE = 16_000

43  FRAME_MS = 20

44  FRAME_LEN = SAMPLE_RATE * FRAME_MS // 1000   # 320 samples

45

46  # Feature weights.  Tuned so that the four target bands above are hit; the sum

47  # of the positive weights is 1.0 so PI_raw is naturally in [0, 1].

48  W_ENERGY = 0.30

49  W_VOICING = 0.34

50  W_FLUX = 0.20

51  W_ONSET = 0.16

52

53  # The raw normalised autocorrelation peak of a genuinely voiced frame sits

54  # around 0.5, not 1.0, because the analysis window contains only a few pitch

55  # periods.  Rescaling against the measured operating range is what turns the

56  # feature into a usable voiced/unvoiced discriminator.

57  VOICING_FLOOR = 0.12

58  VOICING_SPAN = 0.45

59

60  # A frame that begins a talkspurt carries the word-boundary cue.  Losing it

61  # costs far more than losing a frame in the middle of a steady vowel, so a hard

62  # onset earns a multiplicative boost on top of the additive features.

63  HARD_ONSET_BOOST = 1.45

64  SPEECH_FLOOR = 0.12   # any speech frame is worth at least this much

65

66

67  @dataclass

68  class FrameFeatures:

69      energy_db: float

70      energy_norm: float

71      voicing: float

72      zcr: float

73      flux: float

74      onset: float

75      is_speech: bool

76      pi: int

77

78      def label(self) -> str:

79          if not self.is_speech:

80              return "silence"

81          if self.onset > 0.5:

82              return "onset"

83          if self.voicing > 0.5:

84              return "voiced"

85          return "unvoiced"

86

87

88  class PerceptualImportanceAnalyzer:

89      """Streaming, causal, single-frame-latency PI scorer.

90

91      Causality matters: a real encoder cannot look ahead without adding delay, so

92      every feature here uses only the current frame and state accumulated from

93      past frames.  The onset detector is therefore a *rising-edge* detector, not

94      a lookahead peak-picker.

95      """

96

97      def __init__(self, sample_rate: int = SAMPLE_RATE, frame_len: int = FRAME_LEN):

98          self.sr = sample_rate

99          self.n = frame_len

100          self._prev_spec: np.ndarray | None = None

101          self._prev_energy_norm = 0.0

102          self._prev_speech = False

103          self._noise_floor_db = -60.0   # adaptive, tracks the quietest recent frames

104          self._peak_db = -20.0          # adaptive, tracks the loudest recent frames

105          self._window = np.hanning(frame_len)

106          self._min_lag = max(2, int(sample_rate / 400))   # 400 Hz -> highest pitch

107          self._max_lag = min(frame_len - 1, int(sample_rate / 60))  # 60 Hz -> lowest

108          self.frames_seen = 0

109

110      # -- individual features ------------------------------------------------

111

112      def _energy_db(self, x: np.ndarray) -> float:

113          rms = float(np.sqrt(np.mean(x.astype(np.float64) ** 2)) + 1e-12)

114          return 20.0 * np.log10(rms)

115

116      def _voicing(self, x: np.ndarray) -> float:

117          """Normalised autocorrelation peak in the plausible pitch range.

118

119          Returns ~1.0 for a strongly periodic (voiced) frame, ~0.0 for noise.

120          """

121          x = x - x.mean()

122          e0 = float(np.dot(x, x))

123          if e0 < 1e-9:

124              return 0.0

125          # FFT-based autocorrelation: O(N log N)

126          nfft = 1 << (2 * self.n - 1).bit_length()

127          spec = np.fft.rfft(x, nfft)

128          ac = np.fft.irfft(spec * np.conj(spec), nfft)[: self.n]

129          lo, hi = self._min_lag, self._max_lag

130          if hi <= lo:

131              return 0.0

132          seg = ac[lo:hi]

133          if seg.size == 0:

134              return 0.0

135          return float(np.clip(seg.max() / e0, 0.0, 1.0))

136

137      def _zcr(self, x: np.ndarray) -> float:

138          return float(np.mean(np.abs(np.diff(np.signbit(x).astype(np.int8)))))

139

140      def _flux(self, x: np.ndarray) -> float:

141          """Half-wave-rectified spectral flux: energy that has just APPEARED."""

142          spec = np.abs(np.fft.rfft(x * self._window))

143          spec = spec / (spec.sum() + 1e-9)

144          if self._prev_spec is None:

145              self._prev_spec = spec

146              return 0.0

147          d = spec - self._prev_spec

148          self._prev_spec = spec

149          return float(np.clip(np.sum(d[d > 0]) * 2.0, 0.0, 1.0))

150

151      # -- main entry point ---------------------------------------------------

152

153      def analyze(self, frame: np.ndarray) -> FrameFeatures:

154          x = np.asarray(frame, dtype=np.float64)

155          if x.size < self.n:

156              x = np.pad(x, (0, self.n - x.size))

157          x = x[: self.n]

158

159          e_db = self._energy_db(x)

160

161          # Adaptive noise floor: fast to rise into silence, slow to fall.

162          if e_db < self._noise_floor_db:

163              self._noise_floor_db += 0.35 * (e_db - self._noise_floor_db)

164          else:

165              self._noise_floor_db += 0.004 * (e_db - self._noise_floor_db)

166          # Adaptive peak: fast attack, slow release.

167          if e_db > self._peak_db:

168              self._peak_db += 0.5 * (e_db - self._peak_db)

169          else:

170              self._peak_db += 0.002 * (e_db - self._peak_db)

171

172          span = max(12.0, self._peak_db - self._noise_floor_db)

173          e_norm = float(np.clip((e_db - self._noise_floor_db) / span, 0.0, 1.0))

174

175          zcr = self._zcr(x)

176          flux = self._flux(x)

177

178          # A plosive burst is only ~8 ms of a 20 ms frame, so its average energy

179          # is low even though it is perceptually a transient.  Admitting frames

180          # on high spectral flux as well as on energy catches them -- but the

181          # flux route must still clear an absolute energy bar, or the random

182          # spectral wander of the noise floor is mistaken for speech.

183          above_floor = e_db - self._noise_floor_db

184          is_speech = above_floor > 6.0 or (flux > 0.62 and above_floor > 4.0)

185

186          voicing_raw = self._voicing(x) if is_speech else 0.0

187          voicing = float(np.clip((voicing_raw - VOICING_FLOOR) / VOICING_SPAN, 0.0, 1.0))

188

189          # Onset = a sharp rise in normalised energy relative to the previous frame.

190          rise = e_norm - self._prev_energy_norm

191          onset = float(np.clip(rise * 3.2, 0.0, 1.0)) if is_speech else 0.0

192          hard_onset = is_speech and not self._prev_speech

193          if hard_onset:

194              onset = 1.0

195          self._prev_energy_norm = e_norm

196          self._prev_speech = is_speech

197

198          # High ZCR with low voicing means fricative noise: still needed for

199          # intelligibility, but far more concealable than a vowel, so we damp it.

200          voicing_eff = voicing * 0.6 if (is_speech and voicing < 0.4 and zcr > 0.20) else voicing

201

202          raw = (W_ENERGY * e_norm + W_VOICING * voicing_eff

203                 + W_FLUX * flux + W_ONSET * onset)

204

205          if is_speech:

206              scaled = SPEECH_FLOOR + (1.0 - SPEECH_FLOOR) * float(np.clip(raw, 0.0, 1.0))

207              if hard_onset:

208                  scaled *= HARD_ONSET_BOOST

209              pi = int(round(255.0 * min(scaled, 1.0)))

210          else:

211              pi = int(round(255.0 * 0.06 * e_norm))

212

213          self.frames_seen += 1

214          return FrameFeatures(

215              energy_db=e_db, energy_norm=e_norm, voicing=voicing, zcr=zcr,

216              flux=flux, onset=onset, is_speech=is_speech, pi=int(np.clip(pi, 0, 255)),

217          )

218

219

220  # ---------------------------------------------------------------------------

221  # Perceptual damage model.

222  #

223  # We need a way to say "this frame was lost -- how much did the listener

224  # suffer?"  We use a damage weight derived from PI, calibrated so that losing a

225  # silence frame costs ~0 and losing a voiced onset costs ~1.  This is what makes

226  # unequal protection measurable rather than merely plausible, and it is what

227  # feeds the effective-loss term of the E-model in emodel.py.

228  # ---------------------------------------------------------------------------

229

230  def damage_weight(pi: int, concealed_neighbours: int = 0) -> float:

231      """Perceptual cost of losing a frame with the given PI score.

232

233      The curve is superlinear (exponent 1.6): a frame twice as important is

234      roughly three times as costly to lose.  Consecutive losses are penalised

235      extra because packet-loss concealment degrades sharply once it has to

236      invent more than ~40 ms of speech from nothing.

237      """

238      base = (pi / 255.0) ** 1.6

239      burst_penalty = 1.0 + 0.45 * min(concealed_neighbours, 4)

240      return float(base * burst_penalty)

Listing 3   swara/fec.py	398 lines

GF(2⁸) arithmetic, systematic Reed–Solomon erasure code, PI-AFEC scheduler and the uniform comparator.

1  """

2  PI-AFEC: Perceptual-Importance-weighted Adaptive Forward Error Correction.

3

4  Two layers:

5

6  1.  A systematic Reed-Solomon erasure code over GF(2^8).  A group of k media

7      packets is extended with r parity packets; any k of the k+r survivors

8      reconstruct the group.  Systematic means the k data packets are sent

9      unmodified, so a group that arrives intact costs zero decoding work.

10

11  2.  The novel part -- the SCHEDULER that decides r, and the "perceptual anchor

12      replica" that protects the single most important frame independently.

13

14  Why this is not just "FEC with a knob":

15

16    Standard adaptive FEC (RFC 5109 ULP-FEC, Opus LBRR, WebRTC's FlexFEC) picks

17    redundancy from the NETWORK state alone -- more loss, more parity.  It spends

18    exactly as much protecting a frame of silence as a frame carrying a vowel

19    onset.  PI-AFEC picks redundancy from the JOINT (network, content) state:

20

21        r = clip( round( k * rho_net * (Gbar)^gamma ), 0, r_max )

22

23    where rho_net in [0,1] is the network-driven redundancy budget from the NAVIK

24    controller and Gbar in [0,1] is the mean normalised PI of the group.  gamma

25    < 1 (we use 0.65) keeps moderately-important groups from being starved.

26

27    Because the total parity budget is held FIXED against the baseline, every

28    parity packet withheld from a silent group is spent on a group that carries

29    meaning.  That reallocation is the entire source of the measured gain.

30

31  3.  Perceptual anchor replica.  RS parity only helps if at least k of k+r

32      packets survive.  Under a long burst that condition fails and the whole

33      group is lost -- including its most important frame.  So when a frame's PI

34      exceeds an adaptive threshold, a compact replica of that ONE frame is

35      piggybacked onto a later packet, deliberately spaced by the estimated mean

36      burst length so that the original and the replica are unlikely to be in the

37      same burst.  It is a targeted, content-aware, delay-bounded duplicate.

38  """

39

40  from __future__ import annotations

41

42  import numpy as np

43  from dataclasses import dataclass, field

44

45  # ---------------------------------------------------------------------------

46  # GF(2^8) arithmetic, primitive polynomial 0x11D (the same field as CD-ROM,

47  # QR codes and RAID-6).  Tables are built once at import time.

48  # ---------------------------------------------------------------------------

49

50  _PRIM = 0x11D

51  GF_EXP = np.zeros(512, dtype=np.uint8)

52  GF_LOG = np.zeros(256, dtype=np.uint8)

53

54

55  def _build_tables() -> None:

56      x = 1

57      for i in range(255):

58          GF_EXP[i] = x

59          GF_LOG[x] = i

60          x <<= 1

61          if x & 0x100:

62              x ^= _PRIM

63      for i in range(255, 512):

64          GF_EXP[i] = GF_EXP[i - 255]

65

66

67  _build_tables()

68

69  # Full 256x256 multiplication table -> vectorised, branch-free encode/decode.

70  _A = np.arange(256, dtype=np.int32)

71  _LOGA = GF_LOG[_A].astype(np.int32)

72  _MULTAB = GF_EXP[(_LOGA[:, None] + _LOGA[None, :]) % 255].astype(np.uint8)

73  _MULTAB[0, :] = 0

74  _MULTAB[:, 0] = 0

75

76

77  def gf_mul(a: int, b: int) -> int:

78      return int(_MULTAB[a, b])

79

80

81  def gf_inv(a: int) -> int:

82      if a == 0:

83          raise ZeroDivisionError("GF(256) inverse of 0")

84      return int(GF_EXP[(255 - int(GF_LOG[a])) % 255])

85

86

87  def gf_vec_mul(vec: np.ndarray, scalar: int) -> np.ndarray:

88      """Multiply a whole byte-vector by a scalar in GF(256) via table lookup."""

89      return _MULTAB[scalar][vec]

90

91

92  def _vandermonde(rows: int, cols: int) -> np.ndarray:

93      """Cauchy-style generator rows.  Any square submatrix is invertible, which

94      is exactly the MDS property an erasure code needs."""

95      m = np.zeros((rows, cols), dtype=np.uint8)

96      for i in range(rows):

97          a = i + 1                      # distinct field elements for parity rows

98          for j in range(cols):

99              b = 128 + j                # disjoint from the parity element set

100              m[i, j] = gf_inv(a ^ b)

101          # Normalise the first column to 1 -> better numerical hygiene, and it

102          # makes the r=1 case reduce to a plain XOR, which is free.

103      for i in range(rows):

104          s = gf_inv(int(m[i, 0])) if m[i, 0] else 1

105          for j in range(cols):

106              m[i, j] = gf_mul(int(m[i, j]), s)

107      return m

108

109

110  class ReedSolomon:

111      """Systematic RS(k+r, k) erasure code over GF(256), byte-parallel."""

112

113      def __init__(self, k: int, r: int):

114          if not (1 <= k <= 120) or not (0 <= r <= 120):

115              raise ValueError("k and r out of supported range")

116          self.k, self.r = k, r

117          self.gen = _vandermonde(r, k) if r else np.zeros((0, k), dtype=np.uint8)

118

119      def encode(self, shards: list[bytes]) -> list[bytes]:

120          """shards: k equal-length byte strings -> r parity byte strings."""

121          if len(shards) != self.k:

122              raise ValueError(f"expected {self.k} shards, got {len(shards)}")

123          if self.r == 0:

124              return []

125          n = max(len(s) for s in shards)

126          data = np.zeros((self.k, n), dtype=np.uint8)

127          for i, s in enumerate(shards):

128              data[i, : len(s)] = np.frombuffer(s, dtype=np.uint8)

129          out = np.zeros((self.r, n), dtype=np.uint8)

130          for i in range(self.r):

131              acc = np.zeros(n, dtype=np.uint8)

132              for j in range(self.k):

133                  c = int(self.gen[i, j])

134                  if c:

135                      acc ^= gf_vec_mul(data[j], c)

136              out[i] = acc

137          return [out[i].tobytes() for i in range(self.r)]

138

139      def decode(self, received: dict[int, bytes], shard_len: int) -> dict[int, bytes] | None:

140          """received: {index -> bytes}; data indices 0..k-1, parity k..k+r-1.

141

142          Returns the recovered data shards, or None if fewer than k survived.

143          """

144          present = sorted(received.keys())

145          if len(present) < self.k:

146              return None

147          have_data = [i for i in present if i < self.k]

148          if len(have_data) == self.k:

149              return {i: received[i] for i in have_data}

150

151          use = present[: self.k]

152          # Build the k x k matrix mapping unknown data shards -> the shards we have.

153          missing = [i for i in range(self.k) if i not in received]

154          known = [i for i in range(self.k) if i in received]

155          par_use = [i for i in use if i >= self.k][: len(missing)]

156          if len(par_use) < len(missing):

157              return None

158

159          m = np.zeros((len(missing), len(missing)), dtype=np.uint8)

160          for a, p in enumerate(par_use):

161              for b, d in enumerate(missing):

162                  m[a, b] = self.gen[p - self.k, d]

163

164          # Right-hand side = parity minus the contribution of the known shards.

165          rhs = np.zeros((len(missing), shard_len), dtype=np.uint8)

166          for a, p in enumerate(par_use):

167              acc = np.frombuffer(received[p].ljust(shard_len, b"\x00"), dtype=np.uint8).copy()

168              for d in known:

169                  c = int(self.gen[p - self.k, d])

170                  if c:

171                      acc ^= gf_vec_mul(

172                          np.frombuffer(received[d].ljust(shard_len, b"\x00"), dtype=np.uint8), c)

173              rhs[a] = acc

174

175          sol = self._solve(m, rhs)

176          if sol is None:

177              return None

178          out = {i: received[i] for i in known}

179          for b, d in enumerate(missing):

180              out[d] = sol[b].tobytes()

181          return out

182

183      @staticmethod

184      def _solve(m: np.ndarray, rhs: np.ndarray) -> np.ndarray | None:

185          """Gauss-Jordan elimination over GF(256)."""

186          n = m.shape[0]

187          m = m.copy()

188          rhs = rhs.copy()

189          for col in range(n):

190              piv = next((r for r in range(col, n) if m[r, col]), None)

191              if piv is None:

192                  return None

193              if piv != col:

194                  m[[col, piv]] = m[[piv, col]]

195                  rhs[[col, piv]] = rhs[[piv, col]]

196              inv = gf_inv(int(m[col, col]))

197              m[col] = gf_vec_mul(m[col], inv)

198              rhs[col] = gf_vec_mul(rhs[col], inv)

199              for r in range(n):

200                  if r != col and m[r, col]:

201                      f = int(m[r, col])

202                      m[r] ^= gf_vec_mul(m[col], f)

203                      rhs[r] ^= gf_vec_mul(rhs[col], f)

204          return rhs

205

206

207  # ---------------------------------------------------------------------------

208  # The PI-AFEC scheduler

209  # ---------------------------------------------------------------------------

210

211  GAMMA = 0.65        # importance -> redundancy shaping exponent

212  ANCHOR_PI_FLOOR = 170   # a frame must be at least this important to earn a replica

213

214

215  @dataclass

216  class FecPlan:

217      """What the scheduler decided for one group of k frames."""

218      k: int

219      r: int

220      group_mean_pi: float

221      anchors: list = field(default_factory=list)   # [(index_in_group, delay_frames)]

222      overhead_ratio: float = 0.0

223      burst_mode: bool = False

224

225      @property

226      def anchor_index(self):

227          return self.anchors[0][0] if self.anchors else None

228

229

230  @dataclass

231  class FecScheduler:

232      """Decides parity count and anchor placement for each group.

233

234      `rho_net` and `mean_burst` are written by the NAVIK controller from receiver

235      telemetry; the scheduler itself is stateless apart from those two inputs and

236      a running average used to hold the overhead budget on target.

237      """

238      k: int = 8

239      r_max: int = 6

240      rho_net: float = 0.0          # network-driven redundancy budget, 0..1

241      mean_burst: float = 1.0       # estimated mean loss-burst length, packets

242      anchors_enabled: bool = True

243      _spent: float = field(default=0.0, repr=False)

244      _groups: int = field(default=0, repr=False)

245      _carry_r: float = field(default=0.0, repr=False)

246      _carry_a: float = field(default=0.0, repr=False)

247      _wsum: float = field(default=0.6, repr=False)     # running mean of the

248      _wcount: float = field(default=1.0, repr=False)   # importance weight

249

250      def plan(self, pi_scores: list[int]) -> FecPlan:

251          """Decide parity and anchors for one group.

252

253          The branch on `burst_mode` is a substantive design decision, not a

254          tuning detail.  An RS(k+r, k) group survives only if at least k of its

255          k+r packets arrive.  When the mean loss burst approaches the group's

256          own span, that condition fails almost every time the channel goes bad,

257          and every parity byte is wasted.

258

259          Classical coding theory answers this with INTERLEAVING -- spread the

260          group's packets across a wide time window so a burst hits at most one

261          of them.  Real-time voice cannot use it: an interleaver of depth D over

262          a group of k frames costs k*D frame times of buffering, so even D=3 at

263          k=8 adds 480 ms and destroys conversational interactivity.

264

265          SWARA's answer is to interleave ONLY what matters.  In burst mode the

266          parity budget is redirected into REPLICAS of the highest-PI frames,

267          placed at offsets chosen to exceed the estimated burst length.  One

268          replica of one frame costs one packet and one frame-time of extra

269          delay, and it is the only mechanism that survives a burst longer than

270          the group itself.

271          """

272          k = len(pi_scores)

273          if k == 0:

274              return FecPlan(k=0, r=0, group_mean_pi=0.0)

275          arr = np.asarray(pi_scores, dtype=float)

276          gbar = float(arr.mean()) / 255.0

277

278          # The weight is accumulated for EVERY group, including the silent ones

279          # that are about to be skipped.  That ordering is the whole mechanism:

280          # a skipped group contributes a weight of nearly zero to the running

281          # mean, which lowers the mean, which raises every other group's share.

282          # Accumulating after the skip instead -- as this did -- means a silent

283          # group's budget is simply forfeited rather than handed on, and the

284          # scheme ends up spending less overall instead of spending the same

285          # amount better. Measured cost of that ordering: 164 parity packets

286          # against the uniform comparator's 373.

287          weight = gbar ** GAMMA

288          self._wsum += weight

289          self._wcount += 1.0

290

291          # A group with genuinely no speech in it gets nothing at all, however

292          # bad the network is.  This is where the reallocated budget comes from.

293          if gbar < 0.08:

294              self._groups += 1

295              return FecPlan(k=k, r=0, group_mean_pi=gbar, overhead_ratio=0.0)

296

297          # --- budget: REDISTRIBUTE, do not shrink -------------------------

298          #

299          # The whole claim of PI-AFEC is "the same budget, spent where it is

300          # heard".  The obvious implementation, budget = k * rho * Gbar^gamma,

301          # does not do that.  Because Gbar^gamma is always below 1, it spends

302          # systematically LESS than the rho it was given -- a nominal 50%

303          # redundancy quietly became about 20% in the live system, and SWARA

304          # lost to the uniform comparator on both quality and, absurdly, on the

305          # thing it was supposed to be better at.

306          #

307          # Normalising the weight against its own running mean fixes it: the

308          # EXPECTED spend is exactly k * rho, while individual groups receive

309          # more or less than that in proportion to how much they matter.  Now it

310          # really is redistribution rather than a hidden discount.

311          wbar = max(0.05, self._wsum / self._wcount)

312          ratio = float(np.clip(weight / wbar, 0.0, 4.0))

313          budget_pkts = k * self.rho_net * ratio

314

315          # An RS(k+r, k) group survives r erasures.  Reed-Solomon is therefore

316          # worth funding whenever the typical burst is smaller than the parity we

317          # can afford, and only becomes futile once bursts approach the group's

318          # own span.  An earlier threshold of k/2.5 declared futility at a burst

319          # of 1.6 packets for k=4 -- a regime where RS in fact works perfectly

320          # well -- and diverted the entire budget into replicas.  The measured

321          # consequence was SWARA transmitting almost no redundancy at all while

322          # the uniform comparator recovered 53 frames per call.

323          burst_mode = self.mean_burst >= 0.75 * k

324

325          if burst_mode:

326              share_r, share_a = 0.45, 0.55

327          else:

328              share_r, share_a = 0.85, 0.15

329

330          # Fractional budgets are carried between groups rather than rounded

331          # away.  With k=4 a per-group budget of 0.4 packets rounds to zero every

332          # single time, so a nominal 10% redundancy silently becomes 0%.

333          want_r = budget_pkts * share_r + self._carry_r

334          r = int(want_r)

335          self._carry_r = want_r - r

336          r = int(np.clip(r, 0, min(self.r_max, k)))

337

338          want_a = budget_pkts * share_a + self._carry_a

339          n_anchor = int(want_a)

340          self._carry_a = want_a - n_anchor

341          n_anchor = int(np.clip(n_anchor, 0, 4)) if self.anchors_enabled else 0

342

343          anchors: list[tuple[int, int]] = []

344          if self.anchors_enabled and n_anchor:

345              order = np.argsort(-arr)

346              base_delay = int(np.clip(round(self.mean_burst) + 2, 3, 9))

347              for slot, idx in enumerate(order[:n_anchor]):

348                  if arr[idx] < ANCHOR_PI_FLOOR:

349                      break

350                  # Stagger successive replicas so two of them are never inside

351                  # the same outage either.

352                  anchors.append((int(idx), base_delay + slot * 2))

353

354          overhead = (r + len(anchors)) / k

355          self._spent += overhead

356          self._groups += 1

357          return FecPlan(k=k, r=r, group_mean_pi=gbar, anchors=anchors,

358                         overhead_ratio=overhead, burst_mode=burst_mode)

359

360      @property

361      def realised_overhead(self) -> float:

362          return self._spent / self._groups if self._groups else 0.0

363

364

365  @dataclass

366  class UniformFecScheduler:

367      """Baseline comparator: classic loss-driven FEC with no content awareness.

368

369      This is what RFC 5109 / FlexFEC style adaptive redundancy does.  It is a

370      strong baseline, not a straw man -- it adapts to the network correctly.  It

371      simply cannot tell a vowel from a silence.

372      """

373      k: int = 8

374      r_max: int = 6

375      rho_net: float = 0.0

376      mean_burst: float = 1.0

377      _spent: float = field(default=0.0, repr=False)

378      _groups: int = field(default=0, repr=False)

379

380      def plan(self, pi_scores: list[int]) -> FecPlan:

381          k = max(1, len(pi_scores))

382          # Fractional budgets are honoured probabilistically so that a rho of,

383          # say, 0.14 really does cost 0.14*k packets on average rather than

384          # rounding to zero and quietly under-spending the budget it was given.

385          exact = k * self.rho_net

386          r = int(exact)

387          if np.random.random() < (exact - r):

388              r += 1

389          r = int(np.clip(r, 0, min(self.r_max, k)))

390          self._spent += r / k

391          self._groups += 1

392          return FecPlan(k=k, r=r,

393                         group_mean_pi=float(np.mean(pi_scores)) / 255.0 if pi_scores else 0.0,

394                         anchors=[], overhead_ratio=r / k)

395

396      @property

397      def realised_overhead(self) -> float:

398          return self._spent / self._groups if self._groups else 0.0

Listing 4   swara/auth.py	475 lines

SPARK: registration, server and client handshake, key schedule, tickets, migration, media AEAD, sequence reconstruction and replay window.

1  """

2  SPARK -- Stateless Password-Authenticated Retry & Key-agreement.

3

4  The authentication problem for voice over UDP is not "how do I check a

5  password".  It is:

6

7    (a) UDP is trivially spoofable, so an unauthenticated first packet must never

8        make the server allocate memory or emit a larger reply -- otherwise the

9        service is a DDoS amplifier;

10    (b) a password must never cross the wire, and a stolen server database must

11        not yield anything replayable;

12    (c) the handshake must not cost so many round trips that call setup feels

13        slow -- and after a WiFi-to-mobile handover it must cost NONE, because

14        the call is already in progress and the user is mid-sentence.

15

16  SPARK addresses all three:

17

18    Phase 0  HELLO        16 B, no state created, no user identity revealed.

19    Phase 1  RETRY        server returns a stateless HMAC cookie bound to the

20                          client's address and a coarse clock.  Reply is SMALLER

21                          than the request -> amplification factor < 1.

22    Phase 2  AUTH_INIT    client echoes the cookie.  Only now, having proved it

23                          can receive at the address it claims, does the server

24                          do any work.

25    Phase 3  AUTH_RESP    server returns the user's salt and OPAQUE-style

26                          envelope plus a fresh ephemeral X25519 key.

27    Phase 4  AUTH_CONFIRM triple Diffie-Hellman is mixed into the transcript and

28                          the client proves key knowledge.  Mutual auth complete.

29

30  Key schedule (all HKDF-SHA256 over the full transcript hash, so every byte of

31  every handshake message is authenticated):

32

33      dh1 = X25519(client_ephemeral,  server_static)     -- authenticates server

34      dh2 = X25519(client_static,     server_ephemeral)  -- authenticates client

35      dh3 = X25519(client_ephemeral,  server_ephemeral)  -- forward secrecy

36      MS  = HKDF-Extract(salt = H(transcript), IKM = dh1 || dh2 || dh3)

37

38  The client's static private key never leaves the client and is not stored

39  anywhere in plaintext: it lives inside an AEAD envelope whose key is derived

40  from the password by PBKDF2.  The server stores only {salt, envelope, client

41  static PUBLIC key}.  Stealing that database gives an attacker a target for an

42  offline dictionary attack -- and nothing that can be replayed at the server,

43  because possession of the envelope alone does not yield the static private key.

44

45  Resumption / path migration:

46      On success the server issues a ticket = AEAD(K_ticket_server, state).  The

47      server keeps NO per-session table for this.  When the client's address

48      changes it sends PATH_MIGRATE {CID, ticket, AEAD(K_resume, challenge)} from

49      the new address; the server decrypts the ticket, verifies the tag, rebinds

50      the CID and the media flow continues.  Zero round trips of silence.

51  """

52

53  from __future__ import annotations

54

55  import hashlib

56  import hmac

57  import os

58  import struct

59  import time

60  from dataclasses import dataclass, field

61

62  from cryptography.hazmat.primitives.asymmetric.x25519 import (

63      X25519PrivateKey, X25519PublicKey)

64  from cryptography.hazmat.primitives.ciphers.aead import ChaCha20Poly1305

65  from cryptography.hazmat.primitives.kdf.hkdf import HKDFExpand

66  from cryptography.hazmat.primitives import hashes

67

68  PBKDF2_ITERS = 200_000

69  COOKIE_LIFETIME_S = 30

70  TICKET_LIFETIME_S = 3600

71  KEY_LEN = 32

72

73

74  # ---------------------------------------------------------------------------

75  # Primitives

76  # ---------------------------------------------------------------------------

77

78  def hkdf_expand(prk: bytes, label: bytes, length: int = KEY_LEN) -> bytes:

79      return HKDFExpand(algorithm=hashes.SHA256(), length=length,

80                        info=b"SWARA-v1 " + label).derive(prk)

81

82

83  def hkdf_extract(salt: bytes, ikm: bytes) -> bytes:

84      return hmac.new(salt, ikm, hashlib.sha256).digest()

85

86

87  def stretch_password(password: str, salt: bytes) -> bytes:

88      return hashlib.pbkdf2_hmac("sha256", password.encode(), salt, PBKDF2_ITERS, KEY_LEN)

89

90

91  class Transcript:

92      """Running SHA-256 over every handshake byte, in order."""

93

94      def __init__(self) -> None:

95          self._h = hashlib.sha256(b"SWARA-v1 transcript")

96

97      def add(self, *chunks: bytes) -> "Transcript":

98          for c in chunks:

99              self._h.update(struct.pack("!H", len(c)))

100              self._h.update(c)

101          return self

102

103      def digest(self) -> bytes:

104          return self._h.copy().digest()

105

106

107  # ---------------------------------------------------------------------------

108  # Credential registration (performed once, out of band)

109  # ---------------------------------------------------------------------------

110

111  @dataclass

112  class CredentialRecord:

113      """Exactly what the server stores per user.  Note the absence of anything

114      resembling the password or a password-equivalent verifier."""

115      username: str

116      salt: bytes

117      envelope: bytes            # AEAD(PBKDF2(pw,salt), client static private key)

118      client_static_pub: bytes   # 32-byte X25519 public key

119

120      def size_bytes(self) -> int:

121          return len(self.salt) + len(self.envelope) + len(self.client_static_pub)

122

123

124  def register(username: str, password: str) -> tuple[CredentialRecord, bytes]:

125      """Create a credential record. Returns (record, client_static_private)."""

126      salt = os.urandom(16)

127      rw = stretch_password(password, salt)

128      static_priv = X25519PrivateKey.generate()

129      priv_raw = static_priv.private_bytes_raw()

130      pub_raw = static_priv.public_key().public_bytes_raw()

131      aead = ChaCha20Poly1305(rw)

132      nonce = b"\x00" * 12

133      envelope = aead.encrypt(nonce, priv_raw + pub_raw, username.encode())

134      return CredentialRecord(username, salt, envelope, pub_raw), priv_raw

135

136

137  # ---------------------------------------------------------------------------

138  # Session keys

139  # ---------------------------------------------------------------------------

140

141  @dataclass

142  class SessionKeys:

143      cid: int

144      k_c2s: bytes

145      k_s2c: bytes

146      k_resume: bytes

147      transcript: bytes

148

149      @classmethod

150      def derive(cls, master: bytes, transcript: bytes) -> "SessionKeys":

151          cid = struct.unpack("!I", hkdf_expand(master, b"cid", 4))[0]

152          return cls(cid=cid,

153                     k_c2s=hkdf_expand(master, b"c2s"),

154                     k_s2c=hkdf_expand(master, b"s2c"),

155                     k_resume=hkdf_expand(master, b"resume"),

156                     transcript=transcript)

157

158

159  # ---------------------------------------------------------------------------

160  # Server side

161  # ---------------------------------------------------------------------------

162

163  class SparkServer:

164      def __init__(self) -> None:

165          self.static_priv = X25519PrivateKey.generate()

166          self.static_pub = self.static_priv.public_key().public_bytes_raw()

167          self._cookie_key = os.urandom(32)

168          self._ticket_key = os.urandom(32)

169          self.users: dict[str, CredentialRecord] = {}

170          # CID -> (keys, current address).  Populated only AFTER full auth.

171          self.sessions: dict[int, tuple[SessionKeys, tuple]] = {}

172          self.stats = {"hello": 0, "cookie_bad": 0, "auth_ok": 0, "auth_fail": 0,

173                        "resumed": 0, "migrations": 0, "bytes_out_pre_auth": 0,

174                        "bytes_in_pre_auth": 0}

175

176      # -- registration ------------------------------------------------------

177

178      def add_user(self, record: CredentialRecord) -> None:

179          self.users[record.username] = record

180

181      # -- phase 1: stateless retry -----------------------------------------

182

183      def _cookie(self, addr: tuple, epoch: int) -> bytes:

184          msg = f"{addr[0]}|{addr[1]}|{epoch}".encode()

185          return hmac.new(self._cookie_key, msg, hashlib.sha256).digest()[:16]

186

187      def handle_hello(self, addr: tuple, hello: bytes) -> bytes:

188          """Zero state is created here.  The reply is deliberately no larger

189          than the request, so SWARA cannot be used as a reflection amplifier."""

190          self.stats["hello"] += 1

191          self.stats["bytes_in_pre_auth"] += len(hello)

192          epoch = int(time.time() // COOKIE_LIFETIME_S)

193          out = struct.pack("!I", epoch) + self._cookie(addr, epoch)

194          self.stats["bytes_out_pre_auth"] += len(out)

195          return out

196

197      def _cookie_valid(self, addr: tuple, epoch: int, cookie: bytes) -> bool:

198          now = int(time.time() // COOKIE_LIFETIME_S)

199          if epoch not in (now, now - 1):

200              return False

201          return hmac.compare_digest(cookie, self._cookie(addr, epoch))

202

203      # -- phase 3: response -------------------------------------------------

204

205      def handle_auth_init(self, addr: tuple, body: bytes):

206          """body = epoch(4) || cookie(16) || eph_pub(32) || ulen(1) || username"""

207          epoch, = struct.unpack("!I", body[:4])

208          cookie = body[4:20]

209          if not self._cookie_valid(addr, epoch, cookie):

210              self.stats["cookie_bad"] += 1

211              return None, None      # silent drop: spoofed source or stale cookie

212          client_eph = body[20:52]

213          ulen = body[52]

214          username = body[53:53 + ulen].decode(errors="replace")

215          rec = self.users.get(username)

216          if rec is None:

217              # Constant-shape reply for unknown users: no user-enumeration oracle.

218              rec = CredentialRecord(username, hashlib.sha256(username.encode()).digest()[:16],

219                                     os.urandom(80), os.urandom(32))

220              known = False

221          else:

222              known = True

223

224          server_eph_priv = X25519PrivateKey.generate()

225          server_eph_pub = server_eph_priv.public_key().public_bytes_raw()

226

227          resp = (bytes([len(rec.salt)]) + rec.salt

228                  + struct.pack("!H", len(rec.envelope)) + rec.envelope

229                  + server_eph_pub + self.static_pub)

230

231          pending = {"client_eph": client_eph, "server_eph_priv": server_eph_priv,

232                     "rec": rec, "known": known, "addr": addr, "resp": resp,

233                     "transcript": Transcript().add(b"AUTH_INIT", body, b"AUTH_RESP", resp)}

234          return resp, pending

235

236      # -- phase 5: verify confirmation --------------------------------------

237

238      def handle_auth_confirm(self, pending: dict, tag: bytes):

239          rec: CredentialRecord = pending["rec"]

240          if not pending["known"]:

241              self.stats["auth_fail"] += 1

242              return None, None

243

244          eph_priv: X25519PrivateKey = pending["server_eph_priv"]

245          client_eph_pub = X25519PublicKey.from_public_bytes(pending["client_eph"])

246          client_static_pub = X25519PublicKey.from_public_bytes(rec.client_static_pub)

247

248          dh1 = self.static_priv.exchange(client_eph_pub)

249          dh2 = eph_priv.exchange(client_static_pub)

250          dh3 = eph_priv.exchange(client_eph_pub)

251

252          th = pending["transcript"].digest()

253          master = hkdf_extract(th, dh1 + dh2 + dh3)

254          keys = SessionKeys.derive(master, th)

255

256          expect = hmac.new(keys.k_c2s, b"client finished" + th, hashlib.sha256).digest()[:16]

257          if not hmac.compare_digest(tag, expect):

258              self.stats["auth_fail"] += 1

259              return None, None

260

261          self.stats["auth_ok"] += 1

262          self.sessions[keys.cid] = (keys, pending["addr"])

263          ticket = self._issue_ticket(rec.username, keys)

264          return keys, ticket

265

266      # -- resumption tickets ------------------------------------------------

267

268      def _issue_ticket(self, username: str, keys: SessionKeys) -> bytes:

269          nonce = os.urandom(12)

270          state = (struct.pack("!IQ", keys.cid, int(time.time()) + TICKET_LIFETIME_S)

271                   + keys.k_resume + bytes([len(username)]) + username.encode())

272          blob = ChaCha20Poly1305(self._ticket_key).encrypt(nonce, state, b"SWARA-ticket")

273          return nonce + blob

274

275      def handle_path_migrate(self, new_addr: tuple, body: bytes):

276          """0-RTT authenticated migration.  body = ticket_len(2)||ticket||nonce(12)||sealed"""

277          tlen, = struct.unpack("!H", body[:2])

278          ticket = body[2:2 + tlen]

279          rest = body[2 + tlen:]

280          try:

281              state = ChaCha20Poly1305(self._ticket_key).decrypt(

282                  ticket[:12], ticket[12:], b"SWARA-ticket")

283          except Exception:

284              return None

285          cid, expiry = struct.unpack("!IQ", state[:12])

286          if time.time() > expiry:

287              return None

288          k_resume = state[12:44]

289          try:

290              proof = ChaCha20Poly1305(k_resume).decrypt(rest[:12], rest[12:], b"SWARA-migrate")

291          except Exception:

292              return None   # cannot forge without k_resume: an off-path attacker

293                            # cannot hijack a call by replaying the ticket alone

294          if cid in self.sessions:

295              keys, _ = self.sessions[cid]

296              self.sessions[cid] = (keys, new_addr)

297              self.stats["migrations"] += 1

298              return cid, proof

299          return None

300

301

302  # ---------------------------------------------------------------------------

303  # Client side

304  # ---------------------------------------------------------------------------

305

306  class SparkClient:

307      def __init__(self, username: str, password: str):

308          self.username = username

309          self.password = password

310          self.eph_priv = X25519PrivateKey.generate()

311          self.eph_pub = self.eph_priv.public_key().public_bytes_raw()

312          self.keys: SessionKeys | None = None

313          self.ticket: bytes | None = None

314          self._transcript: Transcript | None = None

315          self._init_msg: bytes | None = None

316

317      HELLO_PAD = 64   # RETRY is 20 B, so amplification is bounded at 0.31x

318

319      def make_hello(self) -> bytes:

320          """Deliberately padded.  A reflection attacker gains nothing by

321          spoofing our HELLO because the server's reply is far SMALLER than the

322          packet the attacker had to send.  Amplification factor 20/64 = 0.31."""

323          return (b"SWARA/1" + os.urandom(9)).ljust(self.HELLO_PAD, b"\x00")

324

325      def make_auth_init(self, retry: bytes) -> bytes:

326          epoch = retry[:4]

327          cookie = retry[4:20]

328          u = self.username.encode()

329          body = epoch + cookie + self.eph_pub + bytes([len(u)]) + u

330          self._init_msg = body

331          return body

332

333      def make_auth_confirm(self, resp: bytes) -> bytes | None:

334          off = 0

335          slen = resp[off]; off += 1

336          salt = resp[off:off + slen]; off += slen

337          elen, = struct.unpack("!H", resp[off:off + 2]); off += 2

338          envelope = resp[off:off + elen]; off += elen

339          server_eph = resp[off:off + 32]; off += 32

340          server_static = resp[off:off + 32]

341

342          rw = stretch_password(self.password, salt)

343          try:

344              opened = ChaCha20Poly1305(rw).decrypt(b"\x00" * 12, envelope, self.username.encode())

345          except Exception:

346              return None          # wrong password -- detected locally, no oracle

347          static_priv_raw = opened[:32]

348          static_priv = X25519PrivateKey.from_private_bytes(static_priv_raw)

349

350          dh1 = self.eph_priv.exchange(X25519PublicKey.from_public_bytes(server_static))

351          dh2 = static_priv.exchange(X25519PublicKey.from_public_bytes(server_eph))

352          dh3 = self.eph_priv.exchange(X25519PublicKey.from_public_bytes(server_eph))

353

354          th = Transcript().add(b"AUTH_INIT", self._init_msg, b"AUTH_RESP", resp).digest()

355          master = hkdf_extract(th, dh1 + dh2 + dh3)

356          self.keys = SessionKeys.derive(master, th)

357          return hmac.new(self.keys.k_c2s, b"client finished" + th, hashlib.sha256).digest()[:16]

358

359      def make_path_migrate(self) -> bytes:

360          assert self.keys and self.ticket

361          nonce = os.urandom(12)

362          sealed = ChaCha20Poly1305(self.keys.k_resume).encrypt(

363              nonce, struct.pack("!d", time.time()), b"SWARA-migrate")

364          return struct.pack("!H", len(self.ticket)) + self.ticket + nonce + sealed

365

366

367  # ---------------------------------------------------------------------------

368  # Media record protection

369  # ---------------------------------------------------------------------------

370

371  class MediaCipher:

372      """AEAD for media packets.

373

374      The 16-byte SWARA header is passed as ASSOCIATED DATA, not encrypted.  That

375      is what lets the PI octet stay readable to the network while remaining

376      unforgeable -- flipping it invalidates the tag and the packet is dropped.

377

378      The nonce is derived deterministically from (direction, sequence number), so

379      no nonce needs to be transmitted: 12 bytes saved on every single packet,

380      which at 50 packets/s is 4.8 kbit/s of pure saving per stream.

381      """

382

383      def __init__(self, key: bytes, direction: bytes):

384          self.aead = ChaCha20Poly1305(key)

385          self.dir = direction

386          self._salt = hashlib.sha256(key + direction).digest()[:12]

387

388      def _nonce(self, seq: int, epoch: int = 0) -> bytes:

389          counter = struct.pack("!IQ", epoch, seq)

390          return bytes(a ^ b for a, b in zip(self._salt, counter[:12]))

391

392      def seal(self, seq: int, header: bytes, plaintext: bytes) -> bytes:

393          return self.aead.encrypt(self._nonce(seq), plaintext, header)

394

395      def open(self, seq: int, header: bytes, ciphertext: bytes) -> bytes | None:

396          try:

397              return self.aead.decrypt(self._nonce(seq), ciphertext, header)

398          except Exception:

399              return None

400

401

402  # ---------------------------------------------------------------------------

403  # Sequence-number hygiene for long calls

404  # ---------------------------------------------------------------------------

405

406  class SequenceReconstructor:

407      """Recover a full counter from the 16-bit sequence field on the wire.

408

409      The header carries 16 bits, but the AEAD nonce is derived from the full

410      counter.  Without reconstruction the two diverge the moment the field wraps

411      -- after 65536 frames, which at 20 ms per frame is 21.8 minutes.  Every

412      packet after that point fails authentication and the call dies silently.

413      This is the same rollover-counter mechanism SRTP uses, and for the same

414      reason.

415      """

416

417      def __init__(self) -> None:

418          self.roc = 0

419          self.highest = -1

420

421      def unwrap(self, wire: int) -> int:

422          wire &= 0xFFFF

423          if self.highest < 0:

424              self.highest = wire

425              return wire

426          prev_low = self.highest & 0xFFFF

427          roc = self.roc

428          if wire < prev_low and (prev_low - wire) > 32768:

429              roc += 1                      # the field wrapped forward

430          elif wire > prev_low and (wire - prev_low) > 32768:

431              roc = max(0, roc - 1)         # a straggler from the previous epoch

432          full = (roc << 16) | wire

433          if full > self.highest:

434              self.highest = full

435              self.roc = roc

436          return full

437

438      def nearest(self, wire: int) -> int:

439          """Full counter for a 16-bit value near the current position, without

440          advancing state -- used for references such as an FEC group's base."""

441          wire &= 0xFFFF

442          if self.highest < 0:

443              return wire

444          base = (self.highest >> 16) << 16

445          cands = (base + wire, base + wire - 65536, base + wire + 65536)

446          return min(cands, key=lambda c: abs(c - self.highest))

447

448

449  class ReplayWindow:

450      """Sliding bitmap rejecting duplicated or replayed sequence numbers.

451

452      AEAD stops an attacker forging a packet, but it does not stop one CAPTURING

453      a valid packet and sending it again.  Without a replay window a recorded

454      fragment of speech can be re-injected into a live call.

455      """

456

457      def __init__(self, size: int = 1024):

458          self.size = size

459          self.highest = -1

460          self.bits = 0

461          self.rejected = 0

462

463      def check_and_set(self, seq: int) -> bool:

464          if seq > self.highest:

465              shift = seq - self.highest

466              self.bits = 1 if shift >= self.size else \

467                  (((self.bits << shift) | 1) & ((1 << self.size) - 1))

468              self.highest = seq

469              return True

470          delta = self.highest - seq

471          if delta >= self.size or (self.bits >> delta) & 1:

472              self.rejected += 1

473              return False

474          self.bits |= (1 << delta)

475          return True

Listing 5   swara/controller.py	281 lines

NAVIK: capacity estimation, burst-aware residual-loss model, joint optimisation and DSCP mapping.

1  """

2  NAVIK -- Network-Adaptive Voice-quality Integrated Kontroller.

3

4  The sender has three dials and one budget:

5

6      dial 1   codec bitrate b        higher b -> lower Ie -> better clean quality

7      dial 2   FEC redundancy rho     higher rho -> lower residual loss

8      dial 3   DSCP marking           which packets a congested router drops first

9      budget   available capacity C   b * (1 + rho) + header overhead <= C

10

11  Turning any dial up costs the others.  Conventional stacks resolve this with a

12  cascade of independent heuristics: a bandwidth estimator picks b, a separate

13  loss-driven module picks rho, and DSCP is a static configuration item.  Those

14  modules can and do fight each other -- the classic pathology is FEC added in

15  response to congestion loss, which increases the offered load and deepens the

16  congestion that caused the loss in the first place.

17

18  NAVIK replaces the cascade with a single objective.  Every 100 ms it evaluates

19

20      maximise  R( codec(b), delay(b, rho), Ppl_residual(rho, p, B, PI), B )

21      s.t.      b * (1 + rho) + H <= C_est

22                rho <= rho_max(p)          -- congestion safety rail

23

24  over a small discrete grid (9 bitrates x 9 redundancy levels = 81 evaluations,

25  about 40 microseconds).  Because R already contains the delay term, the

26  congestion pathology cannot occur: adding redundancy that pushes the offered

27  rate past C_est raises queueing delay, which lowers R, so the optimiser refuses.

28

29  The residual-loss model is where perceptual importance re-enters.  For an

30  RS(k+r, k) group under Gilbert-Elliott loss, the probability that the group

31  fails to decode is the probability of more than r erasures in k+r packets --

32  computed here with a burst-aware correction rather than the binomial, because

33  under clustered loss the binomial is dramatically optimistic.  PI-AFEC's

34  advantage appears in the model as a reduction in the *perceptual weight* of the

35  groups that are left unprotected.

36  """

37

38  from __future__ import annotations

39

40  import math

41  import numpy as np

42  from dataclasses import dataclass, field

43

44  from . import emodel

45

46  BITRATE_LADDER = ["opus-6k", "opus-8k", "opus-12k", "opus-16k",

47                    "opus-20k", "opus-24k", "opus-32k"]

48  RHO_GRID = [0.0, 0.075, 0.15, 0.225, 0.30, 0.375, 0.45, 0.55, 0.65]

49

50  HEADER_OVERHEAD_BPS = (16 + 16 + 8 + 20) * 8 * 50   # SWARA+AEAD+UDP+IP at 50 pps

51

52

53  @dataclass

54  class ControlDecision:

55      codec: str

56      bitrate_kbps: float

57      rho: float

58      dscp_high: int

59      dscp_low: int

60      predicted_r: float

61      predicted_mos: float

62      offered_kbps: float

63      reason: str = ""

64

65      def __str__(self) -> str:

66          return (f"NAVIK -> {self.codec} @ {self.bitrate_kbps:.0f}kbps, "

67                  f"FEC {self.rho*100:.0f}%, offered {self.offered_kbps:.1f}kbps, "

68                  f"predicted MOS {self.predicted_mos:.2f}")

69

70

71  def group_failure_probability(k: int, r: int, p: float, mean_burst: float) -> float:

72      """P(an RS(k+r, k) group cannot be decoded) under Gilbert-Elliott loss.

73

74      Under independent loss this is P(erasures > r) ~ Binomial.  Under clustered

75      loss the variance of the erasure count is much higher, so we use a

76      beta-binomial with the over-dispersion implied by the burst length.  This

77      matters enormously: at 10% loss with burst 4, the binomial predicts a group

78      failure rate about 6x lower than reality, and a controller that believes it

79      will under-provision FEC exactly when protection is most needed.

80      """

81      n = k + r

82      p = float(np.clip(p, 1e-6, 0.95))

83      if r >= n:

84          return 0.0

85      b = max(1.0, mean_burst)

86

87      if b <= 1.05:

88          # Effectively independent -> plain binomial tail.

89          from math import comb

90          return float(sum(comb(n, i) * p ** i * (1 - p) ** (n - i)

91                           for i in range(r + 1, n + 1)))

92

93      # Beta-binomial: intra-class correlation rho_c grows with burst length.

94      rho_c = float(np.clip((b - 1.0) / (b + 1.0), 1e-3, 0.95))

95      s = (1.0 - rho_c) / rho_c

96      a, bb = p * s, (1.0 - p) * s

97

98      from math import lgamma

99

100      def logbeta(x, y):

101          return lgamma(x) + lgamma(y) - lgamma(x + y)

102

103      from math import comb

104      lb0 = logbeta(a, bb)

105      tail = 0.0

106      for i in range(r + 1, n + 1):

107          logp = (math.log(comb(n, i)) + logbeta(a + i, bb + n - i) - lb0)

108          tail += math.exp(logp)

109      return float(np.clip(tail, 0.0, 1.0))

110

111

112  def split_budget(budget_pkts: float, k: int, mean_burst: float) -> tuple[int, int]:

113      """Divide a group's redundancy budget between RS parity and anchor replicas.

114

115      Mirrors fec.FecScheduler exactly, so the controller predicts what the

116      scheduler will actually do rather than what it wishes it would do.

117      """

118      # Mirrors fec.FecScheduler after normalisation: the EXPECTED per-group

119      # spend is the full budget, so the controller predicts against the same

120      # quantity of redundancy the scheduler will actually emit.

121      if mean_burst >= 0.75 * k:

122          sr, sa = 0.45, 0.55

123      else:

124          sr, sa = 0.85, 0.15

125      return (int(round(budget_pkts * sr)), int(np.clip(round(budget_pkts * sa), 0, 4)))

126

127

128  def residual_loss(p: float, mean_burst: float, k: int, r: int,

129                    n_anchor: int = 0, pi_efficiency: float = 1.0) -> float:

130      """Perceptually weighted residual loss after FEC and anchor replicas.

131

132      The two mechanisms are INDEPENDENT and must be modelled as such.  Reed-

133      Solomon is a group-level mechanism: it either reconstructs the whole group

134      or nothing.  An anchor replica is a frame-level mechanism that works even

135      when the group is a total loss, because the replica sits outside the burst

136      that destroyed the group.  Treating anchors as a modifier on RS -- as an

137      earlier version of this model did -- makes the controller believe

138      protection is futile in exactly the burst-dominated conditions where the

139      anchor is the only thing that still works.

140

141      `pi_efficiency` in (0, 1] is the PI-AFEC advantage: because redundancy is

142      concentrated on the groups that carry meaning, the perceptual weight of the

143      residual loss is a fraction of its raw packet count.  Setting it to 1.0

144      reproduces uniform FEC exactly, which is how the comparison stays honest.

145      """

146      if r <= 0:

147          base = p

148      else:

149          pf = group_failure_probability(k, r, p, mean_burst)

150          base = pf * p

151

152      if n_anchor > 0:

153          # Each replica is displaced past the estimated burst, so it survives

154          # unless an INDEPENDENT loss event hits it.  Covering m of k frames

155          # removes at most m/k of the residual, weighted by replica survival.

156          survive = 1.0 - p

157          covered = min(1.0, n_anchor / k) * survive

158          base *= (1.0 - covered)

159      return float(np.clip(base * pi_efficiency, 0.0, 1.0))

160

161

162  class NavikController:

163      """The joint optimiser.  One call per control epoch (default 100 ms)."""

164

165      def __init__(self, k: int = 8, mobile: bool = False,

166                   pi_afec: bool = True, safety_margin: float = 0.85,

167                   pi_efficiency: float | None = None):

168          # `pi_efficiency` is how much cheaper perceptual placement makes a given

169          # residual loss LOOK to the optimiser.  The offline value of 0.42 is a

170          # perceptual figure and is correct there, because the offline objective

171          # is itself perceptually weighted.  Feeding it to a live controller that

172          # is judged on RAW frame loss makes it believe its redundancy is 2.4x

173          # more effective than it is, so it provisions roughly a quarter of what

174          # a uniform scheme buys and loses on both counts.  Live sessions pass a

175          # conservative value and keep the perceptual PLACEMENT without claiming

176          # the perceptual DISCOUNT up front.

177          self.pi_efficiency_override = pi_efficiency

178          self.k = k

179          self.mobile = mobile

180          self.pi_afec = pi_afec

181          self.safety = safety_margin

182          self.capacity_est_kbps = 200.0

183          self._cap_ewma = 200.0

184          self.history: list[ControlDecision] = []

185

186      def update_capacity(self, goodput_kbps: float, loss: float,

187                          queue_growing: bool, delay_trend_ms_per_s: float = 0.0) -> None:

188          """Capacity estimate with congestion distinguished from plain loss.

189

190          Naive AIMD backs off on ANY loss.  That is correct on a wired path where

191          loss means a full queue, and badly wrong on a wireless one where loss

192          means a fading radio link.  Backing off there reduces quality without

193          relieving anything, and -- worse for this system -- it starves the very

194          redundancy budget that the loss made necessary.

195

196          Congestion has a signature that random loss does not: the queue fills

197          before it overflows, so ONE-WAY DELAY RISES first.  We therefore treat

198          loss as congestion only when it is accompanied by a rising delay trend,

199          which is the same discriminator used by modern real-time congestion

200          controllers.  Loss with flat delay is read as link loss, and we hold the

201          rate and spend the budget on protection instead.

202          """

203          congested = queue_growing or delay_trend_ms_per_s > 8.0

204          if congested:

205              self._cap_ewma *= 0.88

206          elif loss > 0.02:

207              self._cap_ewma *= 0.995          # gentle probe down, not a collapse

208          else:

209              self._cap_ewma = self._cap_ewma * 0.97 + (goodput_kbps * 1.25) * 0.03

210          self._cap_ewma = float(np.clip(self._cap_ewma, 24.0, 2000.0))

211          self.capacity_est_kbps = self._cap_ewma

212

213      def decide(self, loss: float, mean_burst: float, jitter_ms: float,

214                 owd_ms: float, jb_depth_ms: float,

215                 pi_efficiency: float | None = None) -> ControlDecision:

216          if pi_efficiency is None:

217              pi_efficiency = self.pi_efficiency_override

218          if pi_efficiency is None:

219              # Measured empirically in the offline evaluation; see experiments/.

220              pi_efficiency = 0.42 if self.pi_afec else 1.0

221

222          budget = self.capacity_est_kbps * self.safety

223          best: ControlDecision | None = None

224

225          # Congestion safety rail: never let redundancy exceed what the path can

226          # absorb, and taper it as loss rises since high loss usually IS congestion.

227          rho_cap = 0.65 if loss < 0.03 else max(0.15, 0.65 - 1.8 * (loss - 0.03))

228

229          for codec in BITRATE_LADDER:

230              ie, bpl, kbps = emodel.CODEC_PARAMS[codec]

231              for rho in RHO_GRID:

232                  if rho > rho_cap:

233                      continue

234                  offered = kbps * (1.0 + rho) + HEADER_OVERHEAD_BPS / 1000.0

235                  if offered > budget:

236                      continue

237                  group_budget = self.k * rho

238                  if self.pi_afec:

239                      r, n_anchor = split_budget(group_budget, self.k, mean_burst)

240                  else:

241                      r, n_anchor = int(round(group_budget)), 0

242                  p_res = residual_loss(loss, mean_burst, self.k, r,

243                                        n_anchor=n_anchor, pi_efficiency=pi_efficiency)

244                  # A systematic code adds no mouth-to-ear delay of its own; the

245                  # data packets leave immediately.  Only recovery waits.

246                  total_delay = owd_ms + jb_depth_ms + 25.0

247                  r_pred = emodel.r_factor(codec, total_delay, p_res * 100.0,

248                                           mean_burst, self.mobile)

249                  if best is None or r_pred > best.predicted_r:

250                      best = ControlDecision(

251                          codec=codec, bitrate_kbps=kbps, rho=rho,

252                          dscp_high=46, dscp_low=(34 if rho > 0 else 0),

253                          predicted_r=r_pred, predicted_mos=emodel.r_to_mos(r_pred),

254                          offered_kbps=offered)

255

256          if best is None:

257              best = ControlDecision("opus-6k", 6.0, 0.0, 46, 0, 0.0, 1.0,

258                                     6.0 + HEADER_OVERHEAD_BPS / 1000.0,

259                                     reason="capacity below floor")

260          best.reason = (f"budget {budget:.0f}kbps, loss {loss*100:.1f}%, "

261                         f"burst {mean_burst:.1f}")

262          self.history.append(best)

263          return best

264

265

266  def dscp_for(pi: int, decision: ControlDecision) -> int:

267      """Per-packet DiffServ marking driven by perceptual importance.

268

269      This is the cross-layer claim in its most direct form.  A router applying

270      WRED to a congested queue drops from the lowest-precedence class first.  By

271      marking silence and low-energy frames AF41 (or best-effort) and voiced

272      onsets EF, SWARA arranges for the network's own drop decision to be

273      perceptually informed -- WITHOUT the router understanding speech, and

274      without the router being able to read the speech, since the PI octet is

275      authenticated associated data rather than plaintext content.

276      """

277      if pi >= 160:

278          return decision.dscp_high      # EF (46)

279      if pi >= 80:

280          return decision.dscp_low or 34  # AF41

281      return 0                            # best effort

Listing 6   swara/receiver.py	233 lines

Network-state estimation, the R-factor-optimal jitter buffer and group reassembly.

1  """

2  Receiver side: network state estimation, FEC recovery, and the adaptive

3  playout buffer.

4

5  The jitter buffer is where SWARA departs most sharply from convention.

6

7  A conventional adaptive jitter buffer targets a LOSS objective: "size the

8  buffer so that at most 1% of packets arrive too late".  That is the wrong

9  objective, because it treats the delay it adds as free.  It is not free -- the

10  E-model's Id term charges 0.024 R-points per millisecond, and above 177 ms it

11  charges 0.134 per millisecond.

12

13  SWARA sizes the buffer by directly maximising the predicted R-factor:

14

15      d* = argmax_d  R( codec, delay_base + d, Ppl(d), burst )

16

17  where Ppl(d) is the perceptually weighted residual loss that would result from

18  a playout deadline of d, computed from the measured arrival-delay histogram.

19  Deepening the buffer moves packets from "late" to "on time", lowering Ppl; it

20  simultaneously raises Id.  The optimum is the point where the marginal R gained

21  from rescued packets equals the marginal R lost to delay -- and because Ppl is

22  perceptually weighted, the optimum shifts DEEPER when the packets arriving late

23  happen to be important ones, and shallower when they are silence.

24  """

25

26  from __future__ import annotations

27

28  import math

29  import numpy as np

30  from collections import deque

31  from dataclasses import dataclass, field

32

33  from . import emodel

34  from .perceptual import damage_weight

35

36

37  # ---------------------------------------------------------------------------

38  # Network state

39  # ---------------------------------------------------------------------------

40

41  @dataclass

42  class NetworkState:

43      """Everything the controller needs, estimated purely from arrivals."""

44      loss_rate: float = 0.0          # fraction of expected packets never seen

45      mean_burst: float = 1.0         # mean consecutive-loss run length

46      jitter_ms: float = 0.0          # RFC 3550 interarrival jitter estimate

47      delay_spread_ms: float = 0.0    # p95 - p5 of the relative delay histogram

48      rtt_ms: float = 0.0

49      reorder_rate: float = 0.0

50      goodput_kbps: float = 0.0

51      packets_seen: int = 0

52

53      def summary(self) -> str:

54          return (f"loss {self.loss_rate*100:5.2f}%  burst {self.mean_burst:4.2f}  "

55                  f"jitter {self.jitter_ms:5.1f}ms  spread {self.delay_spread_ms:5.1f}ms")

56

57

58  class NetworkStateEstimator:

59      """Streaming estimator over a sliding window of arrivals.

60

61      All the statistics come from the media stream itself.  There are no probe

62      packets and no separate RTCP-style reports on the wire: the 16-byte

63      telemetry block rides in the header of the reverse-direction voice packets,

64      which are being sent anyway.  A duplex call therefore pays ZERO extra

65      packets for full closed-loop measurement.

66      """

67

68      def __init__(self, window: int = 250, frame_ms: int = 20):

69          self.window = window

70          self.frame_ms = frame_ms

71          self._delays: deque[float] = deque(maxlen=window)

72          self._arrivals: deque[tuple[int, float]] = deque(maxlen=window)

73          self._bytes: deque[tuple[float, int]] = deque(maxlen=window)

74          self._jitter = 0.0

75          self._last_transit: float | None = None

76          self._highest_seq = -1

77          self._received: set[int] = set()

78          self._loss_runs: list[int] = []

79          self._cur_run = 0

80          self._reorders = 0

81          self._total = 0

82

83      def on_arrival(self, seq: int, send_ts_ms: float, recv_ts_ms: float,

84                     size_bytes: int) -> None:

85          transit = recv_ts_ms - send_ts_ms

86          self._delays.append(transit)

87          self._arrivals.append((seq, recv_ts_ms))

88          self._bytes.append((recv_ts_ms, size_bytes))

89          self._total += 1

90

91          # RFC 3550 interarrival jitter: exponentially smoothed |D(i-1, i)|.

92          if self._last_transit is not None:

93              d = abs(transit - self._last_transit)

94              self._jitter += (d - self._jitter) / 16.0

95          self._last_transit = transit

96

97          if seq < self._highest_seq:

98              self._reorders += 1

99          else:

100              gap = seq - self._highest_seq - 1 if self._highest_seq >= 0 else 0

101              if gap > 0:

102                  self._loss_runs.append(gap)

103              self._highest_seq = seq

104          self._received.add(seq)

105          if len(self._received) > self.window * 3:

106              cutoff = self._highest_seq - self.window * 2

107              self._received = {s for s in self._received if s >= cutoff}

108

109      def snapshot(self, rtt_ms: float = 0.0) -> NetworkState:

110          expected = max(1, self._highest_seq + 1)

111          seen = min(self._total, expected)

112          loss = 1.0 - seen / expected if expected else 0.0

113          runs = self._loss_runs[-80:]

114          burst = float(np.mean(runs)) if runs else 1.0

115

116          if len(self._delays) >= 8:

117              arr = np.fromiter(self._delays, dtype=float)

118              spread = float(np.percentile(arr, 95) - np.percentile(arr, 5))

119          else:

120              spread = 0.0

121

122          if len(self._bytes) >= 2:

123              t0, t1 = self._bytes[0][0], self._bytes[-1][0]

124              span = max(1e-3, (t1 - t0) / 1000.0)

125              total_b = sum(b for _, b in self._bytes)

126              goodput = total_b * 8.0 / span / 1000.0

127          else:

128              goodput = 0.0

129

130          return NetworkState(

131              loss_rate=float(np.clip(loss, 0.0, 1.0)),

132              mean_burst=max(1.0, burst),

133              jitter_ms=self._jitter,

134              delay_spread_ms=spread,

135              rtt_ms=rtt_ms,

136              reorder_rate=self._reorders / max(1, self._total),

137              goodput_kbps=goodput,

138              packets_seen=self._total,

139          )

140

141      def relative_delays(self) -> np.ndarray:

142          """Arrival delays relative to the minimum observed -- the histogram the

143          jitter buffer optimiser needs."""

144          if not self._delays:

145              return np.zeros(0)

146          arr = np.fromiter(self._delays, dtype=float)

147          return arr - arr.min()

148

149

150  # ---------------------------------------------------------------------------

151  # R-factor-optimal adaptive jitter buffer

152  # ---------------------------------------------------------------------------

153

154  @dataclass

155  class JitterBufferDecision:

156      depth_ms: float

157      predicted_r: float

158      predicted_mos: float

159      late_loss_pct: float

160      considered: list[tuple[float, float]] = field(default_factory=list)

161

162

163  class AdaptiveJitterBuffer:

164      """Playout buffer whose depth is chosen to maximise predicted R-factor.

165

166      `importance_of_late` lets the optimiser know whether the packets that are

167      currently arriving late are perceptually valuable.  When they are, the

168      optimiser accepts more delay to rescue them; when they are silence, it

169      keeps the buffer shallow and the conversation responsive.

170      """

171

172      DEPTH_GRID_MS = (0, 10, 20, 30, 40, 60, 80, 100, 120, 150, 180)

173

174      def __init__(self, base_delay_ms: float = 40.0, frame_ms: int = 20,

175                   min_depth_ms: float = 20.0, max_depth_ms: float = 180.0):

176          self.base_delay_ms = base_delay_ms

177          self.frame_ms = frame_ms

178          self.min_depth = min_depth_ms

179          self.max_depth = max_depth_ms

180          self.depth_ms = 60.0

181          self._smoothed = 60.0

182

183      def optimise(self, rel_delays: np.ndarray, codec: str, mean_burst: float,

184                   other_delay_ms: float, late_importance: float = 0.5,

185                   mobile: bool = False) -> JitterBufferDecision:

186          if rel_delays.size < 8:

187              return JitterBufferDecision(self.depth_ms, 0.0, 0.0, 0.0)

188

189          considered = []

190          best = (-1.0, self.depth_ms, 0.0)

191          for d in self.DEPTH_GRID_MS:

192              if not (self.min_depth <= d <= self.max_depth):

193                  continue

194              late_frac = float(np.mean(rel_delays > d))

195              # Perceptually weight the late packets: a late silence frame costs

196              # almost nothing, a late voiced onset costs more than one packet.

197              eff_loss_pct = 100.0 * late_frac * (0.25 + 1.5 * late_importance)

198              total_delay = other_delay_ms + d

199              r = emodel.r_factor(codec, total_delay, eff_loss_pct, mean_burst, mobile)

200              considered.append((float(d), r))

201              if r > best[0]:

202                  best = (r, float(d), late_frac)

203

204          r_best, d_best, late_frac = best

205          # Hysteresis: buffer depth changes are audible, so move gently.

206          self._smoothed += 0.35 * (d_best - self._smoothed)

207          self.depth_ms = float(np.clip(self._smoothed, self.min_depth, self.max_depth))

208          return JitterBufferDecision(

209              depth_ms=self.depth_ms, predicted_r=r_best,

210              predicted_mos=emodel.r_to_mos(r_best),

211              late_loss_pct=100.0 * late_frac, considered=considered)

212

213

214  # ---------------------------------------------------------------------------

215  # FEC-aware reassembly

216  # ---------------------------------------------------------------------------

217

218  @dataclass

219  class GroupBuffer:

220      """Collects the packets of one FEC group until it can be decoded."""

221      group_id: int

222      k: int

223      r: int

224      shards: dict = field(default_factory=dict)

225      pi: dict = field(default_factory=dict)

226      first_seen_ms: float = 0.0

227      resolved: bool = False

228

229      def complete(self) -> bool:

230          return len([i for i in self.shards if i < self.k]) == self.k

231

232      def decodable(self) -> bool:

233          return len(self.shards) >= self.k

Listing 7   swara/session.py	607 lines

The live call pipeline: sender, receiver, playout and control threads.

1  """

2  The live media session.

3

4  This is where the whole system actually runs.  Three threads per call:

5

6    SENDER    capture -> encode -> perceptual analysis -> FEC scheduling ->

7              replica piggyback -> AEAD seal -> UDP sendto

8    RECEIVER  UDP recvfrom -> AEAD open -> replica extraction -> insert into the

9              reassembly buffer -> telemetry accounting

10    PLAYOUT   every 20 ms: take the next frame in sequence, recovering it from

11              parity or from a replica if the original never arrived, conceal it

12              if nothing can be recovered, decode, write to the speaker

13

14  The sender and playout threads are paced independently against a monotonic

15  clock rather than by sleeping a fixed interval, so accumulated scheduling error

16  does not cause the call to drift.

17

18  Media plaintext layout (before AEAD sealing):

19

20      +--------+----------------------------------------+----------------+

21      | nrep 1B| nrep x { seq 2B, len 2B, payload ... }  | primary frame  |

22      +--------+----------------------------------------+----------------+

23

24  The 16-byte SWARA header is supplied as associated data, so the perceptual

25  importance octet is readable on the wire but cannot be altered.

26  """

27

28  from __future__ import annotations

29

30  import struct

31  import threading

32  import time

33  import numpy as np

34  from collections import defaultdict, deque

35

36  from . import emodel

37  from .auth import MediaCipher, ReplayWindow, SequenceReconstructor

38  from .codec import AdpcmCodec, Mode, conceal, frame_samples

39  from .controller import NavikController, dscp_for

40  from .fec import FecScheduler, ReedSolomon

41  from .packet import AEAD_TAG_LEN, HEADER_LEN, Datagram, Flag, Header, PType

42  from .perceptual import PerceptualImportanceAnalyzer, damage_weight

43  from .receiver import AdaptiveJitterBuffer, NetworkStateEstimator

44

45  FRAME_MS = 20

46  GROUP_K = 4     # see _min_depth_for_fec: parity must beat its own deadline

47  OUT_LEN = 320                      # 20 ms @ 16 kHz

48

49

50  def pack_telemetry(loss: float, burst: float, jitter_ms: float, mos: float) -> bytes:

51      """16-byte report, piggybacked on reverse-direction media -- costs no packet."""

52      return struct.pack("!HHHHQ",

53                         int(min(65535, loss * 10000)),

54                         int(min(65535, burst * 100)),

55                         int(min(65535, jitter_ms * 100)),

56                         int(min(65535, mos * 1000)), 0)

57

58

59  def unpack_telemetry(b: bytes):

60      if len(b) < 16:

61          return None

62      loss, burst, jit, mos, _ = struct.unpack("!HHHHQ", b[:16])

63      return {"loss": loss / 10000.0, "burst": burst / 100.0,

64              "jitter_ms": jit / 100.0, "mos": mos / 1000.0}

65

66

67  class CallStats:

68      def __init__(self) -> None:

69          self.lock = threading.Lock()

70          self.tx_frames = 0

71          self.tx_parity = 0

72          self.tx_replicas = 0

73          self.tx_bytes = 0

74          self.rx_frames = 0

75          self.rx_late = 0

76          self.rx_auth_fail = 0

77          self.played = 0

78          self.recovered_fec = 0

79          self.recovered_replica = 0

80          self.concealed = 0

81          self.pi_sum = 0

82          self.lost_pi_sum = 0

83          self.peer_mos = 0.0

84

85      def snapshot(self) -> dict:

86          with self.lock:

87              d = dict(self.__dict__)

88          d.pop("lock", None)

89          played = max(1, self.played)

90          d["loss_after_repair_pct"] = 100.0 * self.concealed / played

91          d["recovery_pct"] = 100.0 * (self.recovered_fec + self.recovered_replica) / played

92          return d

93

94

95  class CallSession:

96      """One end of one call.  Owns the codec, the protection state and the keys."""

97

98      def __init__(self, endpoint, peer_addr, tx_key: bytes, rx_key: bytes,

99                   call_id: int, source, sink, k: int = GROUP_K,

100                   mode: int = Mode.WIDEBAND, adaptive: bool = True,

101                   protection: str = "swara", name: str = "peer"):

102          self.ep = endpoint

103          self.peer = peer_addr

104          self.call_id = call_id

105          self.name = name

106          self.source = source

107          self.sink = sink

108          self.k = k

109          self.mode = mode

110          self.adaptive = adaptive

111          self.protection = protection          # "swara" | "uniform" | "none"

112

113          self.tx = MediaCipher(tx_key, b"tx")

114          self.rx = MediaCipher(rx_key, b"tx")   # peer's tx direction

115

116          self.codec = AdpcmCodec(mode)

117          self.analyzer = PerceptualImportanceAnalyzer()

118          self.sched = FecScheduler(k=k, rho_net=0.0, mean_burst=1.0)

119          # Provisioning is deliberately IDENTICAL for every protection scheme.

120          #

121          # Letting the controller anticipate the perceptual benefit made it buy

122          # less redundancy than the uniform comparator -- 83 parity packets

123          # against 373 -- so SWARA was being compared at a fifth of the budget

124          # and lost on raw frame loss as a direct result.  That is not the

125          # experiment this project claims to run.  The claim is "the same budget,

126          # spent where it is heard", so the budget is sized the same way for all

127          # arms and only the PLACEMENT differs.

128          self.navik = NavikController(k=k, pi_afec=False, pi_efficiency=1.0)

129          self.navik.capacity_est_kbps = self.navik._cap_ewma = 400.0

130          self.jb = AdaptiveJitterBuffer(base_delay_ms=0.0)

131          self.est = NetworkStateEstimator()

132          self.stats = CallStats()

133

134          # Start at a depth the protection scheme can actually use.  Starting

135          # shallow and deepening later costs a stall the listener can hear.

136          self.depth_ms = 60.0 if protection == "none" else float((k + 3) * FRAME_MS)

137          self.rho = 0.0

138

139          self._seq = 0

140          self._group_id = 0

141          self._pending: list[tuple[int, bytes, int]] = []   # (seq, payload, pi)

142          self._replica_queue: dict[int, list[tuple[int, bytes]]] = defaultdict(list)

143

144          self._rx_payload: dict[int, bytes] = {}

145          self._rx_parity: dict[int, dict[int, bytes]] = defaultdict(dict)

146          self._rx_group_r: dict[int, int] = {}

147          self._rx_lock = threading.Lock()

148          self._roc = SequenceReconstructor()

149          self._replay = ReplayWindow()

150          self._first_seq: int | None = None

151          self._play_seq: int | None = None

152          self._last_pcm: np.ndarray | None = None

153          self._conceal_run = 0

154          self._epoch = 0.0

155          self._epoch_seq = 0

156          self._spread_ms = 0.0        # p95 - min of observed one-way delay

157          self._delays: deque = deque(maxlen=300)

158          # Perceptual accounting.  A raw count of concealed frames is the wrong

159          # scoreboard for this system: PI-AFEC deliberately declines to protect

160          # silence, so it will always concede raw frames to a scheme that

161          # protects everything equally.  What it claims to reduce is the

162          # perceptual DAMAGE, so that is what has to be measured.

163          self.sent_pi: dict[int, int] = {}

164          self.concealed_seqs: list[int] = []

165

166          self._running = threading.Event()

167          self._threads: list[threading.Thread] = []

168          self.t0 = time.monotonic()

169          self.on_frame_sent = None            # optional UI callback

170          self.finished = threading.Event()

171

172      # ------------------------------------------------------------------

173      def _now_ms(self) -> float:

174          return (time.monotonic() - self.t0) * 1000.0

175

176      def start(self) -> None:

177          self._running.set()

178          for fn, nm in ((self._sender_loop, "tx"), (self._receiver_loop, "rx"),

179                         (self._playout_loop, "play"), (self._control_loop, "ctl")):

180              t = threading.Thread(target=fn, name=f"{self.name}-{nm}", daemon=True)

181              t.start()

182              self._threads.append(t)

183

184      def stop(self) -> None:

185          self._running.clear()

186          for t in self._threads:

187              t.join(timeout=2.0)

188

189      # ------------------------------------------------------------------

190      # SENDER

191      # ------------------------------------------------------------------

192      def _sender_loop(self) -> None:

193          next_tick = time.monotonic()

194          while self._running.is_set():

195              pcm = self.source.read(timeout=0.5)

196              if pcm is None:

197                  break

198              payload = self.codec.encode(pcm)

199              pi = self.analyzer.analyze(pcm.astype(np.float64) / 32768.0).pi

200              self._pending.append((self._seq, payload, pi))

201              self.sent_pi[self._seq] = pi

202              with self.stats.lock:

203                  self.stats.pi_sum += pi

204

205              self._emit_media(self._seq, payload, pi)

206              self._seq += 1

207

208              if len(self._pending) >= self.k:

209                  self._close_group()

210

211              next_tick += FRAME_MS / 1000.0

212              sleep = next_tick - time.monotonic()

213              if sleep > 0:

214                  time.sleep(sleep)

215              else:

216                  next_tick = time.monotonic()      # we fell behind; resynchronise

217          self._running.clear()

218          self.finished.set()

219

220      def _emit_media(self, seq: int, payload: bytes, pi: int) -> None:

221          reps = self._replica_queue.pop(seq, [])

222          body = bytearray([len(reps)])

223          for rseq, rpay in reps:

224              # A displacement survives sequence wrap; an absolute 16-bit value

225              # does not.  The receiver reconstructs the source from its own

226              # full counter for the carrier packet.

227              body += struct.pack("!BH", min(255, seq - rseq), len(rpay)) + rpay

228          body += payload

229

230          h = Header(ptype=PType.MEDIA, seq=seq & 0xFFFF, cid=self.call_id,

231                     ts_ms=int(self._now_ms()), pi=pi,

232                     fec_group=self._group_id & 0xFF, fec_index=seq % self.k,

233                     flags=Flag.ENCRYPTED)

234          tel = pack_telemetry(*self._telemetry_values())

235          h.flags |= Flag.TELEMETRY_PIGGYBACK

236          hdr = h.pack()

237          ct = self.tx.seal(seq, hdr + tel, bytes(body))

238          self.ep.set_dscp(dscp_for(pi, self.navik.history[-1]) if self.navik.history else 0)

239          self.ep.sendto(hdr + tel + ct, self.peer)

240          with self.stats.lock:

241              self.stats.tx_frames += 1

242              self.stats.tx_bytes += len(hdr) + len(tel) + len(ct)

243              self.stats.tx_replicas += len(reps)

244

245      def _close_group(self) -> None:

246          group = self._pending[: self.k]

247          self._pending = self._pending[self.k:]

248          gid = self._group_id

249          self._group_id = (self._group_id + 1) & 0xFF

250

251          if self.protection == "none":

252              return

253          pis = [p for _, _, p in group]

254          plan = self.sched.plan(pis) if self.protection == "swara" else None

255          if self.protection == "uniform":

256              r = int(round(self.k * self.rho))

257              anchors = []

258          else:

259              r, anchors = plan.r, plan.anchors

260

261          # Replica displacement.  The copy must clear the loss burst AND still

262          # beat its own frame's playout instant.  The usable budget is not the

263          # whole buffer depth: the copy is itself subject to the same delay

264          # variation as everything else, so the p95 delay spread has to be

265          # subtracted first.  Ignoring that term places the replica about one

266          # frame too late and it is discarded on arrival -- which is precisely

267          # what happened before this was corrected, with only 7% of transmitted

268          # replicas ever being used.

269          usable_ms = self.depth_ms - self._spread_ms - FRAME_MS

270          max_disp = max(1, int(usable_ms // FRAME_MS))

271          for a_idx, a_delay in anchors:

272              src_seq, src_pay, _ = group[a_idx]

273              disp = min(a_delay, max_disp)

274              if disp >= 1:

275                  self._replica_queue[src_seq + disp].append((src_seq, src_pay))

276

277          if r <= 0:

278              return

279          shard_len = max(len(p) for _, p, _ in group)

280          shards = [p.ljust(shard_len, b"\x00") for _, p, _ in group]

281          try:

282              parity = ReedSolomon(self.k, r).encode(shards)

283          except Exception:

284              return

285          base_seq = group[0][0]

286          for j, par in enumerate(parity):

287              h = Header(ptype=PType.FEC, seq=(base_seq + self.k + j) & 0xFFFF,

288                         cid=self.call_id, ts_ms=int(self._now_ms()), pi=0,

289                         fec_group=gid, fec_index=0x80 | j, flags=Flag.ENCRYPTED)

290              hdr = h.pack()

291              body = struct.pack("!HBBH", base_seq & 0xFFFF, self.k, r, shard_len) + par

292              ct = self.tx.seal(h.seq, hdr, body)

293              self.ep.sendto(hdr + ct, self.peer)

294              with self.stats.lock:

295                  self.stats.tx_parity += 1

296                  self.stats.tx_bytes += len(hdr) + len(ct)

297

298      # ------------------------------------------------------------------

299      # RECEIVER

300      # ------------------------------------------------------------------

301      def _receiver_loop(self) -> None:

302          while self._running.is_set():

303              data, addr = self.ep.recvfrom(timeout=0.4)

304              if data is None or len(data) < HEADER_LEN:

305                  continue

306              try:

307                  h = Header.unpack(data)

308              except Exception:

309                  continue

310              if h.cid != self.call_id:

311                  continue

312              off = HEADER_LEN

313              tel = b""

314              if h.flags & Flag.TELEMETRY_PIGGYBACK:

315                  tel = data[off:off + 16]; off += 16

316              aad = data[:off]

317              ct = data[off:]

318

319              full_seq = self._roc.unwrap(h.seq)

320              pt = self.rx.open(full_seq, aad, ct)

321              if pt is None:

322                  with self.stats.lock:

323                      self.stats.rx_auth_fail += 1

324                  continue

325              if h.ptype == PType.MEDIA and not self._replay.check_and_set(full_seq):

326                  continue          # duplicate or replayed packet

327

328              if tel:

329                  t = unpack_telemetry(tel)

330                  if t:

331                      with self.stats.lock:

332                          self.stats.peer_mos = t["mos"]

333

334              if h.ptype == PType.MEDIA:

335                  self._on_media(h, pt, len(data), full_seq)

336              elif h.ptype == PType.FEC:

337                  self._on_parity(h, pt)

338

339      def _on_media(self, h: Header, pt: bytes, wire_len: int, full_seq: int) -> None:

340          nrep = pt[0]

341          off = 1

342          reps = []

343          for _ in range(nrep):

344              if off + 3 > len(pt):

345                  break

346              disp, rlen = struct.unpack("!BH", pt[off:off + 3]); off += 3

347              reps.append((full_seq - disp, pt[off:off + rlen])); off += rlen

348          primary = pt[off:]

349

350          recv_ms = self._now_ms()

351          self._delays.append(recv_ms - h.ts_ms)

352          self.est.on_arrival(full_seq, h.ts_ms, recv_ms, wire_len)

353

354          with self._rx_lock:

355              if self._first_seq is None:

356                  self._first_seq = full_seq

357                  self._play_seq = full_seq

358              if full_seq not in self._rx_payload:

359                  self._rx_payload[full_seq] = primary

360                  with self.stats.lock:

361                      self.stats.rx_frames += 1

362              for rseq, rpay in reps:

363                  if rseq not in self._rx_payload:

364                      self._rx_payload[rseq] = rpay

365                      with self.stats.lock:

366                          self.stats.recovered_replica += 1

367

368      def _on_parity(self, h: Header, pt: bytes) -> None:

369          if len(pt) < 6:

370              return

371          base_seq, k, r, shard_len = struct.unpack("!HBBH", pt[:6])

372          base_full = self._roc.nearest(base_seq)

373          par = pt[6:]

374          j = h.fec_index & 0x7F

375          with self._rx_lock:

376              self._rx_parity[h.fec_group][j] = par

377              self._rx_group_r[h.fec_group] = (base_full, k, r, shard_len)

378

379      def _try_fec(self, gid: int) -> bool:

380          """Attempt to reconstruct a group.  Returns True if anything recovered."""

381          meta = self._rx_group_r.get(gid)

382          if not meta:

383              return False

384          base_seq, k, r, shard_len = meta

385          have = {}

386          missing = []

387          for i in range(k):

388              s = base_seq + i

389              if s in self._rx_payload:

390                  have[i] = self._rx_payload[s].ljust(shard_len, b"\x00")

391              else:

392                  missing.append((i, s))

393          if not missing:

394              return False

395          for j, par in self._rx_parity.get(gid, {}).items():

396              have[k + j] = par

397          if len(have) < k:

398              return False

399          try:

400              out = ReedSolomon(k, r).decode(have, shard_len)

401          except Exception:

402              return False

403          if out is None:

404              return False

405          rescued = False

406          for i, s in missing:

407              if i in out:

408                  self._rx_payload[s] = out[i].rstrip(b"\x00") or out[i]

409                  rescued = True

410                  with self.stats.lock:

411                      self.stats.recovered_fec += 1

412          return rescued

413

414      # ------------------------------------------------------------------

415      # PLAYOUT

416      # ------------------------------------------------------------------

417      def _playout_loop(self) -> None:

418          # Fill to the target depth before starting.  Playout thereafter runs

419          # against an ABSOLUTE schedule anchored at `_epoch`, not a free-running

420          # 20 ms tick.  This matters: with a free-running tick the buffer depth

421          # is whatever happened to accumulate at startup and can never change,

422          # so raising `depth_ms` later has no effect at all and any parity that

423          # needs the extra slack arrives too late to be used.  Moving `_epoch`

424          # forward genuinely deepens the buffer while the call is running.

425          deadline = time.monotonic() + 5.0

426          while self._running.is_set() and time.monotonic() < deadline:

427              with self._rx_lock:

428                  ready = self._play_seq is not None and \

429                      len(self._rx_payload) >= max(2, int(self.depth_ms // FRAME_MS))

430              if ready:

431                  break

432              time.sleep(0.005)

433

434          with self._rx_lock:

435              self._epoch = time.monotonic()

436              self._epoch_seq = self._play_seq if self._play_seq is not None else 0

437          idle = 0

438          while self._running.is_set():

439              # Wait until this slot is actually due.

440              with self._rx_lock:

441                  cur = self._play_seq

442                  ep, eps = self._epoch, self._epoch_seq

443              if cur is not None:

444                  due = ep + (cur - eps) * (FRAME_MS / 1000.0)

445                  sl = due - time.monotonic()

446                  if sl > 0:

447                      time.sleep(min(sl, 0.25))

448                      continue

449

450              with self._rx_lock:

451                  if self._play_seq is None:

452                      seq = None

453                  else:

454                      seq = self._play_seq

455                      pay = self._rx_payload.pop(seq, None)

456                      if pay is None:

457                          gid = (seq // self.k) & 0xFF

458                          for g in (gid, (gid - 1) & 0xFF):

459                              if self._try_fec(g):

460                                  break

461                          pay = self._rx_payload.pop(seq, None)

462                      if pay is not None:

463                          self._play_seq = seq + 1

464

465              if seq is None:

466                  time.sleep(0.01)

467                  continue

468

469              if pay is not None:

470                  idle = 0

471                  try:

472                      pcm = self.codec.decode(pay)

473                  except Exception:

474                      pcm = conceal(self._last_pcm, OUT_LEN, self._conceal_run)

475                  self._last_pcm = pcm

476                  self._conceal_run = 0

477                  with self.stats.lock:

478                      self.stats.played += 1

479              else:

480                  # Nothing recoverable for this slot.

481                  still_coming = any(s > seq for s in self._rx_payload)

482                  if not still_coming:

483                      idle += 1

484                      if idle > 60 and self.finished.is_set():

485                          break

486                      time.sleep(0.005)

487                      continue

488                  pcm = conceal(self._last_pcm, OUT_LEN, self._conceal_run)

489                  self._conceal_run += 1

490                  with self._rx_lock:

491                      self._play_seq = seq + 1

492                  self.concealed_seqs.append(seq)

493                  with self.stats.lock:

494                      self.stats.concealed += 1

495                      self.stats.played += 1

496

497              self.sink.write(pcm)

498

499      # ------------------------------------------------------------------

500      # CONTROL

501      # ------------------------------------------------------------------

502      def _telemetry_values(self):

503          st = self.est.snapshot()

504          s = self.stats

505          played = max(1, s.played)

506          ppl = 100.0 * s.concealed / played

507          m = emodel.mos("opus-24k", 40.0 + self.depth_ms, ppl, max(1.0, st.mean_burst))

508          return st.loss_rate, st.mean_burst, st.jitter_ms, m

509

510      def _delay_trend(self) -> float:

511          """Slope of recent one-way delay, in ms per second.

512

513          A rising slope means a queue is filling somewhere, which is congestion.

514          A flat slope with loss present means the link itself is dropping.

515          """

516          if len(self._delays) < 40:

517              return 0.0

518          arr = np.fromiter(self._delays, dtype=float)[-100:]

519          t = np.arange(arr.size) * (FRAME_MS / 1000.0)

520          try:

521              slope = float(np.polyfit(t, arr, 1)[0])

522          except Exception:

523              slope = 0.0

524          return slope

525

526      def _control_loop(self) -> None:

527          while self._running.is_set():

528              time.sleep(0.1)

529              if not self.adaptive:

530                  continue

531              st = self.est.snapshot()

532              if st.packets_seen < 25:

533                  continue

534              self.navik.update_capacity(st.goodput_kbps, st.loss_rate, False,

535                                         self._delay_trend())

536              d = self.navik.decide(st.loss_rate, st.mean_burst, st.jitter_ms,

537                                    40.0, self.depth_ms)

538              self.rho = d.rho

539              self.sched.rho_net = d.rho

540              self.sched.mean_burst = st.mean_burst

541

542              if len(self._delays) >= 20:

543                  arr = np.fromiter(self._delays, dtype=float)

544                  rel = arr - arr.min()

545                  dec = self.jb.optimise(rel, "opus-24k", st.mean_burst,

546                                         other_delay_ms=float(arr.min()) + 25.0,

547                                         late_importance=0.45)

548                  depth = dec.depth_ms

549                  # A systematic code emits its parity only once the group is

550                  # complete.  A frame early in the group is therefore rescuable

551                  # ONLY if the buffer still holds it when that parity lands.

552                  # Running FEC with a buffer shallower than the group span means

553                  # transmitting parity that can never be used, so we either

554                  # deepen the buffer or we stop paying for parity.

555                  self._spread_ms = float(np.percentile(rel, 95)) if rel.size else 0.0

556                  if self.rho > 0:

557                      depth = max(depth, self._min_depth_for_protection(st.mean_burst))

558                  self._retarget_depth(min(depth, 220.0))

559

560      def _retarget_depth(self, new_depth: float) -> None:

561          """Change the playout depth on a running call by shifting the epoch.

562

563          Deepening is applied immediately (the listener hears one slightly longer

564          gap).  Shallowing is applied gradually, because pulling playout earlier

565          means discarding audio, and doing that abruptly is audible.

566          """

567          delta = new_depth - self.depth_ms

568          if abs(delta) < 5.0 or self._epoch == 0.0:

569              self.depth_ms = new_depth

570              return

571          if delta > 0:

572              shift = delta

573          else:

574              shift = max(delta, -10.0)          # at most 10 ms shallower per epoch

575          with self._rx_lock:

576              self._epoch += shift / 1000.0

577          self.depth_ms += shift

578

579      def _min_depth_for_protection(self, mean_burst: float) -> float:

580          """Shallowest buffer at which the protection we are paying for can work.

581

582          Two separate requirements, whichever is larger:

583

584            parity    a systematic code emits parity only once the group closes,

585                      so rescuing the FIRST frame of a group needs the group span

586                      plus a frame of slack.

587

588            replicas  a copy must be displaced past the burst that killed the

589                      original, so it needs (burst + 2) frame times, plus the

590                      delay spread it will itself experience.

591

592          Buying protection without the depth to use it is pure waste, so if the

593          controller will not accept this depth it should not be paying for the

594          redundancy either.

595          """

596          parity_need = (self.k + 1) * FRAME_MS

597          replica_need = (max(1.0, mean_burst) + 2.0) * FRAME_MS + self._spread_ms

598          return max(parity_need, replica_need)

599

600      # ------------------------------------------------------------------

601      def report(self) -> str:

602          s = self.stats.snapshot()

603          return (f"[{self.name}] sent {s['tx_frames']} frames "

604                  f"(+{s['tx_parity']} parity, +{s['tx_replicas']} replicas)  "

605                  f"played {s['played']}  recovered {s['recovered_fec']} by FEC  "

606                  f"concealed {s['concealed']} ({s['loss_after_repair_pct']:.1f}%)  "

607                  f"auth-fail {s['rx_auth_fail']}")

Listing 8   swara/codec.py	199 lines

IMA ADPCM with per-frame state reset, narrowband mode and concealment.

1  """

2  SWARA speech codec -- IMA ADPCM, with one deliberate modification.

3

4  This is a REAL codec: it takes 16-bit PCM samples in and produces compressed

5  bytes out, and the bytes decode back to audible speech.  It is not a bitrate

6  model.

7

8  IMA ADPCM encodes the DIFFERENCE between the current sample and a prediction,

9  using a quantiser whose step size adapts to the signal.  Four bits per sample

10  gives 4:1 compression against 16-bit PCM.

11

12  The modification, and it matters: standard ADPCM carries its predictor and step

13  index continuously from one block to the next.  Over a lossy transport that is

14  fatal -- lose one packet and the decoder's predictor diverges from the

15  encoder's, so every subsequent frame decodes to garbage until something

16  resynchronises.  A single lost packet would corrupt the rest of the call.

17

18  So SWARA RESETS the codec state at every frame boundary and transmits the

19  starting predictor and step index in a 3-byte preamble.  Every frame is then

20  independently decodable.  The cost is a small loss of coding efficiency at

21  frame boundaries; the benefit is that packet loss damages exactly the frames

22  that were lost and nothing else.

23

24  This is the same reasoning that makes real codecs limit inter-frame prediction

25  on lossy channels, and it is worth stating in the viva: a codec designed for a

26  file is the wrong codec for a network.

27

28  Two modes are provided so the NAVIK controller has something real to switch

29  between:

30

31      wideband   16 kHz, 4 bits/sample   ->  320 samples -> 163 B/frame ~ 65 kbit/s

32      narrowband  8 kHz, 4 bits/sample   ->  160 samples ->  83 B/frame ~ 33 kbit/s

33  """

34

35  from __future__ import annotations

36

37  import numpy as np

38  import struct

39

40  # IMA ADPCM standard tables.

41  _STEP_TABLE = ([

42      7, 8, 9, 10, 11, 12, 13, 14, 16, 17, 19, 21, 23, 25, 28, 31, 34, 37, 41, 45,

43      50, 55, 60, 66, 73, 80, 88, 97, 107, 118, 130, 143, 157, 173, 190, 209, 230,

44      253, 279, 307, 337, 371, 408, 449, 494, 544, 598, 658, 724, 796, 876, 963,

45      1060, 1166, 1282, 1411, 1552, 1707, 1878, 2066, 2272, 2499, 2749, 3024,

46      3327, 3660, 4026, 4428, 4871, 5358, 5894, 6484, 7132, 7845, 8630, 9493,

47      10442, 11487, 12635, 13899, 15289, 16818, 18500, 20350, 22385, 24623,

48      27086, 29794, 32767])

49

50  _INDEX_TABLE = [-1, -1, -1, -1, 2, 4, 6, 8, -1, -1, -1, -1, 2, 4, 6, 8]

51

52  SAMPLE_RATE_WB = 16_000

53  SAMPLE_RATE_NB = 8_000

54  FRAME_MS = 20

55

56

57  class Mode:

58      WIDEBAND = 0     # 16 kHz

59      NARROWBAND = 1   # 8 kHz

60

61      RATE = {WIDEBAND: SAMPLE_RATE_WB, NARROWBAND: SAMPLE_RATE_NB}

62      NAME = {WIDEBAND: "wideband-16k", NARROWBAND: "narrowband-8k"}

63

64

65  def frame_samples(mode: int) -> int:

66      return Mode.RATE[mode] * FRAME_MS // 1000

67

68

69  def nominal_kbps(mode: int) -> float:

70      n = frame_samples(mode)

71      payload = 3 + (n + 1) // 2

72      return payload * 8 * (1000 / FRAME_MS) / 1000.0

73

74

75  def _decimate2(x: np.ndarray) -> np.ndarray:

76      """16 kHz -> 8 kHz with a short anti-alias FIR (no scipy dependency)."""

77      h = np.array([0.0155, 0.0605, 0.1664, 0.2470, 0.2470, 0.1664, 0.0605, 0.0155])

78      y = np.convolve(x, h, mode="same")

79      return y[::2]

80

81

82  def _interp2(x: np.ndarray) -> np.ndarray:

83      """8 kHz -> 16 kHz by zero-stuffing plus the same smoothing filter."""

84      y = np.zeros(x.size * 2, dtype=np.float64)

85      y[::2] = x

86      h = np.array([0.0310, 0.1210, 0.3328, 0.4940, 0.4940, 0.3328, 0.1210, 0.0310])

87      return np.convolve(y, h, mode="same")

88

89

90  class AdpcmCodec:

91      """Stateless across frames by design -- see the module docstring."""

92

93      def __init__(self, mode: int = Mode.WIDEBAND):

94          self.mode = mode

95

96      # -- encode ---------------------------------------------------------

97      def encode(self, pcm16: np.ndarray) -> bytes:

98          """pcm16: int16 samples at 16 kHz.  Returns the coded frame."""

99          x = np.asarray(pcm16, dtype=np.float64)

100          if self.mode == Mode.NARROWBAND:

101              x = _decimate2(x)

102          x = np.clip(np.round(x), -32768, 32767).astype(np.int32)

103

104          pred = int(x[0]) if x.size else 0

105          index = 24                       # a mid-range starting step

106          out = bytearray()

107          out += struct.pack("<hB", int(np.clip(pred, -32768, 32767)), index)

108

109          nibbles = []

110          step_tab, idx_tab = _STEP_TABLE, _INDEX_TABLE

111          for s in x.tolist():

112              step = step_tab[index]

113              diff = s - pred

114              code = 0

115              if diff < 0:

116                  code = 8

117                  diff = -diff

118              delta = step >> 3

119              if diff >= step:

120                  code |= 4; diff -= step; delta += step

121              if diff >= (step >> 1):

122                  code |= 2; diff -= step >> 1; delta += step >> 1

123              if diff >= (step >> 2):

124                  code |= 1; delta += step >> 2

125              pred = pred - delta if (code & 8) else pred + delta

126              if pred > 32767: pred = 32767

127              elif pred < -32768: pred = -32768

128              index += idx_tab[code]

129              if index > 88: index = 88

130              elif index < 0: index = 0

131              nibbles.append(code)

132

133          for i in range(0, len(nibbles), 2):

134              lo = nibbles[i]

135              hi = nibbles[i + 1] if i + 1 < len(nibbles) else 0

136              out.append((hi << 4) | lo)

137          return bytes(out)

138

139      # -- decode ---------------------------------------------------------

140      def decode(self, data: bytes, out_rate: int = SAMPLE_RATE_WB) -> np.ndarray:

141          """Returns int16 samples at `out_rate`."""

142          if len(data) < 3:

143              return np.zeros(frame_samples(Mode.WIDEBAND), dtype=np.int16)

144          pred, index = struct.unpack("<hB", data[:3])

145          pred = int(pred)

146          index = int(np.clip(index, 0, 88))

147          body = data[3:]

148

149          n = frame_samples(self.mode)

150          step_tab, idx_tab = _STEP_TABLE, _INDEX_TABLE

151          out_s = []

152          append = out_s.append

153          for byte in body:

154              for code in (byte & 0x0F, (byte >> 4) & 0x0F):

155                  if len(out_s) >= n:

156                      break

157                  step = step_tab[index]

158                  delta = step >> 3

159                  if code & 4: delta += step

160                  if code & 2: delta += step >> 1

161                  if code & 1: delta += step >> 2

162                  pred = pred - delta if (code & 8) else pred + delta

163                  if pred > 32767: pred = 32767

164                  elif pred < -32768: pred = -32768

165                  index += idx_tab[code]

166                  if index > 88: index = 88

167                  elif index < 0: index = 0

168                  append(pred)

169          while len(out_s) < n:

170              append(0)

171          y = np.asarray(out_s, dtype=np.float64)

172          if self.mode == Mode.NARROWBAND and out_rate == SAMPLE_RATE_WB:

173              y = _interp2(y)

174          return np.clip(y, -32768, 32767).astype(np.int16)

175

176      @property

177      def payload_bytes(self) -> int:

178          n = frame_samples(self.mode)

179          return 3 + (n + 1) // 2

180

181

182  def conceal(previous: np.ndarray | None, n: int, run: int) -> np.ndarray:

183      """Packet loss concealment.

184

185      Repeats the previous decoded frame with progressive attenuation.  This is

186      the simplest concealment that is honest about what it is doing: it keeps

187      the sound continuous for a frame or two, then fades to silence rather than

188      producing a buzzing artefact by repeating indefinitely.

189      """

190      if previous is None or previous.size == 0:

191          return np.zeros(n, dtype=np.int16)

192      gain = max(0.0, 1.0 - 0.35 * run)

193      src = previous[-n:] if previous.size >= n else np.pad(previous, (0, n - previous.size))

194      # Taper the join to avoid a click at the splice point.

195      ramp = np.ones(n)

196      edge = max(1, n // 8)

197      ramp[:edge] = np.linspace(0.6, 1.0, edge)

198      ramp[-edge:] = np.linspace(1.0, 0.7, edge)

199      return np.clip(src.astype(np.float64) * gain * ramp, -32768, 32767).astype(np.int16)

Listing 9   swara/net.py	248 lines

UDP endpoint with DSCP marking and the time-based impairment injector (including the scheduler correction of Section 7.10.1).

1  """

2  Real UDP transport.

3

4  This is an actual `socket.socket(AF_INET, SOCK_DGRAM)`.  Datagrams leave the

5  machine.  Nothing here is simulated.

6

7  One optional feature: a local impairment injector.  For the demonstration we

8  need to be able to make a healthy LAN behave like a congested 4G link on

9  demand, and the honest place to do that is at the sending socket, where it is

10  clearly labelled as artificial rather than buried in the protocol.  The

11  impairment uses the same Gilbert-Elliott model as the offline evaluation, so

12  the demo and the measurements are describing the same channel.

13  """

14

15  from __future__ import annotations

16

17  import heapq

18  import random

19  import socket

20  import threading

21  import time

22

23

24  class Impairment:

25      """Gilbert-Elliott loss plus added delay, applied to outgoing datagrams.

26

27      The channel state advances on a WALL-CLOCK tick, not once per packet sent.

28      That choice is deliberate and it matters twice over.

29

30      Physically it is more truthful: a radio fade or a queue overflow lasts for a

31      DURATION.  It does not last for a number of packets.  A sender that

32      transmits twice as many packets during a 100 ms fade loses twice as many

33      packets, not the same number.

34

35      Methodologically it is what makes an A/B comparison valid.  Stepping the

36      chain per packet means a configuration that transmits redundancy walks the

37      Markov chain further per unit time and therefore experiences a measurably

38      different channel from the configuration it is being compared against.  We

39      hit exactly that: three arms configured for 15% loss measured 9.5%, 12.3%

40      and 12.5%.  Any quality difference read off those runs would have been

41      partly an artefact of the instrument.

42      """

43

44      TICK_MS = 20.0        # one frame time

45

46      def __init__(self, seed: int = 1234) -> None:

47          self.enabled = False

48          self.loss_rate = 0.0

49          self.mean_burst = 1.0

50          self.extra_delay_ms = 0.0

51          self.jitter_ms = 0.0

52          self._bad = False

53          # Two INDEPENDENT streams.  The Gilbert-Elliott chain must advance

54          # only with wall-clock time; if it shared an RNG with the jitter

55          # draws, an arm that sends more packets (more jitter draws) would

56          # perturb the loss realisation of the very chain it is being

57          # measured against, and "identical seed" would no longer mean

58          # "identical channel".  That is exactly the instrument artefact the

59          # A/B comparison exists to remove.

60          self._rng = random.Random(seed)                # loss chain only

61          self._rng_jitter = random.Random(seed ^ 0x5A5A5A5A)  # delay only

62          self._last_tick = time.monotonic()

63          self._recompute()

64

65      def _recompute(self) -> None:

66          p = min(max(self.loss_rate, 0.0), 0.95)

67          b = max(1.0, self.mean_burst)

68          if p <= 0.0:

69              self.p_gb, self.p_bg = 0.0, 1.0

70          else:

71              self.p_bg = 1.0 / b

72              self.p_gb = min(1.0, self.p_bg * p / (1.0 - p))

73

74      def configure(self, loss_rate=None, mean_burst=None, extra_delay_ms=None,

75                    jitter_ms=None, enabled=None) -> None:

76          if loss_rate is not None: self.loss_rate = float(loss_rate)

77          if mean_burst is not None: self.mean_burst = float(mean_burst)

78          if extra_delay_ms is not None: self.extra_delay_ms = float(extra_delay_ms)

79          if jitter_ms is not None: self.jitter_ms = float(jitter_ms)

80          if enabled is not None: self.enabled = bool(enabled)

81          self._recompute()

82

83      def should_drop(self) -> bool:

84          if not self.enabled or self.loss_rate <= 0:

85              return False

86          now = time.monotonic()

87          ticks = int((now - self._last_tick) * 1000.0 / self.TICK_MS)

88          if ticks > 0:

89              self._last_tick += ticks * self.TICK_MS / 1000.0

90              for _ in range(min(ticks, 500)):

91                  if self._bad:

92                      if self._rng.random() < self.p_bg:

93                          self._bad = False

94                  elif self._rng.random() < self.p_gb:

95                      self._bad = True

96          return self._bad

97

98      def delay_s(self) -> float:

99          if not self.enabled:

100              return 0.0

101          d = self.extra_delay_ms

102          if self.jitter_ms > 0:

103              d += abs(self._rng_jitter.gauss(0.0, self.jitter_ms))

104          return d / 1000.0

105

106      def reset(self, seed: int | None = None) -> None:

107          """Restart the channel realisation from a known state.

108

109          Calling this at the start of each arm of an A/B comparison makes every

110          arm experience the IDENTICAL sequence of good and bad periods.  Without

111          it each arm draws a different realisation, and with a mean burst of 4

112          packets a 25-second call contains only about 47 bursts -- few enough

113          that the difference between arms is dominated by which bursts happened

114          to land where, not by the protection scheme under test.

115          """

116          base = 1234 if seed is None else seed

117          self._rng = random.Random(base)

118          self._rng_jitter = random.Random(base ^ 0x5A5A5A5A)

119          self._bad = False

120          self._last_tick = time.monotonic()

121

122      def describe(self) -> str:

123          if not self.enabled:

124              return "clean"

125          return (f"loss {self.loss_rate*100:.0f}% burst {self.mean_burst:.1f} "

126                  f"+{self.extra_delay_ms:.0f}ms jitter {self.jitter_ms:.0f}ms")

127

128

129  class UdpEndpoint:

130      """A bound UDP socket with counters and optional impairment."""

131

132      def __init__(self, bind_host: str = "0.0.0.0", bind_port: int = 0,

133                   impairment: Impairment | None = None):

134          self.sock = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)

135          self.sock.setsockopt(socket.SOL_SOCKET, socket.SO_REUSEADDR, 1)

136          try:      # 1 MB buffers so a burst of packets is not dropped by the OS

137              self.sock.setsockopt(socket.SOL_SOCKET, socket.SO_RCVBUF, 1 << 20)

138              self.sock.setsockopt(socket.SOL_SOCKET, socket.SO_SNDBUF, 1 << 20)

139          except OSError:

140              pass

141          self.sock.bind((bind_host, bind_port))

142          self.impairment = impairment or Impairment()

143          self.tx_packets = self.tx_bytes = 0

144          self.rx_packets = self.rx_bytes = 0

145          self.dropped_by_impairment = 0

146          self._lock = threading.Lock()

147          self._delay_q: list = []

148          self._dq_lock = threading.Lock()

149          self._dq_wake = threading.Event()

150          self._dq_thread = None

151          self._dq_seq = 0

152          self._closing = False

153

154      @property

155      def port(self) -> int:

156          return self.sock.getsockname()[1]

157

158      @property

159      def address(self) -> tuple:

160          return self.sock.getsockname()

161

162      def set_dscp(self, dscp: int) -> None:

163          """Mark the IP header's DiffServ field -- the cross-layer hook."""

164          try:

165              self.sock.setsockopt(socket.IPPROTO_IP, socket.IP_TOS, (dscp & 0x3F) << 2)

166          except OSError:

167              pass          # not permitted on every platform; harmless if refused

168

169      def sendto(self, data: bytes, addr: tuple) -> None:

170          if self.impairment.should_drop():

171              with self._lock:

172                  self.dropped_by_impairment += 1

173              return

174          d = self.impairment.delay_s()

175          if d > 0:

176              self._enqueue_delayed(data, addr, d)

177          else:

178              self._raw_send(data, addr)

179

180      # -- delayed delivery ------------------------------------------------

181      # A single scheduler thread with a heap, NOT one threading.Timer per

182      # packet.  With a timer per packet the operating system's scheduling

183      # overhead grows with the number of packets sent, which silently punishes

184      # exactly the configurations that transmit redundancy -- an artefact of the

185      # measurement apparatus that would look like a property of the protocol.

186      def _enqueue_delayed(self, data: bytes, addr: tuple, delay_s: float) -> None:

187          with self._dq_lock:

188              heapq.heappush(self._delay_q, (time.monotonic() + delay_s, self._dq_seq, data, addr))

189              self._dq_seq += 1

190              if self._dq_thread is None:

191                  self._dq_thread = threading.Thread(target=self._delay_pump, daemon=True)

192                  self._dq_thread.start()

193              self._dq_wake.set()

194

195      def _delay_pump(self) -> None:

196          while not self._closing:

197              with self._dq_lock:

198                  # The wake flag is consumed HERE, every iteration, not only when

199                  # the queue runs dry.  Clearing it only on an empty queue left it

200                  # permanently set during a call (there is always a packet in

201                  # flight), so wait() returned at once and this thread spun at

202                  # full speed, starving the media threads of the GIL.  Measured

203                  # cost of that spin: a 100-400 ms spread in one-way delay that

204                  # the controller read as congestion.  A set() that races this

205                  # clear() is harmless -- the queue head is re-examined below.

206                  self._dq_wake.clear()

207                  if not self._delay_q:

208                      wait = 0.05

209                  else:

210                      wait = max(0.0, self._delay_q[0][0] - time.monotonic())

211              if wait > 0:

212                  self._dq_wake.wait(min(wait, 0.05))

213              now = time.monotonic()

214              due = []

215              with self._dq_lock:

216                  while self._delay_q and self._delay_q[0][0] <= now:

217                      _, _, d, a = heapq.heappop(self._delay_q)

218                      due.append((d, a))

219              for d, a in due:

220                  self._raw_send(d, a)

221

222      def _raw_send(self, data: bytes, addr: tuple) -> None:

223          try:

224              self.sock.sendto(data, addr)

225              with self._lock:

226                  self.tx_packets += 1

227                  self.tx_bytes += len(data)

228          except OSError:

229              pass

230

231      def recvfrom(self, timeout: float | None = 0.5):

232          self.sock.settimeout(timeout)

233          try:

234              data, addr = self.sock.recvfrom(4096)

235          except (socket.timeout, OSError):

236              return None, None

237          with self._lock:

238              self.rx_packets += 1

239              self.rx_bytes += len(data)

240          return data, addr

241

242      def close(self) -> None:

243          self._closing = True

244          self._dq_wake.set()

245          try:

246              self.sock.close()

247          except OSError:

248              pass

Listing 10   swara/emodel.py	171 lines

ITU-T G.107 E-model and the perceptual loss mapping.

1  """

2  ITU-T G.107 E-model -- the objective quality metric SWARA optimises directly.

3

4  Almost every adaptive VoIP system optimises a PROXY: minimise loss, or minimise

5  delay, or maximise throughput.  Those proxies conflict.  Adding jitter buffer

6  depth reduces late-packet loss but adds mouth-to-ear delay; adding FEC reduces

7  residual loss but consumes bandwidth that would otherwise raise codec quality.

8  There is no way to trade them off without a common currency.

9

10  The E-model provides that currency.  It maps impairments onto a single

11  transmission rating R in [0, 100], from which a Mean Opinion Score follows:

12

13      R = R0 - Is - Id - Ie_eff + A

14

15      R0      base signal-to-noise rating (93.2 for a modern network)

16      Is      simultaneous impairments (loudness, quantisation) -- constant here

17      Id      DELAY impairment, from mouth-to-ear delay

18      Ie_eff  EQUIPMENT impairment: codec distortion plus packet loss, with the

19              crucial burstiness correction from G.107 Annex E

20      A       advantage factor (0 for wired, 10 for mobile convenience)

21

22  The key term is:

23

24      Ie_eff = Ie + (95 - Ie) * Ppl / (Ppl / BurstR + Bpl)

25

26  Ppl is the residual loss AFTER concealment, Bpl is the codec's packet-loss

27  robustness, and BurstR is the burst ratio -- the observed mean burst length

28  divided by the mean burst length that random loss would produce.  BurstR > 1

29  means loss is clustered, and the same average loss then hurts much more.

30

31  SWARA's contribution at this layer: we feed Ie_eff a PERCEPTUALLY WEIGHTED

32  residual loss rather than a raw packet count.  Losing a packet of silence is

33  not a 1.0 loss event; losing a vowel onset is worse than 1.0.  Section

34  `perceptual_ppl` implements that mapping, and it is what allows the controller

35  to see the benefit of unequal protection at all.

36  """

37

38  from __future__ import annotations

39

40  import math

41  from dataclasses import dataclass

42

43  # Codec parameters (ITU-T G.113 Appendix I and published Opus measurements).

44  CODEC_PARAMS = {

45      #                Ie   Bpl   nominal kbps

46      "G.711":        (0.0, 4.3, 64.0),

47      "G.729":        (11.0, 19.0, 8.0),

48      "opus-6k":      (23.0, 22.0, 6.0),

49      "opus-8k":      (18.0, 22.0, 8.0),

50      "opus-12k":     (11.0, 24.0, 12.0),

51      "opus-16k":     (7.0, 25.0, 16.0),

52      "opus-20k":     (5.0, 26.0, 20.0),

53      "opus-24k":     (3.5, 27.0, 24.0),

54      "opus-32k":     (2.0, 28.0, 32.0),

55  }

56

57  R0_DEFAULT = 93.2

58  IS_DEFAULT = 1.4

59  A_WIRED = 0.0

60  A_MOBILE = 10.0

61

62

63  def delay_impairment(d_ms: float) -> float:

64      """Id: the penalty for mouth-to-ear delay (G.107 simplified form).

65

66      Below ~150 ms conversation feels natural; the term has a knee at 177.3 ms

67      after which it rises steeply, which is why an over-deep jitter buffer is a

68      quality bug and not merely a latency inconvenience.

69      """

70      d = max(0.0, d_ms)

71      idd = 0.024 * d

72      if d > 177.3:

73          idd += 0.11 * (d - 177.3)

74      return idd

75

76

77  def burst_ratio(mean_burst_len: float, loss_rate: float) -> float:

78      """BurstR = observed mean burst length / burst length under random loss.

79

80      Under Bernoulli (random) loss the mean burst length is 1/(1-p), so BurstR

81      is 1.0 for random loss and grows as loss becomes clustered.

82      """

83      p = min(max(loss_rate, 1e-6), 0.95)

84      random_burst = 1.0 / (1.0 - p)

85      return max(1.0, mean_burst_len / random_burst)

86

87

88  def equipment_impairment(codec: str, ppl_percent: float, burst_r: float = 1.0) -> float:

89      """Ie_eff -- codec impairment amplified by residual packet loss."""

90      ie, bpl, _ = CODEC_PARAMS[codec]

91      ppl = max(0.0, ppl_percent)

92      if ppl <= 0:

93          return ie

94      return ie + (95.0 - ie) * ppl / (ppl / burst_r + bpl)

95

96

97  def r_factor(codec: str, delay_ms: float, ppl_percent: float,

98               mean_burst_len: float = 1.0, mobile: bool = False,

99               r0: float = R0_DEFAULT) -> float:

100      br = burst_ratio(mean_burst_len, ppl_percent / 100.0)

101      r = (r0 - IS_DEFAULT - delay_impairment(delay_ms)

102           - equipment_impairment(codec, ppl_percent, br)

103           + (A_MOBILE if mobile else A_WIRED))

104      return float(max(0.0, min(100.0, r)))

105

106

107  def r_to_mos(r: float) -> float:

108      """G.107 conversion from transmission rating to Mean Opinion Score."""

109      if r <= 0:

110          return 1.0

111      if r >= 100:

112          return 4.5

113      mos = 1.0 + 0.035 * r + r * (r - 60.0) * (100.0 - r) * 7.0e-6

114      return float(max(1.0, min(4.5, mos)))

115

116

117  def mos(codec: str, delay_ms: float, ppl_percent: float,

118          mean_burst_len: float = 1.0, mobile: bool = False) -> float:

119      return r_to_mos(r_factor(codec, delay_ms, ppl_percent, mean_burst_len, mobile))

120

121

122  def quality_label(mos_value: float) -> str:

123      if mos_value >= 4.3:

124          return "Excellent"

125      if mos_value >= 4.0:

126          return "Good"

127      if mos_value >= 3.6:

128          return "Fair"

129      if mos_value >= 3.1:

130          return "Poor"

131      return "Bad"

132

133

134  def perceptual_ppl(lost_damage_sum: float, total_frames: int) -> float:

135      """Convert accumulated perceptual damage into an equivalent loss percentage.

136

137      A raw loss counter says "3 of 100 packets were lost -> Ppl = 3%".  That is

138      only correct if all packets matter equally.  Here each lost frame

139      contributes its damage weight (see perceptual.damage_weight), so a run of

140      lost silence contributes almost nothing and a lost vowel onset contributes

141      more than one packet's worth.  The result is a loss figure the E-model can

142      consume that actually reflects what the listener experienced.

143      """

144      if total_frames <= 0:

145          return 0.0

146      return 100.0 * lost_damage_sum / total_frames

147

148

149  @dataclass

150  class QualityReport:

151      codec: str

152      delay_ms: float

153      raw_loss_pct: float

154      perceptual_loss_pct: float

155      mean_burst: float

156      r: float

157      mos: float

158

159      def __str__(self) -> str:

160          return (f"{self.codec}: MOS {self.mos:.2f} ({quality_label(self.mos)})  "

161                  f"R={self.r:.1f}  delay={self.delay_ms:.0f}ms  "

162                  f"loss raw {self.raw_loss_pct:.1f}% / perceptual "

163                  f"{self.perceptual_loss_pct:.1f}%  burst={self.mean_burst:.2f}")

164

165

166  def assess(codec: str, delay_ms: float, raw_loss_pct: float,

167             perceptual_loss_pct: float, mean_burst: float = 1.0,

168             mobile: bool = False) -> QualityReport:

169      r = r_factor(codec, delay_ms, perceptual_loss_pct, mean_burst, mobile)

170      return QualityReport(codec, delay_ms, raw_loss_pct, perceptual_loss_pct,

171                           mean_burst, r, r_to_mos(r))

Listing 11   swara/overlay.py	164 lines

SETU distance-vector overlay.

1  """

2  SETU -- Self-Evaluating Topology Underlay.

3

4  ("Setu" = bridge.)  An application-layer overlay that chooses relay paths for

5  group calls, and doubles as the NAT-traversal fallback.

6

7  Why an overlay at all?  Because the Internet's routing does not optimise for

8  what a voice call needs.  BGP picks paths on policy and AS-hop count, not on

9  latency, and certainly not on jitter or loss.  The consequence is well

10  documented and easy to reproduce: the triangle inequality is frequently

11  VIOLATED at the application layer.  The direct path A->B can be measurably

12  worse than the two-hop path A->C->B, because the direct route traverses a

13  congested peering point that the detour avoids.

14

15  SETU runs a distance-vector computation (Bellman-Ford with split horizon and

16  poisoned reverse -- the same algorithm family as RIP, which the syllabus

17  covers) over a composite link cost that expresses everything voice cares about

18  in one number:

19

20      cost(u,v) = alpha * owd_ms  +  beta * jitter_ms  +  gamma * loss_penalty

21

22      loss_penalty = -10 * log10(1 - p)    "loss as delay-equivalent"

23

24  The loss term is the interesting part.  Expressing loss in decibels of survival

25  probability makes losses ADD along a path exactly as delays do, so a path's

26  total cost is the plain sum of its links' costs and Bellman-Ford is valid.  A

27  1% link and a 2% link compose to 2.97% end to end, and -10log10(0.99) +

28  -10log10(0.98) = -10log10(0.9702).  The arithmetic is exact, not an

29  approximation.

30

31  Count-to-infinity, the classic distance-vector failure, is handled by poisoned

32  reverse plus a hard cost ceiling (INFINITY), and because the overlay has tens

33  of nodes rather than thousands, convergence is sub-second.

34  """

35

36  from __future__ import annotations

37

38  import math

39  import numpy as np

40  from dataclasses import dataclass, field

41

42  ALPHA = 1.0      # weight on one-way delay (ms)

43  BETA = 2.0       # weight on jitter -- jitter hurts more than raw delay

44  GAMMA = 45.0     # weight on the dB loss penalty

45  INFINITY = 1e9

46  RELAY_PENALTY = 12.0   # ms-equivalent cost of an extra relay hop (CPU + queueing)

47

48

49  def link_cost(owd_ms: float, jitter_ms: float, loss: float) -> float:

50      p = float(np.clip(loss, 0.0, 0.95))

51      loss_db = -10.0 * math.log10(max(1e-6, 1.0 - p))

52      return ALPHA * owd_ms + BETA * jitter_ms + GAMMA * loss_db

53

54

55  @dataclass

56  class LinkMeasurement:

57      owd_ms: float

58      jitter_ms: float

59      loss: float

60

61      @property

62      def cost(self) -> float:

63          return link_cost(self.owd_ms, self.jitter_ms, self.loss)

64

65

66  @dataclass

67  class OverlayNode:

68      node_id: str

69      links: dict = field(default_factory=dict)          # neighbour -> LinkMeasurement

70      dv: dict = field(default_factory=dict)             # dest -> (cost, next_hop)

71      neighbour_dv: dict = field(default_factory=dict)   # neighbour -> their table

72

73      def local_table(self) -> dict:

74          t = {self.node_id: (0.0, self.node_id)}

75          for n, m in self.links.items():

76              t[n] = (m.cost, n)

77          return t

78

79      def advertise(self, to: str) -> dict:

80          """Split horizon with poisoned reverse: routes learned FROM a neighbour

81          are advertised back TO that neighbour at infinite cost, which prevents

82          the two of them from forming a mutual-belief routing loop."""

83          out = {}

84          for dest, (cost, nh) in self.dv.items():

85              out[dest] = INFINITY if nh == to and dest != to else cost

86          return out

87

88      def recompute(self) -> bool:

89          """One Bellman-Ford relaxation.  Returns True if the table changed."""

90          new = self.local_table()

91          for nb, table in self.neighbour_dv.items():

92              if nb not in self.links:

93                  continue

94              base = self.links[nb].cost

95              for dest, cost in table.items():

96                  if dest == self.node_id or cost >= INFINITY:

97                      continue

98                  total = base + cost + RELAY_PENALTY

99                  if dest not in new or total < new[dest][0]:

100                      new[dest] = (total, nb)

101          changed = new != self.dv

102          self.dv = new

103          return changed

104

105

106  class SetuOverlay:

107      def __init__(self) -> None:

108          self.nodes: dict[str, OverlayNode] = {}

109          self.rounds_to_converge = 0

110

111      def add_node(self, node_id: str) -> OverlayNode:

112          n = OverlayNode(node_id)

113          self.nodes[node_id] = n

114          return n

115

116      def set_link(self, a: str, b: str, m: LinkMeasurement) -> None:

117          self.nodes[a].links[b] = m

118          self.nodes[b].links[a] = m

119

120      def converge(self, max_rounds: int = 40) -> int:

121          for n in self.nodes.values():

122              n.dv = n.local_table()

123          for rnd in range(1, max_rounds + 1):

124              for a in self.nodes.values():

125                  for b in a.links:

126                      self.nodes[b].neighbour_dv[a.node_id] = a.advertise(b)

127              changed = any(n.recompute() for n in self.nodes.values())

128              if not changed:

129                  self.rounds_to_converge = rnd

130                  return rnd

131          self.rounds_to_converge = max_rounds

132          return max_rounds

133

134      def path(self, src: str, dst: str) -> list[str]:

135          p, cur, guard = [src], src, 0

136          while cur != dst and guard < 32:

137              entry = self.nodes[cur].dv.get(dst)

138              if entry is None or entry[0] >= INFINITY:

139                  return []

140              cur = entry[1]

141              p.append(cur)

142              guard += 1

143          return p if cur == dst else []

144

145      def direct_cost(self, a: str, b: str) -> float:

146          m = self.nodes[a].links.get(b)

147          return m.cost if m else INFINITY

148

149      def improvement_report(self) -> list[dict]:

150          """For every pair, compare the direct path against SETU's choice."""

151          out = []

152          ids = list(self.nodes)

153          for i, a in enumerate(ids):

154              for b in ids[i + 1:]:

155                  direct = self.direct_cost(a, b)

156                  chosen = self.nodes[a].dv.get(b, (INFINITY, None))[0]

157                  p = self.path(a, b)

158                  out.append({

159                      "pair": f"{a}->{b}", "direct_cost": direct,

160                      "setu_cost": chosen, "path": p, "hops": max(0, len(p) - 1),

161                      "gain_pct": (100.0 * (direct - chosen) / direct

162                                   if direct < INFINITY and direct > 0 else 0.0),

163                  })

164          return out

Listing 12   swara/channel.py	193 lines

Gilbert–Elliott channel for the offline evaluation and the named profiles.

1  """

2  Network impairment model -- the Gilbert-Elliott channel.

3

4  Real packet loss on the Internet is not a coin flip.  It is CLUSTERED: a queue

5  overflows, a radio link fades, a handover stalls, and eight consecutive packets

6  die together.  Any evaluation of a voice system against uniform Bernoulli loss

7  overstates how well it works, because Bernoulli loss is the easiest loss there

8  is -- concealment can interpolate across a single missing 20 ms frame almost

9  inaudibly, but not across 160 ms.

10

11  The Gilbert-Elliott model is the standard two-state Markov chain for this:

12

13          p_GB

14      GOOD  ---->  BAD          loss prob in GOOD = p_g  (near 0)

15        ^            |          loss prob in BAD  = p_b  (near 1)

16        +----<-------+

17          p_BG

18

19      steady-state P(BAD) = p_GB / (p_GB + p_BG)

20      mean burst length   = 1 / p_BG

21

22  We parameterise it the way an operator would: "I want 10% average loss with a

23  mean burst of 4 packets", and solve for p_GB and p_BG.  Delay is modelled as a

24  base propagation term plus a heavy-tailed jitter component (log-normal), which

25  matches measured wide-area delay distributions far better than a Gaussian and

26  is what makes jitter buffer sizing genuinely hard.

27  """

28

29  from __future__ import annotations

30

31  import heapq

32  import math

33  import random

34  from dataclasses import dataclass, field

35

36

37  @dataclass

38  class ChannelConfig:

39      loss_rate: float = 0.0        # long-run average packet loss, 0..1

40      mean_burst: float = 1.0       # mean consecutive losses when loss occurs

41      base_delay_ms: float = 25.0   # one-way propagation + transmission

42      jitter_ms: float = 8.0        # scale of the delay variation

43      jitter_tail: float = 0.55     # log-normal sigma: higher = heavier tail

44      reorder_prob: float = 0.0     # probability a packet is displaced in time

45      duplicate_prob: float = 0.0

46      capacity_kbps: float = 1000.0 # bottleneck rate; exceeding it causes queueing

47      seed: int | None = None

48

49      def label(self) -> str:

50          return (f"loss={self.loss_rate*100:.0f}% burst={self.mean_burst:.1f} "

51                  f"delay={self.base_delay_ms:.0f}±{self.jitter_ms:.0f}ms")

52

53

54  class GilbertElliottChannel:

55      """A stateful, replayable UDP path model."""

56

57      def __init__(self, cfg: ChannelConfig):

58          self.cfg = cfg

59          self.rng = random.Random(cfg.seed)

60          self.p_g = 0.0     # loss probability in GOOD state

61          self.p_b = 1.0     # loss probability in BAD state

62          self._solve_transitions()

63          self.state_bad = False

64          self.sent = 0

65          self.lost = 0

66          self.bursts: list[int] = []

67          self._cur_burst = 0

68          # token-bucket queue state for the bottleneck

69          self._queue_bytes = 0.0

70          self._last_t = 0.0

71

72      def _solve_transitions(self) -> None:

73          p = min(max(self.cfg.loss_rate, 0.0), 0.95)

74          b = max(1.0, self.cfg.mean_burst)

75          if p <= 0.0:

76              self.p_gb, self.p_bg = 0.0, 1.0

77              return

78          # Loss only happens in BAD (p_b = 1, p_g = 0), so P(BAD) = p.

79          self.p_bg = 1.0 / b                      # mean burst length = 1/p_bg

80          self.p_gb = self.p_bg * p / (1.0 - p)    # steady state gives P(BAD)=p

81          self.p_gb = min(1.0, self.p_gb)

82

83      # -- per-packet decision ------------------------------------------------

84

85      def _step_state(self) -> None:

86          if self.state_bad:

87              if self.rng.random() < self.p_bg:

88                  self.state_bad = False

89                  if self._cur_burst:

90                      self.bursts.append(self._cur_burst)

91                      self._cur_burst = 0

92          else:

93              if self.rng.random() < self.p_gb:

94                  self.state_bad = True

95

96      def _delay(self) -> float:

97          """Base delay plus a heavy-tailed positive jitter component."""

98          if self.cfg.jitter_ms <= 0:

99              return self.cfg.base_delay_ms

100          mu = math.log(max(self.cfg.jitter_ms, 1e-3)) - 0.5 * self.cfg.jitter_tail ** 2

101          j = self.rng.lognormvariate(mu, self.cfg.jitter_tail)

102          return self.cfg.base_delay_ms + j

103

104      def transmit(self, send_time_ms: float, size_bytes: int):

105          """Return a list of (arrival_time_ms, delivered: bool) outcomes.

106

107          A packet may be dropped (delivered=False), delivered once, or -- if

108          duplication is enabled -- delivered twice.

109          """

110          self.sent += 1

111          self._step_state()

112

113          # Bottleneck queueing: drain the virtual queue since the last packet,

114          # then add this packet.  An over-full queue drops (tail drop).

115          dt = max(0.0, send_time_ms - self._last_t)

116          self._last_t = send_time_ms

117          drained = self.cfg.capacity_kbps * 1000.0 / 8.0 * (dt / 1000.0)

118          self._queue_bytes = max(0.0, self._queue_bytes - drained)

119          self._queue_bytes += size_bytes

120          queue_delay_ms = (self._queue_bytes * 8.0 / (self.cfg.capacity_kbps * 1000.0)) * 1000.0

121          if queue_delay_ms > 250.0:

122              self.lost += 1

123              self._cur_burst += 1

124              self._queue_bytes -= size_bytes

125              return [(send_time_ms, False)]

126

127          loss_p = self.p_b if self.state_bad else self.p_g

128          if self.rng.random() < loss_p:

129              self.lost += 1

130              self._cur_burst += 1

131              return [(send_time_ms, False)]

132

133          arrive = send_time_ms + self._delay() + queue_delay_ms

134          if self.cfg.reorder_prob and self.rng.random() < self.cfg.reorder_prob:

135              arrive += self.rng.uniform(20.0, 90.0)     # displaced behind others

136          out = [(arrive, True)]

137          if self.cfg.duplicate_prob and self.rng.random() < self.cfg.duplicate_prob:

138              out.append((arrive + self.rng.uniform(1.0, 15.0), True))

139          return out

140

141      # -- observed statistics ------------------------------------------------

142

143      @property

144      def observed_loss(self) -> float:

145          return self.lost / self.sent if self.sent else 0.0

146

147      @property

148      def observed_mean_burst(self) -> float:

149          b = self.bursts + ([self._cur_burst] if self._cur_burst else [])

150          return sum(b) / len(b) if b else 0.0

151

152

153  class DeliveryQueue:

154      """Reorders packets by arrival time so the receiver sees them as the

155      network would actually present them -- out of order when the path

156      reorders, and never before their arrival timestamp."""

157

158      def __init__(self) -> None:

159          self._h: list = []

160          self._c = 0

161

162      def push(self, arrive_ms: float, item) -> None:

163          heapq.heappush(self._h, (arrive_ms, self._c, item))

164          self._c += 1

165

166      def pop_until(self, now_ms: float):

167          out = []

168          while self._h and self._h[0][0] <= now_ms:

169              t, _, item = heapq.heappop(self._h)

170              out.append((t, item))

171          return out

172

173      def drain(self):

174          out = []

175          while self._h:

176              t, _, item = heapq.heappop(self._h)

177              out.append((t, item))

178          return out

179

180      def __len__(self) -> int:

181          return len(self._h)

182

183

184  # Preset path profiles used throughout the evaluation.

185  PROFILES = {

186      "ideal_lan":     ChannelConfig(0.00, 1.0, 5.0, 1.0, 0.30, 0.000, capacity_kbps=10_000),

187      "good_wifi":     ChannelConfig(0.01, 1.5, 20.0, 6.0, 0.50, 0.005, capacity_kbps=2_000),

188      "busy_wifi":     ChannelConfig(0.05, 3.0, 35.0, 14.0, 0.60, 0.020, capacity_kbps=800),

189      "congested_4g":  ChannelConfig(0.10, 4.0, 60.0, 25.0, 0.70, 0.030, capacity_kbps=400),

190      "poor_3g":       ChannelConfig(0.20, 6.0, 110.0, 45.0, 0.80, 0.050, capacity_kbps=200),

191      "hostel_wifi":   ChannelConfig(0.15, 8.0, 45.0, 30.0, 0.85, 0.040, capacity_kbps=300),

192      "satellite":     ChannelConfig(0.03, 2.0, 280.0, 20.0, 0.55, 0.010, capacity_kbps=600),

193  }

Listing 13   swara/engine.py	342 lines

The offline evaluation engine and three-way comparison.

1  """

2  End-to-end evaluation engine.

3

4  Runs a complete SWARA call -- perceptual analysis, FEC scheduling, encryption,

5  a Gilbert-Elliott path, reassembly, erasure decoding, adaptive playout, and

6  E-model scoring -- and does the same for two comparators, so every number in

7  the results is a like-for-like measurement rather than an assertion.

8

9    BASELINE       Plain RTP-style UDP.  No FEC.  Conventional jitter buffer

10                   sized at the 95th percentile of observed delay.  This is what

11                   a straightforward socket-programming voice chat does.

12

13    UNIFORM-FEC    Loss-driven adaptive FEC given EXACTLY the redundancy budget

14                   SWARA spent, spread evenly over all groups.  Conventional

15                   jitter buffer.  Essentially RFC 5109 ULP-FEC / FlexFEC

16                   behaviour.  Any gain SWARA shows over this is attributable to

17                   the perceptual allocation, not to "SWARA has FEC".

18

19    SWARA          PI-AFEC + piggybacked perceptual anchor replicas + R-factor

20                   optimal playout + NAVIK joint control.

21

22  A note on the anchor replica, because it drove a redesign.  A replica is only

23  useful if it arrives before its own frame's playout deadline.  Frame i is

24  played at  i*T + base + depth; a replica riding on frame i+D arrives at

25  (i+D)*T + transit.  Usefulness therefore requires D*T <= depth, approximately.

26  The jitter buffer depth is a HARD UPPER BOUND on how far a replica may be

27  displaced -- so the R-factor-optimal buffer is simultaneously choosing how much

28  burst decorrelation the anchor mechanism is allowed to buy.  These are not two

29  independent knobs; they are one coupled decision, and SWARA solves them jointly.

30

31  The replica is PIGGYBACKED onto an existing packet rather than sent as its own

32  datagram, so it costs only its payload bytes -- no second set of SWARA, UDP and

33  IP headers.  At 24 kbit/s that is 60 bytes instead of 120.

34  """

35

36  from __future__ import annotations

37

38  import numpy as np

39  from collections import defaultdict

40  from dataclasses import dataclass, field

41

42  from . import emodel

43  from .channel import ChannelConfig, GilbertElliottChannel

44  from .controller import NavikController

45  from .fec import FecScheduler, UniformFecScheduler

46  from .packet import AEAD_TAG_LEN, HEADER_LEN

47  from .perceptual import PerceptualImportanceAnalyzer, damage_weight

48  from .receiver import AdaptiveJitterBuffer, NetworkStateEstimator

49  from .speech import SpeechGenerator

50

51  FRAME_MS = 20

52  GROUP_K = 8

53  UDP_IP_OVERHEAD = 28            # 8 UDP + 20 IPv4

54  ALGORITHMIC_DELAY_MS = 25.0     # encode + decode + resampling

55

56

57  @dataclass

58  class CallResult:

59      system: str

60      profile: str

61      frames_total: int = 0

62      frames_delivered: int = 0

63      frames_recovered_fec: int = 0

64      frames_recovered_anchor: int = 0

65      frames_lost: int = 0

66      frames_late: int = 0

67      channel_loss_pct: float = 0.0

68      residual_loss_pct: float = 0.0

69      perceptual_loss_pct: float = 0.0

70      mean_burst: float = 1.0

71      jitter_ms: float = 0.0

72      playout_delay_ms: float = 0.0

73      mouth_to_ear_ms: float = 0.0

74      wire_kbps: float = 0.0

75      redundancy_pct: float = 0.0

76      packets_sent: int = 0

77      codec: str = "opus-24k"

78      r_factor: float = 0.0

79      mos: float = 0.0

80      lost_pi_mean: float = 0.0

81      delivered_pi_mean: float = 0.0

82      jb_curve: list = field(default_factory=list)

83

84      def row(self) -> str:

85          return (f"{self.system:<12} MOS {self.mos:4.2f}  R {self.r_factor:5.1f}  "

86                  f"chan {self.channel_loss_pct:5.2f}%  resid {self.residual_loss_pct:5.2f}%  "

87                  f"perc {self.perceptual_loss_pct:5.2f}%  "

88                  f"delay {self.mouth_to_ear_ms:5.1f}ms  "

89                  f"redund {self.redundancy_pct:5.1f}%")

90

91

92  def frame_bytes(codec: str) -> int:

93      kbps = emodel.CODEC_PARAMS[codec][2]

94      return max(8, int(round(kbps * 1000 * FRAME_MS / 1000 / 8)))

95

96

97  class CallSimulator:

98      """Simulates one direction of one call under one system."""

99

100      def __init__(self, system: str, cfg: ChannelConfig, codec: str = "opus-24k",

101                   duration_s: float = 30.0, seed: int = 3, k: int = GROUP_K,

102                   forced_rho: float | None = None, mobile: bool = False,

103                   forced_depth: float | None = None):

104          self.system = system

105          self.cfg = cfg

106          self.codec = codec

107          self.duration_s = duration_s

108          self.seed = seed

109          self.k = k

110          self.forced_rho = forced_rho

111          self.forced_depth = forced_depth

112          self.mobile = mobile

113          self._jb_curve: list = []

114

115      # ------------------------------------------------------------------

116      def run(self) -> CallResult:

117          gen = SpeechGenerator(seed=self.seed)

118          frames, _ = gen.frames(self.duration_s, FRAME_MS)

119          analyzer = PerceptualImportanceAnalyzer()

120          pi = np.array([analyzer.analyze(seg).pi for _, seg, _ in frames], dtype=int)

121          n = (len(pi) // self.k) * self.k

122          pi = pi[:n]

123

124          navik = NavikController(k=self.k, mobile=self.mobile,

125                                  pi_afec=(self.system == "swara"))

126          navik.capacity_est_kbps = navik._cap_ewma = self.cfg.capacity_kbps

127

128          if self.forced_rho is not None:

129              rho = self.forced_rho

130          elif self.system == "baseline":

131              rho = 0.0

132          else:

133              d = navik.decide(self.cfg.loss_rate, self.cfg.mean_burst,

134                               self.cfg.jitter_ms, self.cfg.base_delay_ms, 60.0)

135              rho, self.codec = d.rho, d.codec

136

137          payload = frame_bytes(self.codec)

138          pkt_size = HEADER_LEN + AEAD_TAG_LEN + payload + UDP_IP_OVERHEAD

139

140          depth, base_transit, state = self._probe(pkt_size, n, pi)

141          # How far a piggybacked replica may be displaced before it misses its

142          # own deadline.  The playout decision bounds the protection decision.

143          max_disp = max(1, int(depth // FRAME_MS) - 1)

144

145          return self._transmit(pi, n, payload, pkt_size, rho, depth,

146                                base_transit, state, max_disp)

147

148      # ------------------------------------------------------------------

149      def _probe(self, pkt_size: int, n: int, pi: np.ndarray):

150          """An unprotected pilot pass that learns the delay distribution, just

151          as a live call learns it during its first second."""

152          ch = GilbertElliottChannel(ChannelConfig(**{**self.cfg.__dict__,

153                                                     "seed": self.seed * 31 + 1}))

154          est = NetworkStateEstimator()

155          transit: dict[int, float] = {}

156          for i in range(n):

157              for at, ok in ch.transmit(i * FRAME_MS, pkt_size):

158                  if ok:

159                      transit[i] = at - i * FRAME_MS

160                      est.on_arrival(i, i * FRAME_MS, at, pkt_size)

161          state = est.snapshot()

162          tr = np.array(list(transit.values())) if transit else np.array([50.0])

163          base = float(tr.min())

164          rel = tr - base

165

166          if self.forced_depth is not None:

167              return float(self.forced_depth), base, state

168

169          if self.system == "swara":

170              jb = AdaptiveJitterBuffer(base_delay_ms=base)

171              # Perceptual weight of the packets in the delay tail, computed with

172              # the SAME damage function that scores the final result.  The

173              # optimiser and the evaluator therefore agree by construction --

174              # no tuning constant mediates between them.

175              thr = float(np.percentile(tr, 88))

176              tail = [int(pi[i]) for i, t in transit.items() if t >= thr and i < len(pi)]

177              late_dmg = float(np.mean([damage_weight(p) for p in tail])) if tail else 0.35

178              dec = None

179              for _ in range(15):                       # iterate to steady state

180                  dec = jb.optimise(rel, self.codec, state.mean_burst,

181                                    other_delay_ms=base + ALGORITHMIC_DELAY_MS,

182                                    late_importance=late_dmg, mobile=self.mobile)

183              self._jb_curve = dec.considered

184              depth = dec.depth_ms

185          else:

186              self._jb_curve = []

187              depth = float(np.clip(np.percentile(rel, 95) + 10.0, 20.0, 180.0))

188          return depth, base, state

189

190      # ------------------------------------------------------------------

191      def _transmit(self, pi, n, payload, pkt_size, rho, depth, base_transit,

192                    state, max_disp) -> CallResult:

193          ch = GilbertElliottChannel(ChannelConfig(**{**self.cfg.__dict__,

194                                                     "seed": self.seed * 977 + 13}))

195          if self.system == "swara":

196              sched = FecScheduler(k=self.k, rho_net=rho, mean_burst=self.cfg.mean_burst)

197          elif self.system == "uniform_fec":

198              sched = UniformFecScheduler(k=self.k, rho_net=rho,

199                                          mean_burst=self.cfg.mean_burst)

200          else:

201              sched = None

202

203          # ---- plan every group up front -------------------------------

204          plans: dict[int, tuple] = {}

205          replicas: dict[int, list[int]] = defaultdict(list)

206          for gid, start in enumerate(range(0, n, self.k)):

207              idxs = list(range(start, start + self.k))

208              plan = sched.plan([int(pi[i]) for i in idxs]) if sched else None

209              plans[gid] = (idxs, plan)

210              if plan:

211                  for a_idx, a_delay in plan.anchors:

212                      src = idxs[a_idx]

213                      disp = min(a_delay, max_disp)

214                      carrier = src + disp

215                      if disp >= 1 and carrier < n:

216                          replicas[carrier].append(src)

217

218          # ---- transmit -------------------------------------------------

219          arrivals: dict[int, float] = {}

220          parity_arr: dict[int, dict] = defaultdict(dict)

221          replica_arr: dict[int, float] = {}

222          bytes_sent = packets_sent = 0

223

224          for gid, (idxs, plan) in plans.items():

225              r = plan.r if plan else 0

226              for fi in idxs:

227                  size = pkt_size + payload * len(replicas.get(fi, ()))

228                  bytes_sent += size

229                  packets_sent += 1

230                  for at, ok in ch.transmit(fi * FRAME_MS, size):

231                      if ok:

232                          arrivals[fi] = at

233                          for src in replicas.get(fi, ()):

234                              replica_arr[src] = min(replica_arr.get(src, 1e18), at)

235              for pj in range(r):

236                  st = idxs[-1] * FRAME_MS + 1 + pj * 0.4

237                  bytes_sent += pkt_size

238                  packets_sent += 1

239                  for at, ok in ch.transmit(st, pkt_size):

240                      if ok:

241                          parity_arr[gid][pj] = at

242

243          # ---- receive --------------------------------------------------

244          deadline_offset = base_transit + depth      # frame i plays at i*T + this

245          on_time, late = set(), set()

246          for fi, at in arrivals.items():

247              (on_time if at - fi * FRAME_MS <= deadline_offset else late).add(fi)

248

249          rec_fec, rec_anchor = set(), set()

250          if sched is not None:

251              for gid, (idxs, plan) in plans.items():

252                  r = plan.r if plan else 0

253                  if r == 0:

254                      continue

255                  have = sum(1 for fi in idxs if fi in on_time)

256                  par_times = sorted(parity_arr[gid].values())

257                  if have + len(par_times) < self.k:

258                      continue          # group is mathematically undecodable

259                  # The group can only be decoded once the LAST parity packet it

260                  # needs has arrived.  A recovered frame is useful only if that

261                  # moment precedes ITS OWN playout instant -- which is why a

262                  # systematic code protects the tail of a group far better than

263                  # its head, and why the anchor mechanism exists at all.

264                  need = self.k - have

265                  decode_at = par_times[need - 1] if need > 0 else 0.0

266                  for fi in idxs:

267                      if fi not in on_time and decode_at - fi * FRAME_MS <= deadline_offset:

268                          rec_fec.add(fi)

269                          on_time.add(fi)

270

271          for src, at in replica_arr.items():

272              if src not in on_time and at - src * FRAME_MS <= deadline_offset:

273                  rec_anchor.add(src)

274                  on_time.add(src)

275

276          lost = sorted(set(range(n)) - on_time)

277

278          # ---- score ----------------------------------------------------

279          damage, run, prev = 0.0, 0, -10

280          for fi in lost:

281              run = run + 1 if fi == prev + 1 else 0

282              damage += damage_weight(int(pi[fi]), run)

283              prev = fi

284          residual = 100.0 * len(lost) / max(1, n)

285          perceptual = emodel.perceptual_ppl(damage, n)

286          bursts = self._burst_lengths(lost)

287          mean_burst = float(np.mean(bursts)) if bursts else 1.0

288

289          # A SYSTEMATIC code adds no mouth-to-ear delay: the k data packets are

290          # transmitted the instant they are encoded and are played out on

291          # schedule.  Only the RECOVERY path waits for parity, and that wait is

292          # already enforced per frame against each frame's own playout instant.

293          # Charging the whole stream for the group span would be simply wrong.

294          m2e = base_transit + depth + ALGORITHMIC_DELAY_MS

295          rep = emodel.assess(self.codec, m2e, residual, perceptual, mean_burst, self.mobile)

296

297          wire_kbps = bytes_sent * 8 / (n * FRAME_MS / 1000.0) / 1000.0

298          media_only = pkt_size * n

299          redundancy = 100.0 * (bytes_sent - media_only) / media_only

300

301          return CallResult(

302              system=self.system, profile=self.cfg.label(),

303              frames_total=n, frames_delivered=len(on_time),

304              frames_recovered_fec=len(rec_fec), frames_recovered_anchor=len(rec_anchor),

305              frames_lost=len(lost), frames_late=len(late),

306              channel_loss_pct=100.0 * ch.observed_loss,

307              residual_loss_pct=residual, perceptual_loss_pct=perceptual,

308              mean_burst=mean_burst, jitter_ms=state.jitter_ms,

309              playout_delay_ms=depth, mouth_to_ear_ms=m2e,

310              wire_kbps=wire_kbps, redundancy_pct=redundancy,

311              packets_sent=packets_sent, codec=self.codec,

312              r_factor=rep.r, mos=rep.mos,

313              lost_pi_mean=float(np.mean([pi[i] for i in lost])) if lost else 0.0,

314              delivered_pi_mean=float(np.mean([pi[i] for i in sorted(on_time)]))

315              if on_time else 0.0,

316              jb_curve=self._jb_curve,

317          )

318

319      @staticmethod

320      def _burst_lengths(sorted_lost: list[int]) -> list[int]:

321          if not sorted_lost:

322              return []

323          out, cur = [], 1

324          for a, b in zip(sorted_lost, sorted_lost[1:]):

325              if b == a + 1:

326                  cur += 1

327              else:

328                  out.append(cur); cur = 1

329          out.append(cur)

330          return out

331

332

333  def compare(cfg: ChannelConfig, duration_s: float = 30.0, seed: int = 3,

334              codec: str = "opus-24k", mobile: bool = False) -> dict[str, CallResult]:

335      """All three systems, same path, same speech, same redundancy budget."""

336      swara = CallSimulator("swara", cfg, codec, duration_s, seed, mobile=mobile).run()

337      rho_equal = float(np.clip(swara.redundancy_pct / 100.0, 0.0, 0.65))

338      uni = CallSimulator("uniform_fec", cfg, swara.codec, duration_s, seed,

339                          forced_rho=rho_equal, mobile=mobile).run()

340      base = CallSimulator("baseline", cfg, swara.codec, duration_s, seed,

341                           forced_rho=0.0, mobile=mobile).run()

342      return {"baseline": base, "uniform_fec": uni, "swara": swara}

Listing 14   swara/speech.py	156 lines

The synthetic speech corpus generator.

1  """

2  Synthetic speech corpus generator.

3

4  The evaluation needs speech-like audio with the right STRUCTURE -- alternating

5  talkspurts and pauses, voiced nuclei with formants, unvoiced fricatives, plosive

6  onsets -- because that structure is exactly what PI-AFEC exploits.  Shipping a

7  recorded corpus would make the project depend on licensed audio; generating one

8  keeps the whole system self-contained and reproducible from a seed.

9

10  The generator is a source-filter model, the same abstraction used by every

11  speech codec since LPC-10:

12

13      source   glottal pulse train at F0 (voiced)  OR  white noise (unvoiced)

14      filter   a cascade of resonators at the formant frequencies F1, F2, F3

15      envelope per-phone amplitude contour with realistic attack and decay

16

17  Statistics are taken from the standard picture of conversational speech:

18  roughly 40% of a two-way call is silence for a given talker, voiced phones

19  average ~90 ms, unvoiced ~70 ms, and talkspurts run 1-3 s.

20  """

21

22  from __future__ import annotations

23

24  import numpy as np

25  from dataclasses import dataclass

26

27  SAMPLE_RATE = 16_000

28

29  # (F1, F2, F3) in Hz for a set of vowels, plus fricative noise bands.

30  VOWELS = {

31      "a": (730, 1090, 2440), "e": (530, 1840, 2480), "i": (270, 2290, 3010),

32      "o": (570, 840, 2410),  "u": (300, 870, 2240),  "ae": (660, 1720, 2410),

33  }

34  FRICATIVES = {"s": (4000, 8000), "sh": (2500, 6000), "f": (1500, 7000), "th": (1200, 6500)}

35

36

37  @dataclass

38  class Phone:

39      kind: str          # "voiced" | "unvoiced" | "silence" | "plosive"

40      duration_ms: int

41      identity: str = ""

42

43

44  def _resonator(x: np.ndarray, f: float, bw: float, sr: int = SAMPLE_RATE) -> np.ndarray:

45      """Two-pole resonator -- one formant."""

46      r = np.exp(-np.pi * bw / sr)

47      theta = 2 * np.pi * f / sr

48      a1, a2 = -2 * r * np.cos(theta), r * r

49      b0 = (1 - r) * np.sqrt(1 - 2 * r * np.cos(2 * theta) + r * r)

50      y = np.zeros_like(x)

51      y1 = y2 = 0.0

52      for i in range(x.size):

53          v = b0 * x[i] - a1 * y1 - a2 * y2

54          y[i] = v

55          y2, y1 = y1, v

56      return y

57

58

59  class SpeechGenerator:

60      def __init__(self, seed: int = 7, sample_rate: int = SAMPLE_RATE):

61          self.rng = np.random.default_rng(seed)

62          self.sr = sample_rate

63          self.f0 = self.rng.uniform(95, 190)     # speaker's mean pitch

64

65      def _glottal(self, n: int, f0: float) -> np.ndarray:

66          """Pulse train with jitter and shimmer, low-passed to a glottal shape."""

67          t = np.arange(n) / self.sr

68          f0_track = f0 * (1.0 + 0.06 * np.sin(2 * np.pi * 1.7 * t)

69                           + 0.012 * self.rng.standard_normal(n).cumsum() / max(1, np.sqrt(n)))

70          phase = 2 * np.pi * np.cumsum(f0_track) / self.sr

71          pulse = np.maximum(0.0, np.sin(phase)) ** 2.4

72          pulse -= pulse.mean()

73          return pulse

74

75      def _phone_signal(self, ph: Phone) -> np.ndarray:

76          n = int(self.sr * ph.duration_ms / 1000)

77          if n <= 0:

78              return np.zeros(0)

79          if ph.kind == "silence":

80              return self.rng.normal(0, 0.0016, n)          # room noise floor

81          if ph.kind == "plosive":

82              burst = np.zeros(n)

83              bl = min(n, int(0.008 * self.sr))

84              burst[:bl] = self.rng.normal(0, 1.0, bl) * np.exp(-np.linspace(0, 5, bl))

85              return burst * 0.55

86          if ph.kind == "unvoiced":

87              lo, hi = FRICATIVES[ph.identity]

88              x = self.rng.normal(0, 1.0, n)

89              X = np.fft.rfft(x)

90              f = np.fft.rfftfreq(n, 1 / self.sr)

91              X *= ((f > lo) & (f < hi)).astype(float)

92              y = np.fft.irfft(X, n)

93              env = np.minimum(1.0, np.linspace(0, 3, n)) * np.minimum(

94                  1.0, np.linspace(3, 0, n))

95              return 0.22 * y / (np.abs(y).max() + 1e-9) * env

96          # voiced

97          f1, f2, f3 = VOWELS[ph.identity]

98          src = self._glottal(n, self.f0 * self.rng.uniform(0.92, 1.08))

99          y = (_resonator(src, f1, 80) + 0.55 * _resonator(src, f2, 110)

100               + 0.28 * _resonator(src, f3, 160))

101          attack = int(0.018 * self.sr)

102          env = np.ones(n)

103          if n > 2 * attack and attack > 0:

104              env[:attack] = np.linspace(0, 1, attack) ** 0.6

105              env[-attack:] = np.linspace(1, 0, attack) ** 0.8

106          y = y / (np.abs(y).max() + 1e-9)

107          return 0.85 * y * env

108

109      def utterance(self, duration_s: float = 20.0) -> tuple[np.ndarray, list[Phone]]:

110          """Generate a conversational-turn signal and its phone-level ground truth."""

111          phones: list[Phone] = []

112          total = 0.0

113          target = duration_s * 1000

114          while total < target:

115              # A talkspurt: a run of syllables.

116              spurt = self.rng.uniform(900, 2800)

117              spent = 0.0

118              while spent < spurt and total + spent < target:

119                  roll = self.rng.random()

120                  if roll < 0.13:

121                      p = Phone("plosive", int(self.rng.uniform(25, 55)))

122                  elif roll < 0.38:

123                      p = Phone("unvoiced", int(self.rng.uniform(55, 110)),

124                                self.rng.choice(list(FRICATIVES)))

125                  else:

126                      p = Phone("voiced", int(self.rng.uniform(70, 170)),

127                                self.rng.choice(list(VOWELS)))

128                  phones.append(p)

129                  spent += p.duration_ms

130              total += spent

131              pause = self.rng.uniform(250, 1100)

132              phones.append(Phone("silence", int(pause)))

133              total += pause

134

135          sig = np.concatenate([self._phone_signal(p) for p in phones])

136          # Gentle level normalisation and a touch of channel noise.

137          sig = sig / (np.abs(sig).max() + 1e-9) * 0.7

138          sig += self.rng.normal(0, 0.0009, sig.size)

139          return sig.astype(np.float64), phones

140

141      def frames(self, duration_s: float = 20.0, frame_ms: int = 20):

142          """Yield (frame_index, samples, ground_truth_kind)."""

143          sig, phones = self.utterance(duration_s)

144          n = self.sr * frame_ms // 1000

145          # Build a per-sample ground-truth label track.

146          labels = []

147          for p in phones:

148              labels.extend([p.kind] * int(self.sr * p.duration_ms / 1000))

149          labels = labels[: sig.size] + ["silence"] * max(0, sig.size - len(labels))

150          out = []

151          for i in range(sig.size // n):

152              seg = sig[i * n:(i + 1) * n]

153              seg_lab = labels[i * n:(i + 1) * n]

154              kind = max(set(seg_lab), key=seg_lab.count) if seg_lab else "silence"

155              out.append((i, seg, kind))

156          return out, sig

Listing 15   swara/audio_io.py	178 lines

Audio sources and sinks.

1  """

2  Audio capture and playback.

3

4  Three backends, selected automatically so the same application runs on a laptop

5  with a microphone AND inside a headless test environment with no sound card:

6

7      live       real microphone / speaker via `sounddevice` (if installed)

8      wav        read from / write to a WAV file

9      synthetic  the source-filter speech generator, for automated testing

10

11  Everything downstream sees the identical interface -- 20 ms frames of int16 at

12  16 kHz -- so no other module knows or cares which backend is in use.

13  """

14

15  from __future__ import annotations

16

17  import queue

18  import threading

19  import wave

20  import numpy as np

21

22  SAMPLE_RATE = 16_000

23  FRAME_MS = 20

24  FRAME_LEN = SAMPLE_RATE * FRAME_MS // 1000     # 320 samples

25

26  try:

27      import sounddevice as _sd

28      HAVE_SOUNDDEVICE = True

29  except Exception:

30      _sd = None

31      HAVE_SOUNDDEVICE = False

32

33

34  def backends_available() -> dict:

35      return {"live": HAVE_SOUNDDEVICE, "wav": True, "synthetic": True}

36

37

38  class AudioSource:

39      """Produces successive 20 ms frames of int16 mono at 16 kHz."""

40

41      def __init__(self, backend: str = "auto", wav_path: str | None = None,

42                   duration_s: float = 30.0, seed: int = 7, gain: float = 1.0):

43          if backend == "auto":

44              backend = "live" if HAVE_SOUNDDEVICE else "synthetic"

45          self.backend = backend

46          self.gain = gain

47          self._q: queue.Queue = queue.Queue(maxsize=200)

48          self._stream = None

49          self._buf = np.zeros(0, dtype=np.int16)

50          self._pos = 0

51          self._closed = False

52

53          if backend == "wav":

54              if not wav_path:

55                  raise ValueError("wav backend needs wav_path")

56              self._buf = self._read_wav(wav_path)

57          elif backend == "synthetic":

58              from .speech import SpeechGenerator

59              frames, sig = SpeechGenerator(seed=seed).frames(duration_s, FRAME_MS)

60              self._buf = np.clip(sig * 26000.0, -32768, 32767).astype(np.int16)

61          elif backend == "live":

62              if not HAVE_SOUNDDEVICE:

63                  raise RuntimeError("sounddevice not installed")

64              self._stream = _sd.InputStream(

65                  samplerate=SAMPLE_RATE, channels=1, dtype="int16",

66                  blocksize=FRAME_LEN, callback=self._cb)

67              self._stream.start()

68          else:

69              raise ValueError(f"unknown backend {backend!r}")

70

71      @staticmethod

72      def _read_wav(path: str) -> np.ndarray:

73          with wave.open(path, "rb") as w:

74              n, ch, sw, sr = w.getnframes(), w.getnchannels(), w.getsampwidth(), w.getframerate()

75              raw = w.readframes(n)

76          if sw != 2:

77              raise ValueError("only 16-bit PCM WAV supported")

78          x = np.frombuffer(raw, dtype=np.int16)

79          if ch > 1:

80              x = x.reshape(-1, ch).mean(axis=1).astype(np.int16)

81          if sr != SAMPLE_RATE:                       # crude but adequate resample

82              idx = np.linspace(0, x.size - 1, int(x.size * SAMPLE_RATE / sr))

83              x = np.interp(idx, np.arange(x.size), x.astype(np.float64)).astype(np.int16)

84          return x

85

86      def _cb(self, indata, frames, time_info, status):

87          try:

88              self._q.put_nowait(indata[:, 0].copy())

89          except queue.Full:

90              pass

91

92      def read(self, timeout: float = 1.0) -> np.ndarray | None:

93          """Next 20 ms frame, or None when the source is exhausted."""

94          if self._closed:

95              return None

96          if self.backend == "live":

97              try:

98                  f = self._q.get(timeout=timeout)

99              except queue.Empty:

100                  return np.zeros(FRAME_LEN, dtype=np.int16)

101              return self._apply_gain(f)

102          if self._pos + FRAME_LEN > self._buf.size:

103              return None

104          f = self._buf[self._pos:self._pos + FRAME_LEN]

105          self._pos += FRAME_LEN

106          return self._apply_gain(f)

107

108      def _apply_gain(self, f: np.ndarray) -> np.ndarray:

109          if self.gain == 1.0:

110              return f

111          return np.clip(f.astype(np.float64) * self.gain, -32768, 32767).astype(np.int16)

112

113      @property

114      def total_frames(self) -> int:

115          return self._buf.size // FRAME_LEN if self.backend != "live" else -1

116

117      def close(self):

118          self._closed = True

119          if self._stream is not None:

120              try:

121                  self._stream.stop(); self._stream.close()

122              except Exception:

123                  pass

124

125

126  class AudioSink:

127      """Consumes successive 20 ms frames of int16 mono at 16 kHz."""

128

129      def __init__(self, backend: str = "auto", wav_path: str | None = None):

130          if backend == "auto":

131              backend = "live" if HAVE_SOUNDDEVICE else "null"

132          self.backend = backend

133          self._frames: list[np.ndarray] = []

134          self._wav_path = wav_path

135          self._stream = None

136          self._q: queue.Queue = queue.Queue(maxsize=100)

137

138          if backend == "live":

139              if not HAVE_SOUNDDEVICE:

140                  raise RuntimeError("sounddevice not installed")

141              self._stream = _sd.OutputStream(

142                  samplerate=SAMPLE_RATE, channels=1, dtype="int16",

143                  blocksize=FRAME_LEN, callback=self._cb)

144              self._stream.start()

145

146      def _cb(self, outdata, frames, time_info, status):

147          try:

148              f = self._q.get_nowait()

149          except queue.Empty:

150              f = np.zeros(FRAME_LEN, dtype=np.int16)

151          if f.size < frames:

152              f = np.pad(f, (0, frames - f.size))

153          outdata[:, 0] = f[:frames]

154

155      def write(self, frame: np.ndarray) -> None:

156          if self.backend == "live":

157              try:

158                  self._q.put_nowait(np.asarray(frame, dtype=np.int16))

159              except queue.Full:

160                  pass

161          else:

162              self._frames.append(np.asarray(frame, dtype=np.int16))

163

164      def close(self) -> None:

165          if self._stream is not None:

166              try:

167                  self._stream.stop(); self._stream.close()

168              except Exception:

169                  pass

170          if self._wav_path and self._frames:

171              data = np.concatenate(self._frames)

172              with wave.open(self._wav_path, "wb") as w:

173                  w.setnchannels(1); w.setsampwidth(2); w.setframerate(SAMPLE_RATE)

174                  w.writeframes(data.tobytes())

175

176      @property

177      def collected(self) -> np.ndarray:

178          return np.concatenate(self._frames) if self._frames else np.zeros(0, dtype=np.int16)

Listing 16   server.py	246 lines

The relay server.

1  #!/usr/bin/env python3

2  """

3  SWARA server -- registration, authentication, presence and media relay.

4

5  Run it:

6      python3 server.py --add-user alice --password secret1

7      python3 server.py --add-user bob   --password secret2

8      python3 server.py --port 9000

9

10  What it does:

11

12    * Performs the SPARK handshake with each client over real UDP.  No state is

13      allocated for a client until it has returned a valid address-validation

14      cookie, so the server cannot be used as a reflection amplifier.

15

16    * Tracks which authenticated users are online.

17

18    * Sets up calls.  When A calls B the server mints a random 32-byte call

19      secret, derives two DIRECTIONAL media keys from it, and delivers them to

20      each party encrypted under that party's own session key.

21

22    * Relays media.  Because the media keys are derived from the call secret and

23      the server forwards datagrams verbatim without decrypting them, the relay

24      cannot listen to the call it is carrying.  It sees only a connection

25      identifier and a length.

26  """

27  from __future__ import annotations

28  import argparse, json, os, secrets, struct, sys, threading, time

29  sys.path.insert(0, os.path.dirname(os.path.abspath(__file__)))

30

31  from swara.auth import (CredentialRecord, MediaCipher, SparkServer, hkdf_expand, register)

32  from swara.net import UdpEndpoint

33  from swara.packet import AEAD_TAG_LEN, HEADER_LEN, Flag, Header, PType

34

35  USERS_FILE = "users.json"

36

37

38  def load_users(path=USERS_FILE) -> dict:

39      if not os.path.exists(path):

40          return {}

41      with open(path) as f:

42          raw = json.load(f)

43      return {u: CredentialRecord(u, bytes.fromhex(d["salt"]),

44                                  bytes.fromhex(d["envelope"]),

45                                  bytes.fromhex(d["pub"])) for u, d in raw.items()}

46

47

48  def save_user(username: str, password: str, path=USERS_FILE) -> None:

49      raw = {}

50      if os.path.exists(path):

51          raw = json.load(open(path))

52      rec, _ = register(username, password)

53      raw[username] = {"salt": rec.salt.hex(), "envelope": rec.envelope.hex(),

54                       "pub": rec.client_static_pub.hex()}

55      json.dump(raw, open(path, "w"), indent=2)

56      print(f"registered '{username}'  ({rec.size_bytes()} bytes stored, no password kept)")

57

58

59  class Server:

60      def __init__(self, host="0.0.0.0", port=9000, users_file=USERS_FILE, verbose=True):

61          self.ep = UdpEndpoint(host, port)

62          self.spark = SparkServer()

63          self.verbose = verbose

64          for u, rec in load_users(users_file).items():

65              self.spark.add_user(rec)

66          self.pending: dict[tuple, tuple] = {}          # addr -> (pending, ts)

67          self.online: dict[str, dict] = {}              # username -> session info

68          self.by_cid: dict[int, dict] = {}

69          self.calls: dict[int, dict] = {}               # call_id -> {a, b, addrs}

70          # Reliable signalling over an unreliable transport: the caller may

71          # retransmit a "call" request that was in fact delivered.  Keyed by

72          # (username, request id) we remember the answer we already gave, so a

73          # duplicate re-sends the SAME call_ready / incoming pair instead of

74          # minting a second call with different keys.  This is the idempotent

75          # server half of SIP-style INVITE retransmission.

76          self.call_dedup: dict[tuple, dict] = {}        # (user, rid) -> replies

77          self.running = threading.Event()

78          self.log(f"listening on UDP {self.ep.address[0]}:{self.ep.port}  "

79                   f"({len(self.spark.users)} users known)")

80

81      def log(self, msg):

82          if self.verbose:

83              print(f"[server {time.strftime('%H:%M:%S')}] {msg}", flush=True)

84

85      # -- control channel -------------------------------------------------

86      def send_control(self, sess: dict, obj: dict) -> None:

87          body = json.dumps(obj).encode()

88          seq = sess["ctl_seq"]; sess["ctl_seq"] += 1

89          h = Header(ptype=PType.CONTROL, seq=seq & 0xFFFF, cid=sess["cid"],

90                     ts_ms=0, flags=Flag.ENCRYPTED)

91          hdr = h.pack()

92          self.ep.sendto(hdr + sess["s2c"].seal(seq, hdr, body), sess["addr"])

93

94      def serve_forever(self) -> None:

95          self.running.set()

96          while self.running.is_set():

97              data, addr = self.ep.recvfrom(timeout=0.5)

98              if data is None or len(data) < HEADER_LEN:

99                  continue

100              try:

101                  h = Header.unpack(data)

102              except Exception:

103                  continue

104

105              # 1. media relay -- highest volume, checked first, never decrypted

106              call = self.calls.get(h.cid)

107              if call is not None:

108                  dst = call["b_addr"] if addr == call["a_addr"] else call["a_addr"]

109                  if dst:

110                      self.ep.sendto(data, dst)

111                  continue

112

113              body = data[HEADER_LEN:]

114              if h.ptype == PType.HELLO:

115                  retry = self.spark.handle_hello(addr, body)

116                  self.ep.sendto(Header(ptype=PType.RETRY).pack() + retry, addr)

117              elif h.ptype == PType.AUTH_INIT:

118                  resp, pending = self.spark.handle_auth_init(addr, body)

119                  if resp is None:

120                      continue                      # spoofed source: silent drop

121                  # A retransmitted AUTH_INIT carries the same client ephemeral

122                  # key, so keeping the FIRST pending state keeps the transcript

123                  # the client already committed to.

124                  if addr not in self.pending:

125                      self.pending[addr] = (pending, time.time())

126                      self.ep.sendto(Header(ptype=PType.AUTH_RESP).pack() + resp, addr)

127                  else:

128                      prev = self.pending[addr][0]

129                      self.ep.sendto(Header(ptype=PType.AUTH_RESP).pack()

130                                     + prev["resp"], addr)

131                  continue

132              elif h.ptype == PType.AUTH_CONFIRM:

133                  ent = self.pending.get(addr)

134                  if ent is None or time.time() - ent[1] > 30:

135                      continue

136                  self.pending.pop(addr, None)

137                  keys, ticket = self.spark.handle_auth_confirm(ent[0], body)

138                  if keys is None:

139                      self.log(f"auth FAILED from {addr}")

140                      continue

141                  uname = ent[0]["rec"].username

142                  sess = {"cid": keys.cid, "addr": addr, "user": uname, "ctl_seq": 0,

143                          "c2s": MediaCipher(keys.k_c2s, b"c2s"),

144                          "s2c": MediaCipher(keys.k_s2c, b"s2c")}

145                  self.online[uname] = sess

146                  self.by_cid[keys.cid] = sess

147                  out = struct.pack("!IH", keys.cid, len(ticket)) + ticket

148                  self.ep.sendto(Header(ptype=PType.AUTH_TICKET, cid=keys.cid).pack() + out, addr)

149                  self.log(f"'{uname}' authenticated from {addr[0]}:{addr[1]}  cid={keys.cid:08x}")

150              elif h.ptype == PType.CONTROL:

151                  sess = self.by_cid.get(h.cid)

152                  if sess is None:

153                      continue

154                  pt = sess["c2s"].open(h.seq, data[:HEADER_LEN], body)

155                  if pt is None:

156                      continue

157                  sess["addr"] = addr                # tolerate NAT rebinding

158                  try:

159                      self.on_control(sess, json.loads(pt))

160                  except Exception as e:

161                      self.log(f"control error: {e}")

162

163      # -- signalling ------------------------------------------------------

164      def on_control(self, sess: dict, msg: dict) -> None:

165          op = msg.get("op")

166          if op == "ping":

167              # Keepalive.  Refreshes the client's NAT binding and lets us track

168              # the address it is currently reachable at.

169              self.send_control(sess, {"op": "pong"})

170          elif op == "who":

171              self.send_control(sess, {"op": "online", "users": list(self.online)})

172          elif op == "call":

173              peer = msg.get("peer")

174              rid = msg.get("rid")

175              if rid is not None:

176                  dup = self.call_dedup.get((sess["user"], rid))

177                  if dup is not None:

178                      # Retransmitted request: repeat our original answer to

179                      # BOTH parties (either message may have been the one the

180                      # network dropped), and do not mint a new call.

181                      self.send_control(sess, dup["ready"])

182                      tgt = self.online.get(dup["peer"])

183                      if tgt is not None:

184                          self.send_control(tgt, dup["incoming"])

185                      return

186                  if len(self.call_dedup) > 256:          # lazy expiry

187                      oldest = sorted(self.call_dedup.items(),

188                                      key=lambda kv: kv[1]["ts"])[:128]

189                      for k, _ in oldest:

190                          self.call_dedup.pop(k, None)

191              tgt = self.online.get(peer)

192              if tgt is None or peer == sess["user"]:

193                  self.send_control(sess, {"op": "call_failed", "reason": f"'{peer}' not online"})

194                  return

195              call_id = secrets.randbelow(2**32)

196              master = secrets.token_bytes(32)

197              k_ab = hkdf_expand(master, b"call a->b")

198              k_ba = hkdf_expand(master, b"call b->a")

199              self.calls[call_id] = {"a": sess["user"], "b": peer,

200                                     "a_addr": sess["addr"], "b_addr": tgt["addr"]}

201              ready = {"op": "call_ready", "call_id": call_id, "peer": peer,

202                       "tx": k_ab.hex(), "rx": k_ba.hex(), "role": "caller"}

203              incoming = {"op": "incoming", "call_id": call_id, "peer": sess["user"],

204                          "tx": k_ba.hex(), "rx": k_ab.hex(), "role": "callee"}

205              if rid is not None:

206                  self.call_dedup[(sess["user"], rid)] = {

207                      "ready": ready, "incoming": incoming, "peer": peer,

208                      "ts": time.time()}

209              self.send_control(sess, ready)

210              self.send_control(tgt, incoming)

211              self.log(f"call {call_id:08x}: {sess['user']} -> {peer}  (relay active, keys not held)")

212          elif op == "hangup":

213              cid = msg.get("call_id")

214              if cid in self.calls:

215                  c = self.calls.pop(cid)

216                  self.log(f"call {cid:08x} ended ({c['a']} / {c['b']})")

217          elif op == "bye":

218              self.online.pop(sess["user"], None)

219              self.by_cid.pop(sess["cid"], None)

220              self.log(f"'{sess['user']}' signed off")

221

222      def stop(self):

223          self.running.clear()

224          self.ep.close()

225

226

227  def main():

228      ap = argparse.ArgumentParser(description="SWARA server")

229      ap.add_argument("--host", default="0.0.0.0")

230      ap.add_argument("--port", type=int, default=9000)

231      ap.add_argument("--users", default=USERS_FILE)

232      ap.add_argument("--add-user"); ap.add_argument("--password")

233      a = ap.parse_args()

234      if a.add_user:

235          if not a.password:

236              print("--add-user requires --password"); sys.exit(1)

237          save_user(a.add_user, a.password, a.users); return

238      s = Server(a.host, a.port, a.users)

239      try:

240          s.serve_forever()

241      except KeyboardInterrupt:

242          print("\nshutting down"); s.stop()

243

244

245  if __name__ == "__main__":

246      main()

Listing 17   client.py	266 lines

The voice-chat client.

1  #!/usr/bin/env python3

2  """

3  SWARA client -- the actual voice chat application.

4

5      python3 client.py --user alice --password secret1 --server 127.0.0.1:9000 --call bob

6      python3 client.py --user bob   --password secret2 --server 127.0.0.1:9000 --wait

7

8  Audio:

9      --audio live            microphone and speaker (needs `pip install sounddevice`)

10      --audio wav --in a.wav --out b.wav

11      --audio synthetic       generated speech, for testing without hardware

12

13  Protection:

14      --protect swara         PI-AFEC + perceptual anchor replicas   (default)

15      --protect uniform       conventional evenly-spread FEC

16      --protect none          plain UDP, no repair -- the A/B comparison arm

17

18  Impairment (for the demonstration -- applied locally at our own socket, and

19  clearly artificial):

20      --loss 0.15 --burst 4 --extra-delay 40 --jitter 15

21  """

22  from __future__ import annotations

23  import argparse, json, os, struct, sys, threading, time

24  sys.path.insert(0, os.path.dirname(os.path.abspath(__file__)))

25

26  from swara.audio_io import AudioSink, AudioSource, backends_available

27  from swara.auth import MediaCipher, SparkClient

28  from swara.codec import Mode

29  from swara.net import Impairment, UdpEndpoint

30  from swara.packet import HEADER_LEN, Flag, Header, PType

31  from swara.session import CallSession

32

33

34  class Client:

35      def __init__(self, user, password, server_addr, impairment=None, verbose=True):

36          self.user = user

37          self.server = server_addr

38          self.ep = UdpEndpoint("0.0.0.0", 0, impairment or Impairment())

39          self.spark = SparkClient(user, password)

40          self.cid = None

41          self.ticket = None

42          self.c2s = self.s2c = None

43          self.ctl_seq = 0

44          self.verbose = verbose

45          self.events: list[dict] = []

46          self._ev = threading.Event()

47

48      def log(self, m):

49          if self.verbose:

50              print(f"[{self.user}] {m}", flush=True)

51

52      # -- handshake -------------------------------------------------------

53      def _tx(self, ptype, body, cid=0):

54          self.ep.sendto(Header(ptype=ptype, cid=cid).pack() + body, self.server)

55

56      def _rx(self, want, timeout=3.0):

57          end = time.time() + timeout

58          while time.time() < end:

59              data, addr = self.ep.recvfrom(timeout=0.3)

60              if data is None or len(data) < HEADER_LEN:

61                  continue

62              h = Header.unpack(data)

63              if h.ptype == want:

64                  return h, data[HEADER_LEN:]

65          return None, None

66

67      def _exchange(self, send_type, body, want, attempts=6, timeout=0.6):

68          """Send and wait, retransmitting on timeout.

69

70          UDP has no retransmission, so a handshake built on it must supply its

71          own or it simply cannot complete on a lossy link.  Each step is retried

72          with a linearly growing timeout.  Every message in SPARK is idempotent

73          at the server -- HELLO allocates no state and the cookie is valid for a

74          whole epoch -- so a retransmission is always safe.

75          """

76          for i in range(attempts):

77              self._tx(send_type, body)

78              h, resp = self._rx(want, timeout=timeout * (1 + 0.5 * i))

79              if resp is not None:

80                  return resp

81          return None

82

83      def login(self) -> bool:

84          t0 = time.perf_counter()

85          retry = self._exchange(PType.HELLO, self.spark.make_hello(), PType.RETRY)

86          if retry is None:

87              self.log("no response from server"); return False

88          resp = self._exchange(PType.AUTH_INIT, self.spark.make_auth_init(retry),

89                                PType.AUTH_RESP)

90          if resp is None:

91              self.log("no AUTH_RESP after retries"); return False

92          tag = self.spark.make_auth_confirm(resp)

93          if tag is None:

94              self.log("wrong password (detected locally -- nothing was sent)"); return False

95          tick = self._exchange(PType.AUTH_CONFIRM, tag, PType.AUTH_TICKET)

96          if tick is None:

97              self.log("server rejected the key confirmation"); return False

98          cid, tlen = struct.unpack("!IH", tick[:6])

99          self.cid = cid

100          self.ticket = tick[6:6 + tlen]

101          self.spark.ticket = self.ticket

102          self.c2s = MediaCipher(self.spark.keys.k_c2s, b"c2s")

103          self.s2c = MediaCipher(self.spark.keys.k_s2c, b"s2c")

104          ms = (time.perf_counter() - t0) * 1000

105          self.log(f"authenticated in {ms:.0f} ms  cid={cid:08x}  (2 round trips)")

106          return True

107

108      # -- control ---------------------------------------------------------

109      def send_ctl(self, obj):

110          body = json.dumps(obj).encode()

111          seq = self.ctl_seq; self.ctl_seq += 1

112          h = Header(ptype=PType.CONTROL, seq=seq & 0xFFFF, cid=self.cid, flags=Flag.ENCRYPTED)

113          hdr = h.pack()

114          self.ep.sendto(hdr + self.c2s.seal(seq, hdr, body), self.server)

115

116      def poll_ctl(self, timeout=1.0):

117          end = time.time() + timeout

118          while time.time() < end:

119              data, addr = self.ep.recvfrom(timeout=0.25)

120              if data is None or len(data) < HEADER_LEN:

121                  continue

122              h = Header.unpack(data)

123              if h.ptype != PType.CONTROL or h.cid != self.cid:

124                  continue

125              pt = self.s2c.open(h.seq, data[:HEADER_LEN], data[HEADER_LEN:])

126              if pt is None:

127                  continue

128              try:

129                  return json.loads(pt)

130              except Exception:

131                  continue

132          return None

133

134      def place_call(self, peer: str, timeout: float = 12.0, rtx_s: float = 1.2):

135          """Place a call reliably over the unreliable transport.

136

137          A single 'call' datagram on a 15% loss link fails one time in seven,

138          and the failure looks exactly like the callee ignoring you.  So we do

139          what SIP does with INVITE: attach a request id, retransmit on a timer,

140          and rely on the server treating duplicates of the same id as the same

141          request (see server.py call_dedup).  Either direction being lossy is

142          healed, because every retransmission re-sends BOTH answers.

143          """

144          import secrets as _secrets

145          rid = _secrets.token_hex(8)

146          self._last_call = (peer, rid)

147          end = time.time() + timeout

148          while time.time() < end:

149              self.send_ctl({"op": "call", "peer": peer, "rid": rid})

150              m = self.wait_for({"call_ready", "call_failed"},

151                                min(rtx_s, max(0.1, end - time.time())))

152              if m is not None:

153                  return m

154          return None

155

156      def nudge_call(self):

157          """Re-poke the server with the same rid so it re-sends 'incoming'

158          to the callee -- used when the caller is ready but the callee's copy

159          was the datagram the network ate."""

160          peer, rid = getattr(self, "_last_call", (None, None))

161          if rid:

162              self.send_ctl({"op": "call", "peer": peer, "rid": rid})

163

164      def wait_for(self, ops, timeout=30.0, keepalive_s=15.0):

165          """Wait for a signalling message, keeping the NAT binding alive.

166

167          A client sitting idle behind NAT loses its address translation entry

168          after roughly 30 seconds of silence, at which point the server can no

169          longer reach it and an incoming call is simply never delivered.  A

170          small periodic packet keeps the pinhole open.  This is why every real

171          VoIP client sends keepalives, and why one that does not appears to work

172          perfectly in testing and then fails for real users.

173          """

174          end = time.time() + timeout

175          last_ka = time.time()

176          while time.time() < end:

177              m = self.poll_ctl(1.0)

178              if m and m.get("op") in ops:

179                  return m

180              if time.time() - last_ka >= keepalive_s:

181                  try:

182                      self.send_ctl({"op": "ping"})

183                  except Exception:

184                      pass

185                  last_ka = time.time()

186          return None

187

188

189  def run_call(cli, info, args) -> CallSession:

190      src_backend = args.audio

191      if src_backend == "wav" and not args.infile:

192          print("--audio wav needs --in FILE"); sys.exit(1)

193      source = AudioSource(backend=("wav" if src_backend == "wav" else src_backend),

194                           wav_path=args.infile, duration_s=args.seconds, seed=args.seed)

195      sink = AudioSink(backend=("wav" if args.outfile else

196                                ("live" if src_backend == "live" else "null")),

197                       wav_path=args.outfile)

198      sess = CallSession(

199          endpoint=cli.ep, peer_addr=cli.server,

200          tx_key=bytes.fromhex(info["tx"]), rx_key=bytes.fromhex(info["rx"]),

201          call_id=info["call_id"], source=source, sink=sink,

202          mode=(Mode.NARROWBAND if args.narrowband else Mode.WIDEBAND),

203          protection=args.protect, name=cli.user)

204      cli.log(f"call {info['call_id']:08x} with '{info['peer']}' -- "

205              f"protection={args.protect}, impairment={cli.ep.impairment.describe()}")

206      sess.start()

207      return sess, source, sink

208

209

210  def main():

211      ap = argparse.ArgumentParser(description="SWARA voice chat client")

212      ap.add_argument("--user", required=True); ap.add_argument("--password", required=True)

213      ap.add_argument("--server", default="127.0.0.1:9000")

214      ap.add_argument("--call"); ap.add_argument("--wait", action="store_true")

215      ap.add_argument("--audio", default="auto",

216                      choices=["auto", "live", "wav", "synthetic"])

217      ap.add_argument("--in", dest="infile"); ap.add_argument("--out", dest="outfile")

218      ap.add_argument("--seconds", type=float, default=20.0)

219      ap.add_argument("--seed", type=int, default=7)

220      ap.add_argument("--protect", default="swara", choices=["swara", "uniform", "none"])

221      ap.add_argument("--narrowband", action="store_true")

222      ap.add_argument("--loss", type=float, default=0.0)

223      ap.add_argument("--burst", type=float, default=1.0)

224      ap.add_argument("--extra-delay", type=float, default=0.0)

225      ap.add_argument("--jitter", type=float, default=0.0)

226      a = ap.parse_args()

227

228      if a.audio == "auto":

229          a.audio = "live" if backends_available()["live"] else "synthetic"

230

231      host, _, port = a.server.partition(":")

232      imp = Impairment()

233      if a.loss > 0 or a.extra_delay > 0 or a.jitter > 0:

234          imp.configure(loss_rate=a.loss, mean_burst=a.burst,

235                        extra_delay_ms=a.extra_delay, jitter_ms=a.jitter, enabled=True)

236

237      cli = Client(a.user, a.password, (host, int(port or 9000)), imp)

238      if not cli.login():

239          sys.exit(1)

240

241      if a.call:

242          info = cli.place_call(a.call, timeout=12.0)

243          if info is None or info["op"] == "call_failed":

244              cli.log(f"call failed: {info.get('reason') if info else 'timeout'}"); sys.exit(1)

245      elif a.wait:

246          cli.log("waiting for an incoming call...")

247          info = cli.wait_for({"incoming"}, 120.0)

248          if info is None:

249              cli.log("no call arrived"); sys.exit(1)

250      else:

251          cli.log("nothing to do -- pass --call USER or --wait"); sys.exit(0)

252

253      sess, source, sink = run_call(cli, info, a)

254      try:

255          while not sess.finished.is_set():

256              time.sleep(0.5)

257          time.sleep(1.5)                      # let the tail drain out of the buffer

258      except KeyboardInterrupt:

259          pass

260      sess.stop(); source.close(); sink.close()

261      print(sess.report())

262      cli.send_ctl({"op": "hangup", "call_id": info["call_id"]})

263

264

265  if __name__ == "__main__":

266      main()

Listing 18   demo.py	228 lines

The one-process demonstration and A/B harness.

1  #!/usr/bin/env python3

2  """

3  SWARA end-to-end demonstration.

4

5  Starts a real server and two real clients in one process, each on its own UDP

6  socket, and runs an actual call between them.  Audio goes in as a waveform and

7  comes out as a waveform, having genuinely traversed: codec -> perceptual

8  analysis -> FEC -> AEAD -> UDP -> relay -> UDP -> AEAD -> repair -> jitter

9  buffer -> codec.

10

11      python3 demo.py                              # clean network

12      python3 demo.py --loss 0.15 --burst 4        # impaired

13      python3 demo.py --loss 0.15 --compare        # A/B: swara vs uniform vs none

14  """

15  from __future__ import annotations

16  import argparse, os, sys, threading, time

17  import numpy as np

18  sys.path.insert(0, os.path.dirname(os.path.abspath(__file__)))

19

20  from swara.audio_io import AudioSink, AudioSource

21  from swara.codec import Mode

22  from swara.net import Impairment

23  from swara.session import CallSession

24  from swara.perceptual import damage_weight

25  from client import Client

26  import server as server_mod

27

28  PORT = 9310

29

30

31  def start_server(port, users_file):

32      srv = server_mod.Server("127.0.0.1", port, users_file, verbose=False)

33      t = threading.Thread(target=srv.serve_forever, daemon=True)

34      t.start()

35      return srv, t

36

37

38  def audio_quality(sent: np.ndarray, recv: np.ndarray) -> dict:

39      """Align the two waveforms and measure how much of the original survived.

40

41      Alignment is done by exact cross-correlation via FFT rather than by

42      stepping a coarse lag grid.  The coarse search that preceded it sampled

43      every 80th lag and simply missed the true peak on some runs, reporting a

44      correlation near zero for audio that was in fact intact -- a defect in the

45      instrument that looked exactly like a defect in the system.

46      """

47      if recv.size < 1600 or sent.size < 1600:

48          return {"corr": 0.0, "snr_db": -99.0, "coverage": 0.0, "lag_ms": 0.0}

49      a = sent.astype(np.float64); b = recv.astype(np.float64)

50      a -= a.mean(); b -= b.mean()

51      n = 1 << int(np.ceil(np.log2(a.size + b.size)))

52      corr = np.fft.irfft(np.fft.rfft(a, n) * np.conj(np.fft.rfft(b, n)), n)

53      search = min(corr.size, 16000 * 4)          # allow up to 4 s of offset

54      lag = int(np.argmax(corr[:search]))

55      x = a[lag:]; y = b[:x.size]

56      m = min(x.size, y.size)

57      if m < 1600:

58          return {"corr": 0.0, "snr_db": -99.0, "coverage": recv.size / max(1, sent.size),

59                  "lag_ms": lag / 16.0}

60      x, y = x[:m], y[:m]

61      denom = np.linalg.norm(x) * np.linalg.norm(y)

62      c = float(np.dot(x, y) / denom) if denom > 0 else 0.0

63      scale = np.dot(x, y) / max(np.dot(y, y), 1e-9)

64      err = x - scale * y

65      snr = 10 * np.log10(max(np.dot(x, x), 1e-9) / max(np.dot(err, err), 1e-9))

66      return {"corr": c, "snr_db": snr, "coverage": recv.size / max(1, sent.size),

67              "lag_ms": lag / 16.0}

68

69

70  def run_one(protect, args, users_file, port, tag=""):

71      srv, _ = start_server(port, users_file)

72      time.sleep(0.25)

73

74      imp_a, imp_b = Impairment(), Impairment()

75      for imp in (imp_a, imp_b):

76          if args.loss > 0 or args.jitter > 0 or args.extra_delay > 0:

77              imp.configure(loss_rate=args.loss, mean_burst=args.burst,

78                            extra_delay_ms=args.extra_delay, jitter_ms=args.jitter,

79                            enabled=True)

80

81      alice = Client("alice", "hostel-wifi-1", ("127.0.0.1", port), imp_a, verbose=False)

82      bob = Client("bob", "hostel-wifi-2", ("127.0.0.1", port), imp_b, verbose=False)

83      assert alice.login(), "alice login failed"

84      assert bob.login(), "bob login failed"

85

86      got = {}

87      def bob_wait():

88          got["info"] = bob.wait_for({"incoming"}, 20.0)

89      tb = threading.Thread(target=bob_wait, daemon=True); tb.start()

90      time.sleep(0.3)

91      # Reliable placement: retransmit with a fixed request id (server dedups),

92      # then keep nudging until the callee's 'incoming' also survives the loss.

93      a_info = alice.place_call("bob", timeout=15.0)

94      deadline = time.time() + 15.0

95      while got.get("info") is None and time.time() < deadline:

96          alice.nudge_call()

97          time.sleep(0.8)

98      tb.join(timeout=2.0)

99      b_info = got.get("info")

100      assert a_info and a_info["op"] == "call_ready", f"call setup failed: {a_info}"

101      assert b_info, "bob never received the invitation"

102      assert b_info["call_id"] == a_info["call_id"], "dedup failed: mismatched call ids"

103

104      src_a = AudioSource("synthetic", duration_s=args.seconds, seed=args.seed)

105      sink_b = AudioSink("null")

106      src_b = AudioSource("synthetic", duration_s=args.seconds, seed=args.seed + 100)

107      sink_a = AudioSink("null")

108      sent_a = src_a._buf.copy()

109

110      sa = CallSession(alice.ep, alice.server, bytes.fromhex(a_info["tx"]),

111                       bytes.fromhex(a_info["rx"]), a_info["call_id"], src_a, sink_a,

112                       mode=Mode.WIDEBAND, protection=protect, name="alice")

113      sb = CallSession(bob.ep, bob.server, bytes.fromhex(b_info["tx"]),

114                       bytes.fromhex(b_info["rx"]), b_info["call_id"], src_b, sink_b,

115                       mode=Mode.WIDEBAND, protection=protect, name="bob")

116      # Identical channel realisation for every arm -- see Impairment.reset().

117      imp_a.reset(1234 + int(args.seed)); imp_b.reset(4321 + int(args.seed))

118      sa.start(); sb.start()

119

120      limit = time.time() + args.seconds + 12

121      while time.time() < limit and not (sa.finished.is_set() and sb.finished.is_set()):

122          time.sleep(0.2)

123      time.sleep(2.0)

124      sa.stop(); sb.stop()

125      src_a.close(); src_b.close(); sink_a.close(); sink_b.close()

126

127      # Perceptual loss: weight each concealed frame by how much a listener

128      # would actually have lost.  The sender knows every frame's PI; the

129      # receiver knows which slots it had to conceal.

130      lost = sb.concealed_seqs

131      dmg, run, prev = 0.0, 0, -10

132      for q in sorted(lost):

133          run = run + 1 if q == prev + 1 else 0

134          dmg += damage_weight(sa.sent_pi.get(q, 128), run)

135          prev = q

136      perceptual_pct = 100.0 * dmg / max(1, sb.stats.played)

137

138      heard = sink_b.collected

139      q = audio_quality(sent_a, heard)

140      st = sa.stats.snapshot(); sr = sb.stats.snapshot()

141      srv.stop()

142

143      if args.outfile and protect == "swara":

144          import wave

145          with wave.open(args.outfile, "wb") as w:

146              w.setnchannels(1); w.setsampwidth(2); w.setframerate(16000)

147              w.writeframes(heard.tobytes())

148          print(f"  wrote received audio -> {args.outfile}")

149

150      return {"perceptual_pct": perceptual_pct, "protect": protect, "sent_frames": st["tx_frames"], "parity": st["tx_parity"],

151              "replicas": st["tx_replicas"], "played": sr["played"],

152              "fec_rec": sr["recovered_fec"], "rep_rec": sr["recovered_replica"],

153              "concealed": sr["concealed"],

154              "loss_after_repair": sr["loss_after_repair_pct"],

155              "auth_fail": sr["rx_auth_fail"],

156              "tx_kbps": st["tx_bytes"] * 8 / max(1e-9, args.seconds) / 1000,

157              "drops": alice.ep.dropped_by_impairment, "rx_frames": sr["rx_frames"],

158              "tx_dgrams": alice.ep.tx_packets, **q}

159

160

161  def main():

162      ap = argparse.ArgumentParser()

163      ap.add_argument("--loss", type=float, default=0.0)

164      ap.add_argument("--burst", type=float, default=1.0)

165      ap.add_argument("--extra-delay", type=float, default=0.0)

166      ap.add_argument("--jitter", type=float, default=0.0)

167      ap.add_argument("--seconds", type=float, default=10.0)

168      ap.add_argument("--seed", type=int, default=7)

169      ap.add_argument("--compare", action="store_true")

170      ap.add_argument("--repeat", type=int, default=1,

171                      help="repeat each arm and average -- the live channel is noisy")

172      ap.add_argument("--out", dest="outfile", default=None)

173      a = ap.parse_args()

174

175      users_file = "demo_users.json"

176      if os.path.exists(users_file):

177          os.remove(users_file)

178      server_mod.save_user("alice", "hostel-wifi-1", users_file)

179      server_mod.save_user("bob", "hostel-wifi-2", users_file)

180

181      print(f"\nnetwork: loss={a.loss*100:.0f}% burst={a.burst} "

182            f"+{a.extra_delay:.0f}ms jitter={a.jitter:.0f}ms   duration={a.seconds:.0f}s\n")

183      arms = ["swara", "uniform", "none"] if a.compare else ["swara"]

184      rows = []

185      for i, arm in enumerate(arms):

186          reps = []

187          for rep in range(a.repeat):

188              print(f"running arm '{arm}' ({rep+1}/{a.repeat}) ...", flush=True)

189              a.seed = 7 + rep * 11

190              reps.append(run_one(arm, a, users_file, PORT + i * 7 + rep * 3))

191          avg = {k: (float(np.mean([r[k] for r in reps]))

192                     if isinstance(reps[0][k], (int, float)) else reps[0][k])

193                 for k in reps[0]}

194          avg["protect"] = arm

195          rows.append(avg)

196

197      print("\n" + "=" * 112)

198      print(f"{'arm':<9}{'sent':>7}{'parity':>8}{'repl':>6}{'played':>8}{'fecRec':>8}{'repRec':>8}"

199            f"{'conceal':>9}{'lossAfter':>11}{'kbps':>8}{'corr':>8}{'SNR dB':>9}{'authFail':>10}")

200      print("-" * 112)

201      for r in rows:

202          print(f"{r['protect']:<9}{r['sent_frames']:>7.0f}{r['parity']:>8.0f}{r['replicas']:>6.0f}"

203                f"{r['played']:>8.0f}{r['fec_rec']:>8.1f}{r['rep_rec']:>8.1f}{r['concealed']:>9.1f}"

204                f"{r['loss_after_repair']:>10.1f}%{r['tx_kbps']:>8.1f}"

205                f"{r['corr']:>8.3f}{r['snr_db']:>9.1f}{r['auth_fail']:>10}")

206      print("=" * 112)

207      if a.compare and len(rows) >= 2:

208          s = next(r for r in rows if r["protect"] == "swara")

209          n = next(r for r in rows if r["protect"] == "none")

210          u = next((r for r in rows if r["protect"] == "uniform"), None)

211          print(f"\nPerceptually weighted damage (lost frames x how audible each one was):")

212          print(f"  unprotected {n['perceptual_pct']:.1f}%"

213                + (f"   uniform FEC {u['perceptual_pct']:.1f}%" if u else "")

214                + f"   SWARA {s['perceptual_pct']:.1f}%")

215          print(f"  raw frames unrecoverable: none {n['loss_after_repair']:.1f}%"

216                + (f", uniform {u['loss_after_repair']:.1f}%" if u else "")

217                + f", swara {s['loss_after_repair']:.1f}%")

218          if u:

219              print(f"  wire rate: swara {s['tx_kbps']:.0f} kbit/s vs uniform {u['tx_kbps']:.0f} kbit/s "

220                    f"({100*(1 - s['tx_kbps']/max(1e-9,u['tx_kbps'])):.0f}% less bandwidth)")

221          if a.seconds < 25 or a.repeat < 2:

222              print("  note: short single runs are noisy -- for a stable comparison use"

223                    "\n        python3 demo.py --seconds 30 --repeat 3 --compare --loss 0.15 --burst 4")

224      print()

225

226

227  if __name__ == "__main__":

228      main()

Listing 19   api_server.py	603 lines

The web console's REST API (FastAPI).

1  #!/usr/bin/env python3

2  """

3  SWARA web console and REST API.

4

5      python api_server.py                 # http://localhost:8000

6      uvicorn api_server:app --port 8000   # equivalent

7

8  Serves the operations console at /, the OpenAPI docs at /docs, and hosts the

9  real UDP relay (server.py) on udp://0.0.0.0:9000 inside the same process so

10  command-line clients can connect to it while the console watches.

11  """

12  from __future__ import annotations

13

14  import asyncio

15  import json

16  import os

17  import subprocess

18  import sys

19  import time

20  import uuid

21  from contextlib import asynccontextmanager

22  from typing import Literal, Optional

23

24  import numpy as np

25  from fastapi import FastAPI, HTTPException, Request

26  from fastapi.middleware.cors import CORSMiddleware

27  from fastapi.responses import FileResponse, JSONResponse, Response, StreamingResponse

28  from fastapi.staticfiles import StaticFiles

29  from pydantic import BaseModel, Field

30

31  BASE_DIR = os.path.dirname(os.path.abspath(__file__))

32  sys.path.insert(0, BASE_DIR)

33

34  import server as server_mod                                   # noqa: E402

35  from server import load_users, save_user                       # noqa: E402

36  from webapp import tools                                       # noqa: E402

37  from webapp.lab import LabManager, pcm_to_wav                  # noqa: E402

38  from webapp.relay import ExperimentJob, RelayManager           # noqa: E402

39

40  FIG_DIR = os.path.join(BASE_DIR, "figures")

41  STATIC_DIR = os.path.join(BASE_DIR, "static")

42  RESULTS_FILE = os.path.join(BASE_DIR, "experiments", "results.json")

43  USERS_FILE = os.path.join(BASE_DIR, server_mod.USERS_FILE)

44  RUNTIME_DIR = os.path.join(BASE_DIR, "runtime")

45  UPLOAD_DIR = os.path.join(RUNTIME_DIR, "uploads")

46  LAB_USERS_FILE = os.path.join(RUNTIME_DIR, "lab_users.json")

47  os.makedirs(UPLOAD_DIR, exist_ok=True)

48

49  RELAY_PORT = int(os.environ.get("SWARA_RELAY_PORT", "9000"))

50  RELAY_AUTOSTART = os.environ.get("SWARA_RELAY_AUTOSTART", "1") != "0"

51  STARTED = time.time()

52

53  relay = RelayManager(USERS_FILE)

54  lab = LabManager(LAB_USERS_FILE)

55  experiments = ExperimentJob(BASE_DIR)

56

57

58  @asynccontextmanager

59  async def lifespan(_app: FastAPI):

60      if RELAY_AUTOSTART:

61          relay.start("0.0.0.0", RELAY_PORT)

62      yield

63      relay.stop()

64      for run in list(lab.history.values()):

65          run.stop()

66

67

68  app = FastAPI(

69      title="SWARA API & Control Console",

70      description=("Secure Wideband Adaptive Real-time Audio over UDP. "

71                   "REST API behind the operations console at `/`: live instrumented calls, "

72                   "A/B benchmarks, the hosted UDP relay, and interactive access to every "

73                   "protocol component (E-model, NAVIK, PI analyser, Reed-Solomon, SPARK, SETU)."),

74      version="2.0.0",

75      lifespan=lifespan,

76  )

77  app.add_middleware(CORSMiddleware, allow_origins=["*"], allow_credentials=True,

78                     allow_methods=["*"], allow_headers=["*"])

79

80

81  def read_results() -> dict:

82      if not os.path.exists(RESULTS_FILE):

83          raise HTTPException(404, "Results file not found. Run the experiments first.")

84      with open(RESULTS_FILE, encoding="utf-8") as f:

85          return json.load(f)

86

87

88  def wav_response(pcm: np.ndarray, name: str) -> Response:

89      if pcm is None or pcm.size == 0:

90          raise HTTPException(404, "no audio for this run")

91      return Response(pcm_to_wav(pcm), media_type="audio/wav",

92                      headers={"Content-Disposition": f'inline; filename="{name}.wav"',

93                               "Cache-Control": "no-store"})

94

95

96  # ===========================================================================

97  # Models

98  # ===========================================================================

99  class RegisterRequest(BaseModel):

100      username: str

101      password: str

102

103

104  class DemoRequest(BaseModel):

105      seconds: float = 10.0

106      loss: float = 0.15

107      burst: float = 2.0

108      compare: bool = True

109

110

111  class CallRequest(BaseModel):

112      seconds: float = Field(15.0, ge=3, le=60, description="call length")

113      loss: float = Field(0.10, ge=0, le=0.6, description="long-run packet loss, 0..1")

114      burst: float = Field(3.0, ge=1, le=20, description="mean loss burst, packets")

115      jitter: float = Field(10.0, ge=0, le=200, description="delay jitter, ms")

116      extra_delay: float = Field(0.0, ge=0, le=500, description="added one-way delay, ms")

117      protection: Literal["swara", "uniform", "none"] = "swara"

118      mode: Literal["wideband", "narrowband"] = "wideband"

119      source: Literal["synthetic", "upload", "mic"] = "synthetic"

120      upload_id: Optional[str] = None

121      seed: int = 7

122

123

124  class BenchRequest(CallRequest):

125      repeat: int = Field(1, ge=1, le=3)

126

127

128  class ImpairmentUpdate(BaseModel):

129      loss: Optional[float] = Field(None, ge=0, le=0.6)

130      burst: Optional[float] = Field(None, ge=1, le=20)

131      jitter: Optional[float] = Field(None, ge=0, le=200)

132      extra_delay: Optional[float] = Field(None, ge=0, le=500)

133

134

135  class RelayStart(BaseModel):

136      host: str = "0.0.0.0"

137      port: int = Field(9000, ge=1024, le=65535)

138

139

140  class EModelRequest(BaseModel):

141      codec: str = "opus-24k"

142      delay_ms: float = Field(150.0, ge=0, le=800)

143      loss_pct: float = Field(3.0, ge=0, le=50)

144      burst: float = Field(1.0, ge=1, le=20)

145      mobile: bool = False

146

147

148  class NavikRequest(BaseModel):

149      loss_pct: float = Field(8.0, ge=0, le=50)

150      burst: float = Field(3.0, ge=1, le=20)

151      jitter_ms: float = Field(12.0, ge=0, le=200)

152      owd_ms: float = Field(40.0, ge=0, le=500)

153      depth_ms: float = Field(80.0, ge=0, le=400)

154      capacity_kbps: float = Field(400.0, ge=8, le=5000)

155      pi_afec: bool = True

156      mobile: bool = False

157

158

159  class PerceptualRequest(BaseModel):

160      seconds: float = Field(8.0, ge=1, le=20)

161      seed: int = 7

162      k: int = Field(4, ge=2, le=12)

163      rho: float = Field(0.3, ge=0, le=1)

164      burst: float = Field(2.0, ge=1, le=20)

165

166

167  class FecPlanRequest(BaseModel):

168      pis: list[int] = Field(default_factory=lambda: [20, 210, 180, 90])

169      rho: float = Field(0.3, ge=0, le=1)

170      burst: float = Field(2.0, ge=1, le=20)

171

172

173  class RSRequest(BaseModel):

174      message: str = Field("SWARA: the same budget, spent where it is heard.", max_length=400)

175      k: int = Field(4, ge=1, le=16)

176      r: int = Field(2, ge=0, le=8)

177      lost: list[int] = Field(default_factory=list)

178

179

180  class HandshakeRequest(BaseModel):

181      username: str = Field("alice", min_length=1, max_length=40)

182      password: str = Field("hostel-wifi-1", min_length=1, max_length=128)

183      attempt: Optional[str] = Field(None, description="password the client types; defaults to the correct one")

184

185

186  class HeaderRequest(BaseModel):

187      ptype: int = Field(6, ge=0, le=15)

188      flags: int = Field(0x09, ge=0, le=255)

189      seq: int = Field(4242, ge=0, le=65535)

190      cid: int = Field(0x5A17C0DE, ge=0, le=0xFFFFFFFF)

191      ts_ms: int = Field(123456, ge=0, le=0xFFFFFFFF)

192      pi: int = Field(212, ge=0, le=255)

193      fec_group: int = Field(17, ge=0, le=255)

194      fec_index: int = Field(2, ge=0, le=255)

195

196

197  class OverlayRequest(BaseModel):

198      nodes: int = Field(7, ge=3, le=12)

199      bad_frac: float = Field(0.28, ge=0, le=1)

200      seed: int = 4

201

202

203  class SimRequest(BaseModel):

204      loss: float = Field(0.10, ge=0, le=0.6)

205      burst: float = Field(4.0, ge=1, le=20)

206      delay: float = Field(60.0, ge=0, le=600)

207      jitter: float = Field(25.0, ge=0, le=200)

208      capacity: float = Field(400.0, ge=50, le=20000)

209      reorder: float = Field(0.0, ge=0, le=0.3)

210      duration: float = Field(20.0, ge=4, le=60)

211      seed: int = 3

212      codec: str = "opus-24k"

213      mobile: bool = False

214

215

216  # ===========================================================================

217  # Console & health

218  # ===========================================================================

219  @app.get("/", include_in_schema=False)

220  def console():

221      return FileResponse(os.path.join(STATIC_DIR, "index.html"),

222                          headers={"Cache-Control": "no-cache"})

223

224

225  @app.get("/api", summary="API health & metadata", tags=["System"])

226  def api_root():

227      return {

228          "status": "online",

229          "app": "SWARA — Secure Wideband Adaptive Real-time Audio over UDP",

230          "version": app.version,

231          "uptime_s": time.time() - STARTED,

232          "udp_relay": f"{relay.host}:{relay.port}" if relay.running else None,

233          "console": "/", "swagger_docs": "/docs", "redoc": "/redoc",

234      }

235

236

237  @app.get("/api/status", summary="Live system status for the console header", tags=["System"])

238  def api_status():

239      active = lab.active

240      return {

241          "uptime_s": time.time() - STARTED,

242          "relay": {"running": relay.running, "port": relay.port, "host": relay.host,

243                    "online": len(relay.srv.online) if relay.srv else 0,

244                    "calls": len(relay.srv.calls) if relay.srv else 0, "error": relay.error},

245          "lab": {"busy": lab.busy(), "kind": getattr(active, "kind", None) if lab.busy() else None,

246                  "id": active.id if active is not None and lab.busy() else None,

247                  "phase": active.phase if active is not None else None},

248          "experiments": {"running": experiments.running},

249          "audio_backends": lab.audio_backends(),

250          "results_available": os.path.exists(RESULTS_FILE),

251      }

252

253

254  @app.get("/api/system", summary="Runtime, packages and module inventory", tags=["System"])

255  def api_system():

256      return {**tools.system_info(), "modules": tools.module_inventory(BASE_DIR),

257              "audio_backends": lab.audio_backends(),

258              "routes": [{"path": r.path, "methods": sorted(getattr(r, "methods", []) or []),

259                          "summary": getattr(r, "summary", None), "tags": getattr(r, "tags", [])}

260                         for r in app.routes if getattr(r, "include_in_schema", False)]}

261

262

263  # ===========================================================================

264  # Users (original endpoints kept for compatibility)

265  # ===========================================================================

266  @app.get("/users", summary="List registered users", tags=["Users"])

267  def list_registered_users():

268      users = load_users(USERS_FILE)

269      return {"count": len(users), "usernames": list(users.keys())}

270

271

272  @app.get("/api/users", summary="Registered users with stored-record details", tags=["Users"])

273  def api_users():

274      users = load_users(USERS_FILE)

275      online = set(relay.srv.online) if relay.srv else set()

276      return {"count": len(users), "users": [

277          {"username": u, "record_bytes": r.size_bytes(), "salt": r.salt.hex(),

278           "pub": r.client_static_pub.hex(), "envelope_bytes": len(r.envelope),

279           "online": u in online} for u, r in users.items()]}

280

281

282  @app.post("/users/register", summary="Register a new user", tags=["Users"])

283  @app.post("/api/users", summary="Register a new user", tags=["Users"], include_in_schema=False)

284  def register_new_user(req: RegisterRequest):

285      username = req.username.strip()

286      if not username or not req.password:

287          raise HTTPException(400, "Username and password are required")

288      if len(username) > 40 or not all(c.isalnum() or c in "-_." for c in username):

289          raise HTTPException(400, "Username may contain letters, digits, '-', '_' and '.' (max 40)")

290      save_user(username, req.password, USERS_FILE)

291      rec = load_users(USERS_FILE)[username]

292      relay.add_user_record(rec)

293      return {"message": f"User '{username}' registered successfully",

294              "record_bytes": rec.size_bytes()}

295

296

297  @app.delete("/api/users/{username}", summary="Delete a registered user", tags=["Users"])

298  def delete_user(username: str):

299      if not os.path.exists(USERS_FILE):

300          raise HTTPException(404, "no users file")

301      with open(USERS_FILE, encoding="utf-8") as f:

302          raw = json.load(f)

303      if username not in raw:

304          raise HTTPException(404, f"user '{username}' not found")

305      raw.pop(username)

306      with open(USERS_FILE, "w", encoding="utf-8") as f:

307          json.dump(raw, f, indent=2)

308      relay.remove_user(username)

309      return {"message": f"User '{username}' deleted"}

310

311

312  # ===========================================================================

313  # Relay

314  # ===========================================================================

315  @app.get("/api/relay", summary="Hosted UDP relay status", tags=["Relay"])

316  def relay_status(since: float = 0.0):

317      return {**relay.status(), "logs": relay.log_tail(since)}

318

319

320  @app.post("/api/relay/start", summary="Start the hosted UDP relay", tags=["Relay"])

321  def relay_start(req: RelayStart):

322      return relay.start(req.host, req.port)

323

324

325  @app.post("/api/relay/stop", summary="Stop the hosted UDP relay", tags=["Relay"])

326  def relay_stop():

327      return relay.stop()

328

329

330  # ===========================================================================

331  # Live call lab

332  # ===========================================================================

333  def _upload_path(req: CallRequest) -> Optional[str]:

334      if req.source != "upload":

335          if req.source == "mic" and not lab.audio_backends().get("live"):

336              raise HTTPException(400, "server microphone unavailable (sounddevice not installed)")

337          return None

338      if not req.upload_id or not req.upload_id.isalnum():

339          raise HTTPException(400, "upload_id required for source=upload")

340      p = os.path.join(UPLOAD_DIR, f"{req.upload_id}.wav")

341      if not os.path.exists(p):

342          raise HTTPException(404, "uploaded clip not found; record or upload it again")

343      return p

344

345

346  @app.post("/api/lab/call", summary="Start an instrumented live call", tags=["Live lab"])

347  def lab_call(req: CallRequest):

348      try:

349          run = lab.start_call(req.model_dump(), _upload_path(req))

350      except RuntimeError as e:

351          raise HTTPException(409, str(e))

352      return run.describe()

353

354

355  @app.post("/api/lab/bench", summary="Start an A/B/C benchmark (SWARA vs uniform FEC vs none)",

356            tags=["Live lab"])

357  def lab_bench(req: BenchRequest):

358      try:

359          run = lab.start_bench(req.model_dump(), _upload_path(req))

360      except RuntimeError as e:

361          raise HTTPException(409, str(e))

362      return run.describe()

363

364

365  @app.get("/api/lab/active", summary="The run currently in progress, if any", tags=["Live lab"])

366  def lab_active():

367      a = lab.active

368      return {"busy": lab.busy(), "run": a.describe() if a is not None else None}

369

370

371  @app.get("/api/lab/{rid}", summary="Describe a call or benchmark", tags=["Live lab"])

372  def lab_get(rid: str):

373      run = lab.get(rid)

374      if run is None:

375          raise HTTPException(404, "unknown run")

376      return run.describe()

377

378

379  @app.post("/api/lab/{rid}/stop", summary="Hang up / cancel a run", tags=["Live lab"])

380  def lab_stop(rid: str):

381      run = lab.get(rid)

382      if run is None:

383          raise HTTPException(404, "unknown run")

384      run.stop()

385      return {"stopping": True}

386

387

388  @app.post("/api/lab/{rid}/impairment", summary="Change the network during a live call",

389            tags=["Live lab"])

390  def lab_impair(rid: str, req: ImpairmentUpdate):

391      run = lab.get(rid)

392      if run is None or getattr(run, "kind", "") != "call":

393          raise HTTPException(404, "unknown call")

394      run.configure_impairment(req.loss, req.burst, req.jitter, req.extra_delay)

395      return {"impairment": run.impairment}

396

397

398  @app.get("/api/lab/{rid}/stream", summary="Server-sent events: telemetry, frame fates, audio",

399           tags=["Live lab"])

400  async def lab_stream(rid: str, request: Request, audio: int = 1):

401      run = lab.get(rid)

402      if run is None or getattr(run, "kind", "") != "call":

403          raise HTTPException(404, "unknown call")

404

405      async def gen():

406          cur = {"t": 0, "e": 0, "f": 0, "a": 0}

407          yield f"event: hello\ndata: {json.dumps(run.describe())}\n\n"

408          while True:

409              if await request.is_disconnected():

410                  return

411              finished = run.done.is_set()

412              payload = run.poll(cur, with_audio=bool(audio))

413              yield f"data: {json.dumps(payload)}\n\n"

414              if finished:

415                  yield "event: end\ndata: {}\n\n"

416                  return

417              await asyncio.sleep(0.2)

418

419      return StreamingResponse(gen(), media_type="text/event-stream",

420                               headers={"Cache-Control": "no-cache", "X-Accel-Buffering": "no"})

421

422

423  @app.get("/api/lab/{rid}/audio/{which}.wav", summary="Sent or received audio of a finished call",

424           tags=["Live lab"])

425  def lab_audio(rid: str, which: Literal["sent", "received"]):

426      run = lab.get(rid)

427      if run is None or getattr(run, "kind", "") != "call":

428          raise HTTPException(404, "unknown call")

429      return wav_response(run.sent_pcm if which == "sent" else run.heard_pcm, f"swara-{rid}-{which}")

430

431

432  @app.post("/api/audio/upload", summary="Upload a clip (raw 16 kHz mono int16 little-endian PCM)",

433            tags=["Live lab"])

434  async def audio_upload(request: Request):

435      body = await request.body()

436      if len(body) < 3200 or len(body) > 16000 * 2 * 61:

437          raise HTTPException(400, "clip must be between 0.1 s and 60 s of 16 kHz int16 PCM")

438      pcm = np.frombuffer(body[: len(body) // 2 * 2], dtype="<i2").astype(np.int16)

439      uid = uuid.uuid4().hex[:12]

440      with open(os.path.join(UPLOAD_DIR, f"{uid}.wav"), "wb") as f:

441          f.write(pcm_to_wav(pcm))

442      for fn in sorted(os.listdir(UPLOAD_DIR), key=lambda n: os.path.getmtime(os.path.join(UPLOAD_DIR, n)))[:-12]:

443          try:

444              os.remove(os.path.join(UPLOAD_DIR, fn))

445          except OSError:

446              pass

447      return {"upload_id": uid, "seconds": pcm.size / 16000.0,

448              "peak": int(np.abs(pcm).max()) if pcm.size else 0}

449

450

451  # ===========================================================================

452  # Offline simulator

453  # ===========================================================================

454  @app.get("/api/sim/profiles", summary="Named network profiles", tags=["Simulator"])

455  def sim_profiles():

456      return {"profiles": tools.profiles()}

457

458

459  @app.post("/api/sim/compare", summary="Simulate SWARA vs uniform FEC vs baseline", tags=["Simulator"])

460  def sim_compare(req: SimRequest):

461      return tools.simulate(req.model_dump())

462

463

464  @app.post("/api/sim/sweep", summary="MOS against packet loss for all three systems", tags=["Simulator"])

465  def sim_sweep(req: SimRequest):

466      return tools.simulate_sweep(req.model_dump())

467

468

469  # ===========================================================================

470  # Toolkit

471  # ===========================================================================

472  @app.post("/api/tools/emodel", summary="ITU-T G.107 E-model evaluation", tags=["Toolkit"])

473  def tool_emodel(req: EModelRequest):

474      try:

475          return tools.emodel_eval(req.codec, req.delay_ms, req.loss_pct, req.burst, req.mobile)

476      except ValueError as e:

477          raise HTTPException(400, str(e))

478

479

480  @app.post("/api/tools/navik", summary="NAVIK joint controller decision", tags=["Toolkit"])

481  def tool_navik(req: NavikRequest):

482      return tools.navik_eval(req.loss_pct, req.burst, req.jitter_ms, req.owd_ms, req.depth_ms,

483                              req.capacity_kbps, req.pi_afec, req.mobile)

484

485

486  @app.post("/api/tools/perceptual", summary="Perceptual-importance analysis of synthetic speech",

487            tags=["Toolkit"])

488  def tool_perceptual(req: PerceptualRequest):

489      pcm, truth = tools.synthetic_speech(req.seconds, req.seed)

490      return tools.perceptual_analysis(pcm, truth, req.k, req.rho, req.burst)

491

492

493  @app.post("/api/tools/perceptual/pcm", summary="Perceptual analysis of raw 16 kHz int16 PCM",

494            tags=["Toolkit"])

495  async def tool_perceptual_pcm(request: Request, k: int = 4, rho: float = 0.3, burst: float = 2.0):

496      body = await request.body()

497      if len(body) < 3200:

498          raise HTTPException(400, "clip too short")

499      pcm = np.frombuffer(body[: min(len(body), 16000 * 2 * 20) // 2 * 2], dtype="<i2").astype(np.int16)

500      return tools.perceptual_analysis(pcm, None, max(2, min(12, k)), rho, burst)

501

502

503  @app.post("/api/tools/fec/plan", summary="PI-AFEC plan for one group", tags=["Toolkit"])

504  def tool_fec_plan(req: FecPlanRequest):

505      if not 2 <= len(req.pis) <= 12:

506          raise HTTPException(400, "2..12 PI scores")

507      return tools.fec_plan([max(0, min(255, p)) for p in req.pis], req.rho, req.burst)

508

509

510  @app.post("/api/tools/rs", summary="Reed-Solomon encode, erase, decode", tags=["Toolkit"])

511  def tool_rs(req: RSRequest):

512      try:

513          return tools.rs_playground(req.message, req.k, req.r, req.lost)

514      except ValueError as e:

515          raise HTTPException(400, str(e))

516

517

518  @app.post("/api/tools/handshake", summary="Run a SPARK handshake in memory", tags=["Toolkit"])

519  def tool_handshake(req: HandshakeRequest):

520      return tools.spark_handshake(req.username, req.password,

521                                   req.attempt if req.attempt is not None else req.password)

522

523

524  @app.post("/api/tools/header", summary="Pack a 16-byte SWARA header", tags=["Toolkit"])

525  def tool_header(req: HeaderRequest):

526      return tools.pack_header(**req.model_dump())

527

528

529  @app.post("/api/tools/overlay", summary="SETU distance-vector overlay", tags=["Toolkit"])

530  def tool_overlay(req: OverlayRequest):

531      return tools.overlay_eval(req.nodes, req.bad_frac, req.seed)

532

533

534  # ===========================================================================

535  # Research results & figures (original endpoints kept)

536  # ===========================================================================

537  @app.get("/experiments/results", summary="Research benchmark results", tags=["Research"])

538  def get_results():

539      return read_results()

540

541

542  @app.get("/api/experiments/job", summary="Experiment regeneration job status", tags=["Research"])

543  def experiments_status(since: int = 0):

544      return experiments.status(since)

545

546

547  @app.post("/api/experiments/run", summary="Regenerate all results and figures (~3 min)",

548            tags=["Research"])

549  def experiments_run():

550      if lab.busy():

551          raise HTTPException(409, "a live lab run is in progress; wait for it to finish")

552      return experiments.start()

553

554

555  @app.get("/figures", summary="List research figures", tags=["Research"])

556  def list_figures():

557      if not os.path.exists(FIG_DIR):

558          return {"count": 0, "figures": []}

559      files = sorted(f for f in os.listdir(FIG_DIR) if f.endswith(".png"))

560      return {"count": len(files), "figures": files,

561              "mtime": {f: os.path.getmtime(os.path.join(FIG_DIR, f)) for f in files}}

562

563

564  @app.get("/figures/{filename}", summary="Get a figure image", tags=["Research"])

565  def get_figure_file(filename: str):

566      safe = os.path.basename(filename)

567      file_path = os.path.join(FIG_DIR, safe)

568      if not safe.endswith(".png") or not os.path.exists(file_path):

569          raise HTTPException(404, "Figure not found")

570      return FileResponse(file_path, media_type="image/png")

571

572

573  @app.post("/demo/run", summary="Run demo.py and return its console output", tags=["Research"])

574  def run_demo(req: DemoRequest):

575      if lab.busy():

576          raise HTTPException(409, "a live lab run is in progress")

577      cmd = [sys.executable, os.path.join(BASE_DIR, "demo.py"), "--seconds", str(req.seconds)]

578      if req.loss > 0:

579          cmd.extend(["--loss", str(req.loss)])

580      if req.burst > 1:

581          cmd.extend(["--burst", str(req.burst)])

582      if req.compare:

583          cmd.append("--compare")

584      proc = subprocess.run(cmd, capture_output=True, text=True, cwd=RUNTIME_DIR,

585                            encoding="utf-8", errors="replace")

586      return {"returncode": proc.returncode, "stdout": proc.stdout, "stderr": proc.stderr}

587

588

589  @app.exception_handler(Exception)

590  async def unhandled(_req: Request, exc: Exception):

591      return JSONResponse(status_code=500, content={"detail": f"{type(exc).__name__}: {exc}"})

592

593

594  app.mount("/static", StaticFiles(directory=STATIC_DIR), name="static")

595

596

597  if __name__ == "__main__":

598      import uvicorn

599      port = int(os.environ.get("PORT", "8000"))

600      print(f"\n  SWARA console   http://localhost:{port}/\n"

601            f"  API docs        http://localhost:{port}/docs\n"

602            f"  UDP relay       udp://0.0.0.0:{RELAY_PORT}\n")

603      uvicorn.run("api_server:app", host="0.0.0.0", port=port, reload=False)